STM32F103xC, STM32F103xD,

STM32F103xE

High-density performance line Arm®-based 32-bit MCU with 256 to 512KB Flash, USB, CAN, 11 timers, 3 ADCs, 13 communication interfaces

Datasheet production data

## Features

- Core: Arm® 32-bit Cortex®-M3 CPU

  - 72 MHz maximum frequency, 1.25 DMIPS/MHz (Dhrystone 2.1) performance at 0 wait state memory access

  - Single-cycle multiplication and hardware division

- Memories

  - 256 to 512 Kbytes of Flash memory

  - up to 64 Kbytes of SRAM

  - Flexible static memory controller with 4 Chip Select. Supports Compact Flash, SRAM, PSRAM, NOR and NAND memories

  - LCD parallel interface, 8080/6800 modes

- Clock, reset and supply management

  - 2.0 to 3.6 V application supply and I/Os

  - POR, PDR, and programmable voltage detector (PVD)

  - 4-to-16 MHz crystal oscillator

  - Internal 8 MHz factory-trimmed RC

  - Internal 40 kHz RC with calibration

  - 32 kHz oscillator for RTC with calibration

- Low power

  - Sleep, Stop and Standby modes

  - VBAT supply for RTC and backup registers

- 3 × 12-bit, 1 µs A/D converters (up to 21 channels)

  - Conversion range: 0 to 3.6 V

  - Triple-sample and hold capability

  - Temperature sensor

- 2 × 12-bit D/A converters

- DMA: 12-channel DMA controller

  - Supported peripherals: timers, ADCs, DAC, SDIO, I2Ss, SPIs, I2Cs and USARTs

- Debug mode

  - Serial wire debug (SWD) & JTAG interfaces

  - Cortex®-M3 Embedded Trace Macrocell™

- Up to 112 fast I/O ports

  - 51/80/112 I/Os, all mappable on 16 external interrupt vectors and almost all 5 V-tolerant

LFBGA100 10 × 10 mm

LFBGA144 10 × 10 mm

LQFP64 10 × 10 mm,

LQFP100 14 × 14 mm,

LQFP144 20 × 20 mm

WLCSP64

- Up to 11 timers

- Up to four 16-bit timers, each with up to 4 IC/OC/PWM or pulse counter and quadrature (incremental) encoder input

- 2 × 16-bit motor control PWM timers with dead- time generation and emergency stop

- 2 × watchdog timers (Independent and Window)

- SysTick timer: a 24-bit downcounter

- 2 × 16-bit basic timers to drive the DAC

- Up to 13 communication interfaces

  - Up to 2 × I2C interfaces (SMBus/PMBus)

  - Up to 5 USARTs (ISO 7816 interface, LIN, IrDA capability, modem control)

  - Up to 3 SPIs (18 Mbit/s), 2 with I2S interface multiplexed

  - CAN interface (2.0B Active)

  - USB 2.0 full speed interface

  - SDIO interface

- CRC calculation unit, 96-bit unique ID

- ECOPACK® packages

##### Table 1.Device summary

| Reference | Part number |
| --- | --- |
| STM32F103xC | STM32F103RC STM32F103VC STM32F103ZC |
| STM32F103xD | STM32F103RD STM32F103VD STM32F103ZD |
| STM32F103xE | STM32F103RE STM32F103ZE STM32F103VE |

July 2018	DS5792 Rev 13	1/143

www.st.com

Contents

- Introduction	9

- Description	10

  - Device overview	11

  - Full compatibility throughout the family	14

  - Overview	15

    - Arm® Cortex®-M3 core with embedded Flash and SRAM	15

    - Embedded Flash memory	15

    - CRC (cyclic redundancy check) calculation unit	15

    - Embedded SRAM	15

    - FSMC (flexible static memory controller)	15

    - LCD parallel interface	16

    - Nested vectored interrupt controller (NVIC)	16

    - External interrupt/event controller (EXTI)	16

    - Clocks and startup	16

    - Boot modes	17

    - Power supply schemes	17

    - Power supply supervisor	17

    - Voltage regulator	17

    - Low-power modes	18

    - DMA	18

    - RTC (real-time clock) and backup registers	18

    - Timers and watchdogs	19

    - I²C bus	21

    - Universal synchronous/asynchronous receiver transmitters (USARTs)	21

    - Serial peripheral interface (SPI)	21

    - Inter-integrated sound (I2S)	21

    - SDIO	22

    - Controller area network (CAN)	22

    - Universal serial bus (USB)	22

    - GPIOs (general-purpose inputs/outputs)	22

    - ADC (analog to digital converter)	22

    - DAC (digital-to-analog converter)	23

    - Temperature sensor	24

    - Serial wire JTAG debug port (SWJ-DP)	24

    - Embedded Trace Macrocell™	24

- Pinouts and pin descriptions	25

- Memory mapping	40

- Electrical characteristics	41

  - Parameter conditions	41

    - Minimum and maximum values	41

    - Typical values	41

    - Typical curves	41

    - Loading capacitor	41

    - Pin input voltage	41

    - Power supply scheme	42

    - Current consumption measurement	42

  - Absolute maximum ratings	43

  - Operating conditions	44

    - General operating conditions	44

    - Operating conditions at power-up / power-down	45

    - Embedded reset and power control block characteristics	45

    - Embedded reference voltage	46

    - Supply current characteristics	46

    - External clock source characteristics	58

    - Internal clock source characteristics	62

    - PLL characteristics	64

    - Memory characteristics	64

    - FSMC characteristics	66

    - EMC characteristics	86

    - Absolute maximum ratings (electrical sensitivity)	87

    - I/O current injection characteristics	88

    - I/O port characteristics	89

    - NRST pin characteristics	94

    - TIM timer characteristics	95

    - Communications interfaces	96

    - CAN (controller area network) interface	106

    - 12-bit ADC characteristics	106

DS5792 Rev 13	3/143

    - DAC electrical specifications	111

    - Temperature sensor characteristics	113

- Package information	114

  - LFBGA144 package information	114

  - LFBGA100 package information	117

  - WLCSP64 package information	120

  - LQFP144 package information	122

  - LQFP100 package information	126

  - LQFP64 package information	129

  - Thermal characteristics	132

    - Reference document	132

    - Selecting the product temperature range	133

- Ordering information	135

- Revision history	136

4/143	DS5792 Rev 13

List of tables

Table 1.	Device summary	1

Table 2.	STM32F103xC, STM32F103xD and STM32F103xE features

and peripheral counts	11

Table 3.	STM32F103xx family	14

Table 4.	High-density timer feature comparison	19

Table 5.	High-density STM32F103xC/D/E pin definitions	31

Table 6.	FSMC pin definition	38

Table 7.	Voltage characteristics	43

Table 8.	Current characteristics	43

Table 9.	Thermal characteristics	44

Table 10.	General operating conditions	44

Table 11.	Operating conditions at power-up / power-down	45

Table 12.	Embedded reset and power control block characteristics	45

Table 13.	Embedded internal reference voltage	46

Table 14.	Maximum current consumption in Run mode, code with data processing

running from Flash	47

Table 15.	Maximum current consumption in Run mode, code with data processing

running from RAM	47

Table 16.	Maximum current consumption in Sleep mode, code running from Flash or RAM	49

Table 17.	Typical and maximum current consumptions in Stop and Standby modes	50

Table 18.	Typical current consumption in Run mode, code with data processing

running from Flash	53

Table 19.	Typical current consumption in Sleep mode, code running from Flash or

RAM	54

Table 20.	Peripheral current consumption	55

Table 21.	High-speed external user clock characteristics	58

Table 22.	Low-speed external user clock characteristics	58

Table 23.	HSE 4-16 MHz oscillator characteristics	60

Table 24.	LSE oscillator characteristics (fLSE = 32.768 kHz)	61

Table 25.	HSI oscillator characteristics	62

Table 26.	LSI oscillator characteristics	63

Table 27.	Low-power mode wakeup timings	63

Table 28.	PLL characteristics	64

Table 29.	Flash memory characteristics	64

Table 30.	Flash memory endurance and data retention	65

Table 31.	Asynchronous non-multiplexed SRAM/PSRAM/NOR read timings	67

Table 32.	Asynchronous non-multiplexed SRAM/PSRAM/NOR write timings	68

Table 33.	Asynchronous multiplexed PSRAM/NOR read timings	69

Table 34.	Asynchronous multiplexed PSRAM/NOR write timings	70

Table 35.	Synchronous multiplexed NOR/PSRAM read timings	72

Table 36.	Synchronous multiplexed PSRAM write timings	74

Table 37.	Synchronous non-multiplexed NOR/PSRAM read timings	75

Table 38.	Synchronous non-multiplexed PSRAM write timings	76

Table 39.	Switching characteristics for PC Card/CF read and write cycles	81

Table 40.	Switching characteristics for NAND Flash read and write cycles	85

Table 41.	EMS characteristics	86

Table 42.	EMI characteristics	87

Table 43.	ESD absolute maximum ratings	87

DS5792 Rev 13	5/143

Table 44.	Electrical sensitivities	88

Table 45.	I/O current injection susceptibility	88

Table 46.	I/O static characteristics	89

Table 47.	Output voltage characteristics	91

Table 48.	I/O AC characteristics	93

Table 49.	NRST pin characteristics	94

Table 50.	TIMx characteristics	95

Table 51.	I2C characteristics	96

Table 52.	SCL frequency (fPCLK1= 36 MHz.,VDD_I2C = 3.3 V)	97

Table 53.	SPI characteristics	98

Table 54.	I2S characteristics	101

Table 55.	SD / MMC characteristics	103

Table 56.	USB startup time	104

Table 57.	USB DC electrical characteristics	105

Table 58.	USB: full-speed electrical characteristics	105

Table 59.	ADC characteristics	106

Table 60.	RAIN max for fADC = 14 MHz	107

Table 61.	ADC accuracy - limited test conditions	107

Table 62.	ADC accuracy	108

Table 63.	DAC characteristics	111

Table 64.	TS characteristics	113

Table 65.	LFBGA144 – 144-ball low profile fine pitch ball grid array, 10 x 10 mm,

0.8 mm pitch, package mechanical data	114

Table 66.	LFBGA144 recommended PCB design rules (0.8 mm pitch BGA)	115

Table 67.	LFBGA100 - 10 x 10 mm low profile fine pitch ball grid array package

mechanical data	117

Table 68.	LFBGA100 recommended PCB design rules (0.8 mm pitch BGA)	118

Table 69.	WLCSP, 64-ball 4.466 × 4.395 mm, 0.500 mm pitch, wafer-level chip-scale

package mechanical data	120

Table 70.	WLCSP64 recommended PCB design rules (0.5 mm pitch)	121

Table 71.	LQFP144 - 144-pin, 20 x 20 mm low-profile quad flat package

mechanical data	123

Table 72.	LQPF100 – 14 x 14 mm 100-pin low-profile quad flat package

mechanical data	126

Table 73.	LQFP64 – 10 x 10 mm 64 pin low-profile quad flat package mechanical data	129

Table 74.	Package thermal characteristics	132

Table 75.	Ordering information scheme	135

6/143	DS5792 Rev 13

List of figures

Figure 1.	STM32F103xC, STM32F103xD and STM32F103xE performance line block diagram . . . 12 Figure 2.	Clock tree	13

Figure 3.	STM32F103xC/D/E BGA144 ballout	25

Figure 4.	STM32F103xC/D/E performance line BGA100 ballout	26

Figure 5.	STM32F103xC/D/E performance line LQFP144 pinout	27

Figure 6.	STM32F103xC/D/E performance line LQFP100 pinout	28

Figure 7.	STM32F103xC/D/E performance line LQFP64 pinout	29

Figure 8.	STM32F103xC/D/E performance line

WLCSP64 ballout, ball side	30

Figure 9.	Memory map	40

Figure 10.	Pin loading conditions	41

Figure 11.	Pin input voltage	41

Figure 12.	Power supply scheme	42

Figure 13.	Current consumption measurement scheme	42

Figure 14.	Typical current consumption in Run mode versus frequency (at 3.6 V) -

code with data processing running from RAM, peripherals enabled	48

Figure 15.	Typical current consumption in Run mode versus frequency (at 3.6 V)-

code with data processing running from RAM, peripherals disabled	48

Figure 16.	Typical current consumption on VBAT with RTC on vs. temperature

at different VBAT values	51

Figure 17.	Typical current consumption in Stop mode with regulator in run mode

versus temperature at different VDD values	51

Figure 18.	Typical current consumption in Stop mode with regulator in low-power

mode versus temperature at different VDD values	52

Figure 19.	Typical current consumption in Standby mode versus temperature at

different VDD values	52

Figure 20.	High-speed external clock source AC timing diagram	59

Figure 21.	Low-speed external clock source AC timing diagram	59

Figure 22.	Typical application with an 8 MHz crystal	60

Figure 23.	Typical application with a 32.768 kHz crystal	62

Figure 24.	Asynchronous non-multiplexed SRAM/PSRAM/NOR read waveforms	66

Figure 25.	Asynchronous non-multiplexed SRAM/PSRAM/NOR write waveforms	67

Figure 26.	Asynchronous multiplexed PSRAM/NOR read waveforms	68

Figure 27.	Asynchronous multiplexed PSRAM/NOR write waveforms	70

Figure 28.	Synchronous multiplexed NOR/PSRAM read timings	71

Figure 29.	Synchronous multiplexed PSRAM write timings	73

Figure 30.	Synchronous non-multiplexed NOR/PSRAM read timings	75

Figure 31.	Synchronous non-multiplexed PSRAM write timings	76

Figure 32.	PC Card/CompactFlash controller waveforms for common memory read access	77

Figure 33.	PC Card/CompactFlash controller waveforms for common memory write access	78

Figure 34.	PC Card/CompactFlash controller waveforms for attribute memory read

access	79

Figure 35.	PC Card/CompactFlash controller waveforms for attribute memory write

access	80

Figure 36.	PC Card/CompactFlash controller waveforms for I/O space read access	80

Figure 37.	PC Card/CompactFlash controller waveforms for I/O space write access	81

Figure 38.	NAND controller waveforms for read access	83

Figure 39.	NAND controller waveforms for write access	84

DS5792 Rev 13	7/143

Figure 40.	NAND controller waveforms for common memory read access	84

Figure 41.	NAND controller waveforms for common memory write access	85

Figure 42.	Standard I/O input characteristics - CMOS port	90

Figure 43.	Standard I/O input characteristics - TTL port	90

Figure 44.	5 V tolerant I/O input characteristics - CMOS port	90

Figure 45.	5 V tolerant I/O input characteristics - TTL port	91

Figure 46.	I/O AC characteristics definition	94

Figure 47.	Recommended NRST pin protection	95

Figure 48.	I2C bus AC waveforms and measurement circuit	97

Figure 49.	SPI timing diagram - slave mode and CPHA = 0	99

Figure 50.	SPI timing diagram - slave mode and CPHA = 1(1)	99

Figure 51.	SPI timing diagram - master mode(1)	100

Figure 52.	I2S slave timing diagram (Philips protocol)(1)	102

Figure 53.	I2S master timing diagram (Philips protocol)(1)	102

Figure 54.	SDIO high-speed mode	103

Figure 55.	SD default mode	103

Figure 56.	USB timings: definition of data signal rise and fall time	105

Figure 57.	ADC accuracy characteristics	108

Figure 58.	Typical connection diagram using the ADC	109

Figure 59.	Power supply and reference decoupling (VREF+ not connected to VDDA)	109

Figure 60.	Power supply and reference decoupling (VREF+ connected to VDDA)	110

Figure 61.	12-bit buffered /non-buffered DAC	112

Figure 62.	LFBGA144 – 144-ball low profile fine pitch ball grid array, 10 x 10 mm,

0.8 mm pitch, package outline	114

Figure 63.	LFBGA144 – 144-ball low profile fine pitch ball grid array, 10 x 10 mm,

0.8 mm pitch, package recommended footprint	115

Figure 64.	LFBGA144 marking example (package top view)	116

Figure 65.	LFBGA100 - 10 x 10 mm low profile fine pitch ball grid array package

outline	117

Figure 66.	LFBGA100 – 100-ball low profile fine pitch ball grid array, 10 x 10 mm,

0.8 mm pitch, package recommended footprintoutline	118

Figure 67.	LFBGA100 marking example (package top view)	119

Figure 68.	WLCSP, 64-ball 4.466 × 4.395 mm, 0.500 mm pitch, wafer-level chip-scale

package outline	120

Figure 69.	WLCSP64 - 64-ball, 4.4757 x 4.4049 mm, 0.5 mm pitch wafer level chip scale

package recommended footprint	121

Figure 70.	LQFP144 - 144-pin, 20 x 20 mm low-profile quad flat package outline	122

Figure 71.	LQFP144 - 144-pin,20 x 20 mm low-profile quad flat package

recommended footprint	124

Figure 72.	LQFP144 marking example (package top view)	125

Figure 73.	LQFP100 – 14 x 14 mm 100 pin low-profile quad flat package outline	126

Figure 74.	LQFP100 recommended footprint	127

Figure 75.	LQFP100 marking example (package top view)	128

Figure 76.	LQFP64 – 10 x 10 mm 64 pin low-profile quad flat package outline	129

Figure 77.	LQFP64 - 64-pin, 10 x 10 mm low-profile quad flat recommended footprint	130

Figure 78.	LQFP64 marking example (package top view)	131

Figure 79.	LQFP100 PD max vs. TA	134

8/143	DS5792 Rev 13

# Introduction

This datasheet provides the ordering information and mechanical device characteristics of the STM32F103xC, STM32F103xD and STM32F103xE high-density performance line microcontrollers. For more details on the whole STMicroelectronics STM32F103xC/D/E family, please refer to Section 2.2: Full compatibility throughout the family.

The high-density STM32F103xC/D/E datasheet should be read in conjunction with the STM32F10xxx reference manual.

For information on programming, erasing and protection of the internal Flash memory please refer to the STM32F10xxx Flash programming manual.

The reference and Flash programming manuals are both available from the STMicroelectronics website www.st.com.

For information on the Arm®(a) Cortex®-M3 core please refer to the Cortex®-M3 Technical Reference Manual, available from the www.arm.com website at the following address: http://infocenter.arm.com.

  - Arm is a registered trademark of Arm Limited (or its subsidiaries) in the US and/or elsewhere.

DS5792 Rev 13	9/143

# Description

The STM32F103xC, STM32F103xD and STM32F103xE performance line family incorporates the high-performance Arm® Cortex®-M3 32-bit RISC core operating at a

72 MHz frequency, high-speed embedded memories (Flash memory up to 512 Kbytes and SRAM up to 64 Kbytes), and an extensive range of enhanced I/Os and peripherals connected to two APB buses. All devices offer three 12-bit ADCs, four general-purpose 16- bit timers plus two PWM timers, as well as standard and advanced communication interfaces: up to two I2Cs, three SPIs, two I2Ss, one SDIO, five USARTs, an USB and a CAN.

The STM32F103xC/D/E high-density performance line family operates in the –40 to

+105 °C temperature range, from a 2.0 to 3.6 V power supply. A comprehensive set of power-saving mode allows the design of low-power applications.

These features make the STM32F103xC/D/E high-density performance line microcontroller family suitable for a wide range of applications such as motor drives, application control, medical and handheld equipment, PC and gaming peripherals, GPS platforms, industrial applications, PLCs, inverters, printers, scanners, alarm systems video intercom, and HVAC.

10/143	DS5792 Rev 13

## Device overview

The STM32F103xC/D/E high-density performance line family offers devices in six different package types: from 64 pins to 144 pins. Depending on the device chosen, different sets of peripherals are included, the description below gives an overview of the complete range of peripherals proposed in this family.

Figure 1 shows the general block diagram of the device family.

##### Table 2. STM32F103xC, STM32F103xD and STM32F103xE features and peripheral counts

| Peripherals |  | STM32F103Rx |  |  | STM32F103Vx |  |  | STM32F103Zx |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Flash memory in Kbytes |  | 256 | 384 | 512 | 256 | 384 | 512 | 256 | 384 | 512 |
| SRAM in Kbytes |  | 48 | 64 |  | 48 | 64 |  | 48 | 64 |  |
| FSMC |  | No |  |  | Yes(1) |  |  | Yes |  |  |
| Timers | General-purpose | 4 |  |  |  |  |  |  |  |  |
|  | Advanced-control | 2 |  |  |  |  |  |  |  |  |
|  | Basic | 2 |  |  |  |  |  |  |  |  |
| Comm | SPI(I2S)(2) | 3(2) |  |  |  |  |  |  |  |  |
|  | I2C | 2 |  |  |  |  |  |  |  |  |
|  | USART | 5 |  |  |  |  |  |  |  |  |
|  | USB | 1 |  |  |  |  |  |  |  |  |
|  | CAN | 1 |  |  |  |  |  |  |  |  |
|  | SDIO | 1 |  |  |  |  |  |  |  |  |
| GPIOs |  | 51 |  |  | 80 |  |  | 112 |  |  |
| 12-bit ADC Number of channels |  | 3 16 |  |  | 3 16 |  |  | 3 21 |  |  |
| 12-bit DAC Number of channels |  | 2 2 |  |  |  |  |  |  |  |  |
| CPU frequency |  | 72 MHz |  |  |  |  |  |  |  |  |
| Operating voltage |  | 2.0 to 3.6 V |  |  |  |  |  |  |  |  |
| Operating temperatures |  | Ambient temperatures: –40 to +85 °C /–40 to +105 °C (see Table 10) Junction temperature: –40 to + 125 °C (see Table 10) |  |  |  |  |  |  |  |  |
| Package |  | LQFP64, WLCSP64 |  |  | LQFP100, BGA100 |  |  | LQFP144, BGA144 |  |  |

    - For the LQFP100 and BGA100 packages, only FSMC Bank1 and Bank2 are available. Bank1 can only support a multiplexed NOR/PSRAM memory using the NE1 Chip Select. Bank2 can only support a 16- or 8-bit NAND Flash memory using the NCE2 Chip Select. The interrupt line cannot be used since Port G is not available in this package.

    - The SPI2 and SPI3 interfaces give the flexibility to work in an exclusive way in either the SPI mode or the I2S audio mode.

DS5792 Rev 13	11/143

42!#%#,+

42!#%$;0:3= AS !3

.*4234

*4$)

*4#+/37#,+

*4-3/37$)/

*4$/ AS !&

6$$

0BUS

6$$

633

)BUS

&LASH 512 +BYTES

64 BIT

6

$BUS

$$!

3UPPLY

0/2

3YSTEM	2ESET

)NT

SUPERVISION

0/2 /0$2

06$

.234

6

$$!

!;25:0=

$;15:0= #,+

./%

.7%

.%;4:1=

.",;1:0=

.7!)4

., (OR .!$6)

AS !&

6$$!   2# 8 -(Z

2# 40 K(Z

0,,

633!

(#,+ &#,+

6$$ 84!, /3#

4-16 -(Z

)7$'

3TANDBY INTERFACE

6"!4 84!,32K(Z

/3#?).

/3#?/54

0#,+1

0#,+2

6"!4 =1.8 6 TO 3.6 6

$;7:0=

#-$ #+ AS !&

24# "ACKUP

!75

REG

/3#32?).

/3#32?/54

4!-0%2-24#/

!,!2-/3%#/.$ /54

"ACKUP INTERFACE

4 CHANNELS, %42 AS !&

112!&

%84.)4

7+50

4 CHANNELS, %42 AS !&

0!;15:0=

4 CHANNELS, %42 AS !&

0";15:0=

0#;15:0=

0$;15:0=

0%;15:0=

4 CHANNELS AS !&

28, 48, #43, 243, #+ AS !&

28, 48, #43, 243, #+ AS !&

28,48 AS !&

0&;15:0=

28,48 AS !&

0';15:0=

4 CHANNELS

- COMPL. CHANNELS "+)., %42 AS !&

- CHANNELS

3 COMPL. CHANNELS "+).,  %42  AS !&

-/3), -)3/, 3#+, .33 AS !&

28, 48, #43,

243, #+ AS !&

-/3)/3$, -)3/

3#+/#+, -#+, .33/73 AS !&

-/3)/3$, -)3/

3#+/#+, -#+, .33/73 AS !&

3#,, 3$!, 3-"! AS !&

3#,, 3$!, 3-"! AS !&

53"?$0/#!.?48

53"?$-/#!.?28

8   !$#123?).S COMMON TO THE 3 !$#S

8 !$#12?).S COMMON TO !$#1 & !$#2

5 !$#3?).S ON !$#3

12BIT  $!#1

$!#?/541 AS !&

)&

12BIT $!# 2

$!#?/542 AS !&

6$$!

62%&n

62%&+

6$$!

AI14666G

)2#2

)2#1

2ESET & #LOCK CONTROL

.6)#

#ORTEX--3 #05 &MAX: 48/72 -(Z

4RACE/TRIG

37/*4!'

40)5

0OWER

6OLT. REG.

3.3 6 TO 1.8 6

4)-7

4)-6

77$'

53!241

32!- 512 "

30)1

4)-8

30)3 / )233

4)-1

30)2/ )232

4)-5

'0)/ PORT "

4)-4

'0)/ PORT !

4)-3

4)-2

3$)/

&3-#

'0 $-!2

5 CHANNELS

'0 $-!1 7 CHANNELS

32!-

64 +"

4RACE CONTROLLER

"US  -ATRIX

!(": &MAX = 48/72 -(Z

&LASH  OBL INTERFACE

!0"1: &MAX = 24/36 -(Z

Figure 1. STM32F103xC, STM32F103xD and STM32F103xE performance line block diagram

| !("2 | !("2 |
| --- | --- |
| !0"2 | !0"1 |

'0)/ PORT #

'0)/ PORT $

'0)/ PORT %

'0)/ PORT &

'0)/ PORT '

53!242

53!243

5!244

5!245

| 4EMP. SENSOR |  |
| --- | --- |
| 12-BIT !$#1 | )& |
| 12-BIT !$#2 | )& |
| 12-BIT !$#3 | )& |

BX#!. DEVICE

53" 2.0 &3 DEVICE

2X(8X16BIT)

2X(8X16BIT)

)&)&

!0"2: &MAX = 48/72 -(Z

TA = –40 °C to +85 °C (suffix 6, see Table 75) or –40 °C to +105 °C (suffix 7, see Table 75), junction temperature up to 105 °C or 125 °C, respectively.

- AF = alternate function on I/O port pin.9

12/143	DS5792 Rev 13

0&2

/HJHQG:

+6(   +LJK 6SHHG ([WHUQDO FORFN VLJQDO

+6,   +LJK 6SHHG ,QWHUQDO FORFN VLJQDO

/6,   /RZ 6SHHG ,QWHUQDO FORFN VLJQDO

/6(   /RZ 6SHHG ([WHUQDO FORFN VLJQDO

3//&/.

+6,

+6(

6<6&/.

/2

0DLQ

&ORFN 2XWSXW

3HULSKHUDO FORFN HQDEOH

WR ,QGHSHQGHQW :DWFKGRJ (,:'*)

,:'*&/.

/6,

/6, 5& 40 N+]

+&/./2

7R 6',2 $+% LQWHUIDFH

57&6(/>1:0@	/2

WR 7,01 DQG 7,08 7,0[&/.

3HULSKHUDO &ORFN

(QDEOH (2 ELW)

WR $'&1, 2 RU 3

$'&&/.

$'& 3UHVFDOHU

/2, 4, 6, 8

57&&/.

WR 57&

/128

/6(

/6( 26& 32.768 N+]

26&32B,1

26&32B287

7,01 & 8 WLPHUV

,I ($3%2 SUHVFDOHU 1) [1

HOVH [2

SHULSKHUDOV WR $3%2

3HULSKHUDO &ORFN

(QDEOH (15 ELWV)

/2

4-16 0+]

+6( 26&

26&B287

26&B,1

3&/.2

72 0+] PD[

$3%2 3UHVFDOHU

/1, 2, 4, 8, 16

3HULSKHUDO &ORFN (QDEOH (6 ELWV)

3//;735(

WR 7,02,3,4,5,6 DQG 7 7,0;&/.

HOVH [2

&66

7,02,3,4,5,6,7

,I ($3%1 SUHVFDOHU 1) [1

WR $3%1

3HULSKHUDO &ORFN  SHULSKHUDOV

(QDEOH (20 ELWV)

/1, 2..512

PD[

3&/.1

72 0+] 3UHVFDOHU

3//&/.

+6(

+&/.

WR $+% EXV, FRUH, PHPRU\ DQG '0$

WR &RUWH[ 6\VWHP WLPHU

)&/. &RUWH[  IUHH UXQQLQJ FORFN

36 0+] PD[

$3%1 3UHVFDOHU

/1, 2, 4, 8, 16

6<6&/. $+%

+6,

6:

3//65& 3//08/

..., [16

[2, [3, [4

3//

/2

WR )60&

)60&&/.

3HULSKHUDO FORFN HQDEOH

3HULSKHUDO FORFN HQDEOH

72 0+] PD[

&ORFN

/8   (QDEOH (4 ELWV)

+6,

WR 6',2

6',2&/.

HQDEOH

8 0+]

+6, 5&

,262&/.	WR ,262

3HULSKHUDO FORFN HQDEOH

3HULSKHUDO FORFN

WR ,263

,263&/.

86%&/.

WR 86% LQWHUIDFH

48 0+]

86%

3UHVFDOHU

/1, 1.5

)/,7)&/.

WR )ODVK SURJUDPPLQJ LQWHUIDFH

Figure 2. Clock tree

DL14752E

0&2

- When the HSI is used as a PLL clock input, the maximum system clock frequency that can be achieved is 64 MHz.

- For the USB function to be available, both HSE and PLL must be enabled, with the USBCLK at 48 MHz.

- To have an ADC conversion time of 1 µs, APB2 must be at 14 MHz, 28 MHz or 56 MHz.

DS5792 Rev 13	13/143

## Full compatibility throughout the family

The STM32F103xC/D/E is a complete family whose members are fully pin-to-pin, software and feature compatible. In the reference manual, the STM32F103x4 and STM32F103x6 are identified as low-density devices, the STM32F103x8 and STM32F103xB are referred to as medium-density devices and the STM32F103xC, STM32F103xD and STM32F103xE are referred to as high-density devices.

Low-density and high-density devices are an extension of the STM32F103x8/B medium- density devices, they are specified in the STM32F103x4/6 and STM32F103xC/D/E datasheets, respectively. Low-density devices feature lower Flash memory and RAM capacities, less timers and peripherals. High-density devices have higher Flash memory and RAM capacities, and additional peripherals like SDIO, FSMC, I2S and DAC while remaining fully compatible with the other members of the family.

The STM32F103x4, STM32F103x6, STM32F103xC, STM32F103xD and STM32F103xE

are a drop-in replacement for the STM32F103x8/B devices, allowing the user to try different memory densities and providing a greater degree of freedom during the development cycle.

Moreover, the STM32F103xx performance line family is fully compatible with all existing STM32F101xx access line and STM32F102xx USB access line devices.

##### Table 3. STM32F103xx family

| Pinout | Low-density devices |  | Medium-density devices |  | High-density devices |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | 16 KB Flash | 32 KB Flash(1) | 64 KB Flash | 128 KB Flash | 256 KB Flash | 384 KB Flash | 512 KB Flash |
|  | 6 KB RAM | 10 KB RAM | 20 KB RAM | 20 KB RAM | 48 RAM | 64 KB RAM | 64 KB RAM |
| 144 |  |  |  |  | 5 × USARTs 4 × 16-bit timers, 2 × basic timers 3 × SPIs, 2 × I2Ss, 2 × I2Cs USB, CAN, 2 × PWM timers 3 × ADCs, 2 × DACs, 1 × SDIO FSMC (100- and 144-pin packages(2)) |  |  |
| 100 |  |  | 3 × USARTs 3 × 16-bit timers 2 × SPIs, 2 × I2Cs, USB, CAN, 1 × PWM timer 2 × ADCs |  |  |  |  |
| 64 | 2 × USARTs 2 × 16-bit timers 1 × SPI, 1 × I2C, USB, CAN, 1 × PWM timer 2 × ADCs |  |  |  |  |  |  |
| 48 |  |  |  |  |  |  |  |
| 36 |  |  |  |  |  |  |  |

- For orderable part numbers that do not show the A internal code after the temperature range code (6 or 7), the reference datasheet for electrical characteristics is that of the STM32F103x8/B medium-density devices.

- Ports F and G are not available in devices delivered in 100-pin packages.

14/143	DS5792 Rev 13

## Overview

### Arm® Cortex®-M3 core with embedded Flash and SRAM

The Arm Cortex®-M3 processor is the latest generation of Arm processors for embedded systems. It has been developed to provide a low-cost platform that meets the needs of MCU implementation, with a reduced pin count and low-power consumption, while delivering outstanding computational performance and an advanced system response to interrupts.

The Arm Cortex®-M3 32-bit RISC processor features exceptional code-efficiency, delivering the high-performance expected from an Arm core in the memory size usually associated with 8- and 16-bit devices.

With its embedded Arm core, STM32F103xC, STM32F103xD and STM32F103xE performance line family is compatible with all Arm tools and software.

Figure 1 shows the general block diagram of the device family.

### Embedded Flash memory

Up to 512 Kbytes of embedded Flash is available for storing programs and data.

### CRC (cyclic redundancy check) calculation unit

The CRC (cyclic redundancy check) calculation unit is used to get a CRC code from a 32-bit data word and a fixed generator polynomial.

Among other applications, CRC-based techniques are used to verify data transmission or storage integrity. In the scope of the EN/IEC 60335-1 standard, they offer a means of verifying the Flash memory integrity. The CRC calculation unit helps compute a signature of the software during runtime, to be compared with a reference signature generated at link- time and stored at a given memory location.

### Embedded SRAM

Up to 64 Kbytes of embedded SRAM accessed (read/write) at CPU clock speed with 0 wait states.

### FSMC (flexible static memory controller)

The FSMC is embedded in the STM32F103xC, STM32F103xD and STM32F103xE performance line family. It has four Chip Select outputs supporting the following modes: PC Card/Compact Flash, SRAM, PSRAM, NOR and NAND.

Functionality overview:

      - The three FSMC interrupt lines are ORed in order to be connected to the NVIC

      - Write FIFO

      - Code execution from external memory except for NAND Flash and PC Card

      - The targeted frequency, fCLK, is HCLK/2, so external access is at 36 MHz when HCLK is at 72 MHz and external access is at 24 MHz when HCLK is at 48 MHz

DS5792 Rev 13	15/143

### LCD parallel interface

The FSMC can be configured to interface seamlessly with most graphic LCD controllers. It supports the Intel 8080 and Motorola 6800 modes, and is flexible enough to adapt to specific LCD interfaces. This LCD parallel interface capability makes it easy to build cost- effective graphic applications using LCD modules with embedded controllers or high- performance solutions using external controllers with dedicated acceleration.

### Nested vectored interrupt controller (NVIC)

The STM32F103xC, STM32F103xD and STM32F103xE performance line embeds a nested vectored interrupt controller able to handle up to 60 maskable interrupt channels (not including the 16 interrupt lines of Cortex®-M3) and 16 priority levels.

      - Closely coupled NVIC gives low latency interrupt processing

      - Interrupt entry vector table address passed directly to the core

      - Closely coupled NVIC core interface

      - Allows early processing of interrupts

      - Processing of late arriving higher priority interrupts

      - Support for tail-chaining

      - Processor state automatically saved

      - Interrupt entry restored on interrupt exit with no instruction overhead

This hardware block provides flexible interrupt management features with minimal interrupt latency.

### External interrupt/event controller (EXTI)

The external interrupt/event controller consists of 19 edge detector lines used to generate interrupt/event requests. Each line can be independently configured to select the trigger event (rising edge, falling edge, both) and can be masked independently. A pending register maintains the status of the interrupt requests. The EXTI can detect an external line with a pulse width shorter than the Internal APB2 clock period. Up to 112 GPIOs can be connected to the 16 external interrupt lines.

### Clocks and startup

System clock selection is performed on startup, however the internal RC 8 MHz oscillator is selected as default CPU clock on reset. An external 4-16 MHz clock can be selected, in which case it is monitored for failure. If failure is detected, the system automatically switches back to the internal RC oscillator. A software interrupt is generated if enabled. Similarly, full interrupt management of the PLL clock entry is available when necessary (for example with failure of an indirectly used external oscillator).

Several prescalers allow the configuration of the AHB frequency, the high speed APB (APB2) and the low speed APB (APB1) domains. The maximum frequency of the AHB and the high speed APB domains is 72 MHz. The maximum allowed frequency of the low speed APB domain is 36 MHz. See Figure 2 for details on the clock tree.

16/143	DS5792 Rev 13

### Boot modes

At startup, boot pins are used to select one of three boot options:

      - Boot from user Flash: you have an option to boot from any of two memory banks. By default, boot from Flash memory bank 1 is selected. You can choose to boot from Flash memory bank 2 by setting a bit in the option bytes.

      - Boot from system memory

      - Boot from embedded SRAM

The boot loader is located in system memory. It is used to reprogram the Flash memory by using USART1.

### Power supply schemes

      - VDD = 2.0 to 3.6 V: external power supply for I/Os and the internal regulator. Provided externally through VDD pins.

      - VSSA, VDDA = 2.0 to 3.6 V: external analog power supplies for ADC, DAC, Reset blocks, RCs and PLL (minimum voltage to be applied to VDDA is 2.4 V when the ADC or DAC is used). VDDA and VSSA must be connected to VDD and VSS, respectively.

      - VBAT = 1.8 to 3.6 V: power supply for RTC, external clock 32 kHz oscillator and backup registers (through power switch) when VDD is not present.

For more details on how to connect power pins, refer to Figure 12: Power supply scheme.

### Power supply supervisor

The device has an integrated power-on reset (POR)/power-down reset (PDR) circuitry. It is always active, and ensures proper operation starting from/down to 2 V. The device remains in reset mode when VDD is below a specified threshold, VPOR/PDR, without the need for an external reset circuit.

The device features an embedded programmable voltage detector (PVD) that monitors the VDD/VDDA power supply and compares it to the VPVD threshold. An interrupt can be generated when VDD/VDDA drops below the VPVD threshold and/or when VDD/VDDA is higher than the VPVD threshold. The interrupt service routine can then generate a warning message and/or put the MCU into a safe state. The PVD is enabled by software. Refer to Table 12: Embedded reset and power control block characteristics for the values of VPOR/PDR and VPVD.

### Voltage regulator

The regulator has three operation modes: main (MR), low-power (LPR) and power down.

      - MR is used in the nominal regulation mode (Run)

      - LPR is used in the Stop modes.

      - Power down is used in Standby mode: the regulator output is in high impedance: the kernel circuitry is powered down, inducing zero consumption (but the contents of the registers and SRAM are lost)

This regulator is always enabled after reset. It is disabled in Standby mode.

DS5792 Rev 13	17/143

### Low-power modes

The STM32F103xC, STM32F103xD and STM32F103xE performance line supports three low-power modes to achieve the best compromise between low-power consumption, short startup time and available wakeup sources:

      - Sleep mode

In Sleep mode, only the CPU is stopped. All peripherals continue to operate and can wake up the CPU when an interrupt/event occurs.

      - Stop mode

Stop mode achieves the lowest power consumption while retaining the content of SRAM and registers. All clocks in the 1.8 V domain are stopped, the PLL, the HSI RC and the HSE crystal oscillators are disabled. The voltage regulator can also be put either in normal or in low-power mode.

The device can be woken up from Stop mode by any of the EXTI line. The EXTI line source can be one of the 16 external lines, the PVD output, the RTC alarm or the USB wakeup.

      - Standby mode

The Standby mode is used to achieve the lowest power consumption. The internal voltage regulator is switched off so that the entire 1.8 V domain is powered off. The PLL, the HSI RC and the HSE crystal oscillators are also switched off. After entering Standby mode, SRAM and register contents are lost except for registers in the Backup domain and Standby circuitry.

The device exits Standby mode when an external reset (NRST pin), an IWDG reset, a rising edge on the WKUP pin, or an RTC alarm occurs.

Note:	The RTC, the IWDG, and the corresponding clock sources are not stopped by entering Stop or Standby mode.

### DMA

The flexible 12-channel general-purpose DMAs (7 channels for DMA1 and 5 channels for DMA2) are able to manage memory-to-memory, peripheral-to-memory and memory-to- peripheral transfers. The two DMA controllers support circular buffer management, removing the need for user code intervention when the controller reaches the end of the buffer.

Each channel is connected to dedicated hardware DMA requests, with support for software trigger on each channel. Configuration is made by software and transfer sizes between source and destination are independent.

The DMA can be used with the main peripherals: SPI, I2C, USART, general-purpose, basic and advanced-control timers TIMx, DAC, I2S, SDIO and ADC.

### RTC (real-time clock) and backup registers

The RTC and the backup registers are supplied through a switch that takes power either on VDD supply when present or through the VBAT pin. The backup registers are forty-two 16-bit registers used to store 84 bytes of user application data when VDD power is not present.

They are not reset by a system or power reset, and they are not reset when the device

wakes up from the Standby mode.

The real-time clock provides a set of continuously running counters which can be used with suitable software to provide a clock calendar function, and provides an alarm interrupt and a

18/143	DS5792 Rev 13

periodic interrupt. It is clocked by a 32.768 kHz external crystal, resonator or oscillator, the internal low-power RC oscillator or the high-speed external clock divided by 128. The internal low-speed RC has a typical frequency of 40 kHz. The RTC can be calibrated using an external 512 Hz output to compensate for any natural quartz deviation. The RTC features a 32-bit programmable counter for long term measurement using the Compare register to generate an alarm. A 20-bit prescaler is used for the time base clock and is by default configured to generate a time base of 1 second from a clock at 32.768 kHz.

### Timers and watchdogs

The high-density STM32F103xC/D/E performance line devices include up to two advanced- control timers, up to four general-purpose timers, two basic timers, two watchdog timers and a SysTick timer.

Table 4 compares the features of the advanced-control, general-purpose and basic timers.

##### Table 4. High-density timer feature comparison

| Timer | Counter resolution | Counter type | Prescaler factor | DMA request generation | Capture/compare channels | Complementary outputs |
| --- | --- | --- | --- | --- | --- | --- |
| TIM1, TIM8 | 16-bit | Up, down, up/down | Any integer between 1 and 65536 | Yes | 4 | Yes |
| TIM2, TIM3, TIM4, TIM5 | 16-bit | Up, down, up/down | Any integer between 1 and 65536 | Yes | 4 | No |
| TIM6, TIM7 | 16-bit | Up | Any integer between 1 and 65536 | Yes | 0 | No |

DS5792 Rev 13	19/143

Advanced-control timers (TIM1 and TIM8)

The two advanced-control timers (TIM1 and TIM8) can each be seen as a three-phase PWM multiplexed on 6 channels. They have complementary PWM outputs with programmable inserted dead-times. They can also be seen as a complete general-purpose timer. The 4 independent channels can be used for:

      - Input capture

      - Output compare

      - PWM generation (edge or center-aligned modes)

      - One-pulse mode output

If configured as a standard 16-bit timer, it has the same features as the TIMx timer. If configured as the 16-bit PWM generator, it has full modulation capability (0-100%).

In debug mode, the advanced-control timer counter can be frozen and the PWM outputs disabled to turn off any power switch driven by these outputs.

Many features are shared with those of the general-purpose TIM timers which have the same architecture. The advanced-control timer can therefore work together with the TIM timers via the Timer Link feature for synchronization or event chaining.

#### General-purpose timers (TIMx)

There are up to 4 synchronizable general-purpose timers (TIM2, TIM3, TIM4 and TIM5) embedded in the STM32F103xC, STM32F103xD and STM32F103xE performance line devices. These timers are based on a 16-bit auto-reload up/down counter, a 16-bit prescaler and feature 4 independent channels each for input capture/output compare, PWM or one- pulse mode output. This gives up to 16 input captures / output compares / PWMs on the largest packages.

The general-purpose timers can work together with the advanced-control timer via the Timer Link feature for synchronization or event chaining. Their counter can be frozen in debug mode. Any of the general-purpose timers can be used to generate PWM outputs. They all have independent DMA request generation.

These timers are capable of handling quadrature (incremental) encoder signals and the digital outputs from 1 to 3 hall-effect sensors.

#### Basic timers TIM6 and TIM7

These timers are mainly used for DAC trigger generation. They can also be used as a generic 16-bit time base.

#### Independent watchdog

The independent watchdog is based on a 12-bit downcounter and 8-bit prescaler. It is clocked from an independent 40 kHz internal RC and as it operates independently from the main clock, it can operate in Stop and Standby modes. It can be used either as a watchdog to reset the device when a problem occurs, or as a free running timer for application timeout management. It is hardware or software configurable through the option bytes. The counter can be frozen in debug mode.

#### Window watchdog

The window watchdog is based on a 7-bit downcounter that can be set as free running. It can be used as a watchdog to reset the device when a problem occurs. It is clocked from

20/143	DS5792 Rev 13

the main clock. It has an early warning interrupt capability and the counter can be frozen in debug mode.

#### SysTick timer

This timer is dedicated to real-time operating systems, but could also be used as a standard down counter. It features:

      - A 24-bit down counter

      - Autoreload capability

      - Maskable system interrupt generation when the counter reaches 0.

      - Programmable clock source

### I²C bus

Up to two I²C bus interfaces can operate in multimaster and slave modes. They can support standard and fast modes.

They support 7/10-bit addressing mode and 7-bit dual addressing mode (as slave). A hardware CRC generation/verification is embedded.

They can be served by DMA and they support SMBus 2.0/PMBus.

### Universal synchronous/asynchronous receiver transmitters (USARTs)

The STM32F103xC, STM32F103xD and STM32F103xE performance line embeds three universal synchronous/asynchronous receiver transmitters (USART1, USART2 and USART3) and two universal asynchronous receiver transmitters (UART4 and UART5).

These five interfaces provide asynchronous communication, IrDA SIR ENDEC support, multiprocessor communication mode, single-wire half-duplex communication mode and have LIN Master/Slave capability.

The USART1 interface is able to communicate at speeds of up to 4.5 Mbit/s. The other available interfaces communicate at up to 2.25 Mbit/s.

USART1, USART2 and USART3 also provide hardware management of the CTS and RTS signals, Smart Card mode (ISO 7816 compliant) and SPI-like communication capability. All interfaces can be served by the DMA controller except for UART5.

### Serial peripheral interface (SPI)

Up to three SPIs are able to communicate up to 18 Mbits/s in slave and master modes in full-duplex and simplex communication modes. The 3-bit prescaler gives 8 master mode frequencies and the frame is configurable to 8 bits or 16 bits. The hardware CRC generation/verification supports basic SD Card/MMC modes.

All SPIs can be served by the DMA controller.

### Inter-integrated sound (I2S)

Two standard I2S interfaces (multiplexed with SPI2 and SPI3) are available, that can be operated in master or slave mode. These interfaces can be configured to operate with 16/32 bit resolution, as input or output channels. Audio sampling frequencies from 8 kHz up to

48 kHz are supported. When either or both of the I2S interfaces is/are configured in master

DS5792 Rev 13	21/143

mode, the master clock can be output to the external DAC/CODEC at 256 times the sampling frequency.

### SDIO

An SD/SDIO/MMC host interface is available, that supports MultiMediaCard System Specification Version 4.2 in three different databus modes: 1-bit (default), 4-bit and 8-bit. The interface allows data transfer at up to 48 MHz in 8-bit mode, and is compliant with SD Memory Card Specifications Version 2.0.

The SDIO Card Specification Version 2.0 is also supported with two different databus modes: 1-bit (default) and 4-bit.

The current version supports only one SD/SDIO/MMC4.2 card at any one time and a stack of MMC4.1 or previous.

In addition to SD/SDIO/MMC, this interface is also fully compliant with the CE-ATA digital protocol Rev1.1.

### Controller area network (CAN)

The CAN is compliant with specifications 2.0A and B (active) with a bit rate up to 1 Mbit/s. It can receive and transmit standard frames with 11-bit identifiers as well as extended frames with 29-bit identifiers. It has three transmit mailboxes, two receive FIFOs with 3 stages and 14 scalable filter banks.

### Universal serial bus (USB)

The STM32F103xC, STM32F103xD and STM32F103xE performance line embed a USB device peripheral compatible with the USB full-speed 12 Mbs. The USB interface implements a full-speed (12 Mbit/s) function interface. It has software-configurable endpoint setting and suspend/resume support. The dedicated 48 MHz clock is generated from the internal main PLL (the clock source must use a HSE crystal oscillator).

### GPIOs (general-purpose inputs/outputs)

Each of the GPIO pins can be configured by software as output (push-pull or open-drain), as input (with or without pull-up or pull-down) or as peripheral alternate function. Most of the GPIO pins are shared with digital or analog alternate functions. All GPIOs are high current- capable.

The I/Os alternate function configuration can be locked if needed following a specific sequence in order to avoid spurious writing to the I/Os registers.

### ADC (analog to digital converter)

Three 12-bit analog-to-digital converters are embedded into STM32F103xC, STM32F103xD and STM32F103xE performance line devices and each ADC shares up to 21 external channels, performing conversions in single-shot or scan modes. In scan mode, automatic conversion is performed on a selected group of analog inputs.

Additional logic functions embedded in the ADC interface allow:

      - Simultaneous sample and hold

      - Interleaved sample and hold

      - Single shunt

22/143	DS5792 Rev 13

The ADC can be served by the DMA controller.

An analog watchdog feature allows very precise monitoring of the converted voltage of one, some or all selected channels. An interrupt is generated when the converted voltage is outside the programmed thresholds.

The events generated by the general-purpose timers (TIMx) and the advanced-control timers (TIM1 and TIM8) can be internally connected to the ADC start trigger and injection trigger, respectively, to allow the application to synchronize A/D conversion and timers.

### DAC (digital-to-analog converter)

The two 12-bit buffered DAC channels can be used to convert two digital signals into two analog voltage signal outputs. The chosen design structure is composed of integrated resistor strings and an amplifier in inverting configuration.

This dual digital Interface supports the following features:

      - two DAC converters: one for each output channel

      - 8-bit or 12-bit monotonic output

      - left or right data alignment in 12-bit mode

      - synchronized update capability

      - noise-wave generation

      - triangular-wave generation

      - dual DAC channel independent or simultaneous conversions

      - DMA capability for each channel

      - external triggers for conversion

      - input voltage reference VREF+

Eight DAC trigger inputs are used in the STM32F103xC, STM32F103xD and STM32F103xE performance line family. The DAC channels are triggered through the timer update outputs that are also connected to different DMA channels.

DS5792 Rev 13	23/143

### Temperature sensor

The temperature sensor has to generate a voltage that varies linearly with temperature. The conversion range is between 2 V < VDDA < 3.6 V. The temperature sensor is internally connected to the ADC1_IN16 input channel which is used to convert the sensor output voltage into a digital value.

### Serial wire JTAG debug port (SWJ-DP)

The Arm SWJ-DP Interface is embedded, and is a combined JTAG and serial wire debug port that enables either a serial wire debug or a JTAG probe to be connected to the target. The JTAG TMS and TCK pins are shared respectively with SWDIO and SWCLK and a specific sequence on the TMS pin is used to switch between JTAG-DP and SW-DP.

### Embedded Trace Macrocell™

The Arm® Embedded Trace Macrocell provides a greater visibility of the instruction and data flow inside the CPU core by streaming compressed data at a very high rate from the STM32F10xxx through a small number of ETM pins to an external hardware trace port analyzer (TPA) device. The TPA is connected to a host computer using USB, Ethernet, or any other high-speed channel. Real-time instruction and data flow activity can be recorded and then formatted for display on the host computer running debugger software. TPA hardware is commercially available from common development tool vendors. It operates with third party debugger software tools.

24/143	DS5792 Rev 13

# Pinouts and pin descriptions

AI14798b

M

L

K

J

H

G

F

E

D

C

B

A

12

11

10

9

8

7

6

5

4

3

2

1

PB13

PB12

PB11

PB10

PE15

PE7

PF14

PF11

PB1

PA7

PA3

VDDA

PB15

PB14

PD12

PD8

PE14

PE8

PF15

PF12

PB0

PA6

PA2

VREF+

PD15

PD14

PD13

PD9

PE13

PE9

PG0

PF13

PC5

PA5

PA1

VREF–

PG2

PG3

PG4

PD10

PE12

PE10

PG1

PB2/ BOOT1

PC4

PA4

PA0-WKUP

VSSA

PG5

PG6

PG7

PD11

PE11

VSS_1

VSS_7

VSS_6

PC3

PC2

PC1

PC0

PC6

PG8

VSS_9

VSS_2

VSS_8

VDD_6	VDD_7	VDD_1

VSS_4

PF8

PF9

PF10

PC7

PC8

VDD_9

VDD_2

VDD_4	VDD_3	VDD_11	VDD_10	VDD_8

PF6

PF7

NRST

PA8

PC9

PD0

PD2

PG9

VSS_10

VSS_3	VSS_11

PF5

PF4

OSC_OUT	PF3

PA9

PA10

PD1

PD3

PG10

PG13

PB7

BOOT0

PF2

VDD_5

VSS_5

OSC_IN

PA11

NC

PC12

PD4

PG11

PG14

PB6

PB8

PF1

PF0

VBAT

PC15- OSC32_OUT

PA12

PC10

PC11

PD5

PG12

PG15

PB5

PB9

PE6

PE5

PE4

PC14- OSC32_IN

PA13 JTMS

PA14 JTCK

PA15 JTDI

PD7

PD6

PB3 JTDO

PB4 JTRST

PE0

PE1

PE2

PE3

PC13- TAMPER-RTC

Figure 3. STM32F103xC/D/E BGA144 ballout

1. The above figure shows the package top view.

DS5792 Rev 13	25/143

AI14601c

K

J

H

G

F

E

D

C

B

A

10

9

8

7

6

5

4

3

2

1

PD12

PD8

PB12

PB11

PE13

PE9

PB1

PA7

PA3

VDDA

PD13

PD9

PB13

PB10

PE12

PE8

PB0

PA6

PA2

VREF+

PD14

PD10

PB14

PE15

PE11

PE7

PC5

PA5

PA1

VREF–

PD15

PD11

PB15

PE14

PE10

PB2

PC4

PA4

PA0-WKUP

VSSA

PC6

PC8

NC

VDD_1

VDD_2

VDD_3

VDD_4

PC3

PC1

PC0

PC7

PC9

PD1

VSS_1

□VSS_2

VSS_3

VSS_4

PE6

PC2

NRST

PA10

PA8

PD0

PD4

PD7

BOOT0

PE0

PE5

VDD_5

OSC_OUT

PA11

PA9

PC12

PD3

PD6

PB5

PE1

PE4

VSS_5

OSC_IN

PA12

PC10

PC11

PD2

PD5

PB6

PB8

PE3

VBAT

PC15-

OSC32_OUT

PA13

PA14

PA15

PB3

PB4

PB7

PB9

PE2

PC13- TAMPER- RTC

PC14- OSC32_IN

Figure 4. STM32F103xC/D/E performance line BGA100 ballout

1. The above figure shows the package top view.

26/143	DS5792 Rev 13

108

107

106

105

104

103

102

101

100

99

98

97

96

95

94

93

92

91

90

89

88

87

86

85

84

83

82

81

80

79

78

77

76

75

74

73

,1&0144

1

2

3

4

5

6

7

8

9

10

11

12

13

14

15

16

17

18

19

20

21

22

23

24

25

26

27

28

29

30

31

32

33

34

35

36

AI14667

6$$?2

633?2

.#

0! 13

0! 12

0! 11

0! 10

0! 9

0! 8

0#9

0#8

0#7

0#6

6$$?9

633?9

0'8

0'7

0'6

0'5

0'4

0'3

0'2

0$15

0$14

6$$?8

633?8

0$13

0$12

0$11

0$10

0$9

0$8

0"15

0"14

0"13

0"12

0%2

0%3

0%4

0%5

0%6

6"!4

0#13-4!-0%2-24#

0#14-/3#32?).

0#15-/3#32?/54

0&0

0&1

0&2

0&3

0&4

0&5

633?5

6$$?5

0&6

0&7

0&8

0&9

0&10

/3#?).

/3#?/54

.234

0#0

0#1

0#2

0#3

633!

62%&-

62%&+

6$$!

0! 0-7+50

0! 1

0! 2

144

143

142

141

140

139

138

137

136

135

134

133

132

131

130

129

128

127

126

125

124

123

122

121

120

119

118

117

116

115

114

113

112

111

110

109

6$$?3

633?3

0%1

0%0

0"9

0"8

"//40 0"7

0"6

0"5

0"4

0"3

0'15

6$$?11

633?11

0'14

0'13

0'12

0'11

0'10

0'9

0$7

0$6

6$$?10

633?10

0$5

0$4

0$3

0$2

0$1

0$0

0#12

0#11

0#10

0! 15

0! 14

Figure 5. STM32F103xC/D/E performance line LQFP144 pinout

0! 3

633?4

6$$?4

0! 4

0! 5

0! 6

0! 7

0#4

0#5

0"0

0"1

0"2

0&11

0&12

633?6

6$$?6

0&13

0&14

0&15

0'0

0'1

0%7

0%8

0%9

633?7

6$$?7

0%10

0%11

0%12

0%13

0%14

0%15

0"10

0"11

633?1

6$$?1

37

38

39

40

41

42

43

44

45

46

47

48

49

50

51

52

53

54

55

56

57

58

59

60

61

62

63

64

65

66

67

68

69

70

71

72

1. The above figure shows the package top view.

DS5792 Rev 13	27/143

75	6$$?2

74	633?2

73	.#

72	0! 13

71	0! 12

70	0! 11

69	0! 10

68	0! 9

67	0! 8

66	0#9

65	0#8

64	0#7

63	0#6

62	0$15

61	0$14

60	0$13

59	0$12

58	0$11

57	0$10

56	0$9

55	0$8

54	0"15

53	0"14

52	0"13

51	0"12

,1&0100

1

2

3

4

5

6

7

8

9

10

11

12

13

14

15

16

17

18

19

20

21

22

23

24

25

AI14391

0%2

0%3

0%4

0%5

0%6

6"!4

0#13-4!-0%2-24#

0#14-/3#32?).

0#15-/3#32?/54

633?5

6$$?5

/3#?).

/3#?/54

.234

0#0

0#1

0#2

0#3

633!

62%&-

62%&+

6$$!

0!0-7+50

0!1

0!2

100

99

98

97

96

95

94

93

92

91

90

89

88

87

86

85

84

83

82

81

80

79

78

77

76

6$$?3

633?3

0%1

0%0

0"9

0"8

"//40 0"7

0"6

0"5

0"4

0"3

0$7

0$6

0$5

0$4

0$3

0$2

0$1

0$0

0#12

0#11

0#10

0!15

0!14

Figure 6. STM32F103xC/D/E performance line LQFP100 pinout

0!3

633?4

6$$?4

0!4

0!5

0!6

0!7

0#4

0#5

0"0

0"1

0"2

0%7

0%8

0%9

0%10

0%11

0%12

0%13

0%14

0%15

0"10

0"11

633?1

6$$?1

26

27

28

29

30

31

32

33

34

35

36

37

38

39

40

41

42

43

44

45

46

47

48

49

50

1. The above figure shows the package top view.

28/143	DS5792 Rev 13

DL14392

sDDͺϮ s^^ͺϮ WA ϭϯ

WA ϭϮ

WA ϭϭ

WA ϭϬ

WA ϵ

WA ϴ

WCϵ WCϴ

WCϳ WCϲ WB ϭϱ WB ϭϰ WB ϭϯ WB ϭϮ

sBAd WCϭϯͲdADWEZͲZdC

WC ϭϰ ͲK ^C ϯϮͺ/E

WC ϭϱ ͲK ^C ϯϮͺKh d

W DϬͲK^ Cͺ/E W DϭͲK^ CͺKhd

EZ^d WCϬ

WCϭ WCϮ

WCϯ s^^A sDDA

WA ϬͲt< hW

WA ϭ

WA Ϯ

sDDͺϯ s^^ͺϯ

WB ϵ WB ϴ BKK d Ϭ

WB ϳ WB ϲ WB ϱ WB ϰ WB ϯ WDϮ WCϭϮ

WCϭϭ WCϭϬ

WA ϭϱ

WA ϭϰ

Figure 7. STM32F103xC/D/E performance line LQFP64 pinout

ϭϳ ϭϴ ϭϵ ϮϬ Ϯϭ ϮϮ Ϯϯ Ϯϰ Ϯϱ Ϯϲ Ϯϳ Ϯϴ Ϯϵ ϯϬ ϯϭ ϯϮ

ϯϯ

ϭϲ

ϰϲ ϰϱ ϰϰ ϰϯ ϰϮ ϰϭ ϰϬ ϯϵ ϯϴ ϯϳ ϯϲ ϯϱ ϯϰ

>Y&Wϲϰ

ϰϴ ϰϳ

Ϯ

ϯ ϰ ϱ ϲ ϳ ϴ

ϵ ϭϬ ϭϭ ϭϮ ϭϯ ϭϰ ϭϱ

ϲϰ ϲϯ ϲϮ ϲϭ ϲϬ ϱϵ ϱϴ ϱϳ ϱϲ ϱϱ ϱϰ ϱϯ ϱϮ ϱϭ ϱϬ ϰϵ

ϭ

WA ϯ s^^ͺϰ sDDͺϰ

WA ϰ

WA ϱ

WA ϲ

WA ϳ WCϰ WCϱ WB Ϭ

WB ϭ WB Ϯ WBϭ Ϭ

WBϭ ϭ s^^ͺϭ sDDͺϭ

1. The above figure shows the package top view.

DS5792 Rev 13	29/143

VDD_1

VSS_1

PB2

PB0

PC5

PC4

PA4

PA2

PB13

PB12

PB10

PA7

PA6

VDD_4

PA3

VDDA

PB15

PB14

PB11

PB1

VSS_4

PA0- WKUP

VREF+

PC1

PC6

PC7

PC8

PA8

PA5

PA1

VSSA

PC0

PC9

PA9

PA10

PA13

PB8

PC2

OSC_OUT

OSC_IN

PA11

PA12

PA15

PC12

PB7

VBAT

NRST

PC13

BYPASS/ VSS_2

PA14

PC11

PB4

PB6

PB9

PC15

PC14

VDD_2

PC10

PD2

PB3

PB5

BOOT0

VSS_3

VDD_3

Figure 8. STM32F103xC/D/E performance line WLCSP64 ballout, ball side

ai15460b

H

G

F

E

D

C

B

A

1

2

3

4

5

6

7

8

30/143	DS5792 Rev 13

##### Table 5. High-density STM32F103xC/D/E pin definitions

| Pins |  |  |  |  |  | Pin name | Type(1) | I / O Level(2) | Main function(3) (after reset) | Alternate functions(4) |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| LFBGA144 | LFBGA100 | WLCSP64 | LQFP64 | LQFP100 | LQFP144 |  |  |  |  | Default | Remap |
| A3 | A3 | - | - | 1 | 1 | PE2 | I/O | FT | PE2 | TRACECK/ FSMC_A23 | - |
| A2 | B3 | - | - | 2 | 2 | PE3 | I/O | FT | PE3 | TRACED0/FSMC_A19 | - |
| B2 | C3 | - | - | 3 | 3 | PE4 | I/O | FT | PE4 | TRACED1/FSMC_A20 | - |
| B3 | D3 | - | - | 4 | 4 | PE5 | I/O | FT | PE5 | TRACED2/FSMC_A21 | - |
| B4 | E3 | - | - | 5 | 5 | PE6 | I/O | FT | PE6 | TRACED3/FSMC_A22 | - |
| C2 | B2 | C6 | 1 | 6 | 6 | VBAT | S | - | VBAT | - | - |
| A1 | A2 | C8 | 2 | 7 | 7 | PC13-TAMPER- RTC(5) | I/O | - | PC13(6) | TAMPER-RTC | - |
| B1 | A1 | B8 | 3 | 8 | 8 | PC14- OSC32_IN(5) | I/O | - | PC14(6) | OSC32_IN | - |
| C1 | B1 | B7 | 4 | 9 | 9 | PC15- OSC32_OUT(5) | I/O | - | PC15(6) | OSC32_OUT | - |
| C3 | - | - | - | - | 10 | PF0 | I/O | FT | PF0 | FSMC_A0 | - |
| C4 | - | - | - | - | 11 | PF1 | I/O | FT | PF1 | FSMC_A1 | - |
| D4 | - | - | - | - | 12 | PF2 | I/O | FT | PF2 | FSMC_A2 | - |
| E2 | - | - | - | - | 13 | PF3 | I/O | FT | PF3 | FSMC_A3 | - |
| E3 | - | - | - | - | 14 | PF4 | I/O | FT | PF4 | FSMC_A4 | - |
| E4 | - | - | - | - | 15 | PF5 | I/O | FT | PF5 | FSMC_A5 | - |
| D2 | C2 | - | - | 10 | 16 | VSS_5 | S | - | VSS_5 | - | - |
| D3 | D2 | - | - | 11 | 17 | VDD_5 | S | - | VDD_5 | - | - |
| F3 | - | - | - | - | 18 | PF6 | I/O | - | PF6 | ADC3_IN4/FSMC_NIORD | - |
| F2 | - | - | - | - | 19 | PF7 | I/O | - | PF7 | ADC3_IN5/FSMC_NREG | - |
| G3 | - | - | - | - | 20 | PF8 | I/O | - | PF8 | ADC3_IN6/FSMC_NIOWR | - |
| G2 | - | - | - | - | 21 | PF9 | I/O | - | PF9 | ADC3_IN7/FSMC_CD | - |
| G1 | - | - | - | - | 22 | PF10 | I/O | - | PF10 | ADC3_IN8/FSMC_INTR | - |
| D1 | C1 | D8 | 5 | 12 | 23 | OSC_IN | I | - | OSC_IN | - | - |
| E1 | D1 | D7 | 6 | 13 | 24 | OSC_OUT | O | - | OSC_OUT | - | - |
| F1 | E1 | C7 | 7 | 14 | 25 | NRST | I/O | - | NRST | - | - |
| H1 | F1 | E8 | 8 | 15 | 26 | PC0 | I/O | - | PC0 | ADC123_IN10 | - |
| H2 | F2 | F8 | 9 | 16 | 27 | PC1 | I/O | - | PC1 | ADC123_IN11 | - |
| H3 | E2 | D6 | 10 | 17 | 28 | PC2 | I/O | - | PC2 | ADC123_IN12 | - |

DS5792 Rev 13	31/143

##### Table 5. High-density STM32F103xC/D/E pin definitions (continued)

| Pins |  |  |  |  |  | Pin name | Type(1) | I / O Level(2) | Main function(3) (after reset) | Alternate functions(4) |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| LFBGA144 | LFBGA100 | WLCSP64 | LQFP64 | LQFP100 | LQFP144 |  |  |  |  | Default | Remap |
| H4 | F3 | - | 11 | 18 | 29 | PC3(7) | I/O | - | PC3 | ADC123_IN13 | - |
| J1 | G1 | E7 | 12 | 19 | 30 | VSSA | S | - | VSSA | - | - |
| K1 | H1 | - | - | 20 | 31 | VREF- | S | - | VREF- | - | - |
| L1 | J1 | F7 (8) | - | 21 | 32 | VREF+ | S | - | VREF+ | - | - |
| M1 | K1 | G8 | 13 | 22 | 33 | VDDA | S | - | VDDA | - | - |
| J2 | G2 | F6 | 14 | 23 | 34 | PA0-WKUP | I/O | - | PA0 | WKUP/USART2_CTS(9) ADC123_IN0 TIM2_CH1_ETR TIM5_CH1/TIM8_ETR | - |
| K2 | H2 | E6 | 15 | 24 | 35 | PA1 | I/O | - | PA1 | USART2_RTS(9) ADC123_IN1/ TIM5_CH2/TIM2_CH2(9) | - |
| L2 | J2 | H8 | 16 | 25 | 36 | PA2 | I/O | - | PA2 | USART2_TX(9)/TIM5_CH3 ADC123_IN2/ TIM2_CH3 (9) | - |
| M2 | K2 | G7 | 17 | 26 | 37 | PA3 | I/O | - | PA3 | USART2_RX(9)/TIM5_CH4 ADC123_IN3/TIM2_CH4(9) | - |
| G4 | E4 | F5 | 18 | 27 | 38 | VSS_4 | S | - | VSS_4 | - | - |
| F4 | F4 | G6 | 19 | 28 | 39 | VDD_4 | S | - | VDD_4 | - | - |
| J3 | G3 | H7 | 20 | 29 | 40 | PA4 | I/O | - | PA4 | SPI1_NSS(9)/ USART2_CK(9) DAC_OUT1/ADC12_IN4 | - |
| K3 | H3 | E5 | 21 | 30 | 41 | PA5 | I/O | - | PA5 | SPI1_SCK(9) DAC_OUT2 ADC12_IN5 | - |
| L3 | J3 | G5 | 22 | 31 | 42 | PA6 | I/O | - | PA6 | SPI1_MISO(9) TIM8_BKIN/ADC12_IN6 TIM3_CH1(9) | TIM1_BKIN |
| M3 | K3 | G4 | 23 | 32 | 43 | PA7 | I/O | - | PA7 | SPI1_MOSI(9)/ TIM8_CH1N/ADC12_IN7 TIM3_CH2(9) | TIM1_CH1N |
| J4 | G4 | H6 | 24 | 33 | 44 | PC4 | I/O | - | PC4 | ADC12_IN14 | - |
| K4 | H4 | H5 | 25 | 34 | 45 | PC5 | I/O | - | PC5 | ADC12_IN15 | - |
| L4 | J4 | H4 | 26 | 35 | 46 | PB0 | I/O | - | PB0 | ADC12_IN8/TIM3_CH3 TIM8_CH2N | TIM1_CH2N |
| M4 | K4 | F4 | 27 | 36 | 47 | PB1 | I/O | - | PB1 | ADC12_IN9/TIM3_CH4(9) TIM8_CH3N | TIM1_CH3N |

32/143	DS5792 Rev 13

##### Table 5. High-density STM32F103xC/D/E pin definitions (continued)

| Pins |  |  |  |  |  | Pin name | Type(1) | I / O Level(2) | Main function(3) (after reset) | Alternate functions(4) |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| LFBGA144 | LFBGA100 | WLCSP64 | LQFP64 | LQFP100 | LQFP144 |  |  |  |  | Default | Remap |
| J5 | G5 | H3 | 28 | 37 | 48 | PB2 | I/O | FT | PB2/BOOT1 | - | - |
| M5 | - | - | - | - | 49 | PF11 | I/O | FT | PF11 | FSMC_NIOS16 | - |
| L5 | - | - | - | - | 50 | PF12 | I/O | FT | PF12 | FSMC_A6 | - |
| H5 | - | - | - | - | 51 | VSS_6 | S | - | VSS_6 | - | - |
| G5 | - | - | - | - | 52 | VDD_6 | S | - | VDD_6 | - | - |
| K5 | - | - | - | - | 53 | PF13 | I/O | FT | PF13 | FSMC_A7 | - |
| M6 | - | - | - | - | 54 | PF14 | I/O | FT | PF14 | FSMC_A8 | - |
| L6 | - | - | - | - | 55 | PF15 | I/O | FT | PF15 | FSMC_A9 | - |
| K6 | - | - | - | - | 56 | PG0 | I/O | FT | PG0 | FSMC_A10 | - |
| J6 | - | - | - | - | 57 | PG1 | I/O | FT | PG1 | FSMC_A11 | - |
| M7 | H5 | - | - | 38 | 58 | PE7 | I/O | FT | PE7 | FSMC_D4 | TIM1_ETR |
| L7 | J5 | - | - | 39 | 59 | PE8 | I/O | FT | PE8 | FSMC_D5 | TIM1_CH1N |
| K7 | K5 | - | - | 40 | 60 | PE9 | I/O | FT | PE9 | FSMC_D6 | TIM1_CH1 |
| H6 | - | - | - | - | 61 | VSS_7 | S | - | VSS_7 | - | - |
| G6 | - | - | - | - | 62 | VDD_7 | S | - | VDD_7 | - | - |
| J7 | G6 | - | - | 41 | 63 | PE10 | I/O | FT | PE10 | FSMC_D7 | TIM1_CH2N |
| H8 | H6 | - | - | 42 | 64 | PE11 | I/O | FT | PE11 | FSMC_D8 | TIM1_CH2 |
| J8 | J6 | - | - | 43 | 65 | PE12 | I/O | FT | PE12 | FSMC_D9 | TIM1_CH3N |
| K8 | K6 | - | - | 44 | 66 | PE13 | I/O | FT | PE13 | FSMC_D10 | TIM1_CH3 |
| L8 | G7 | - | - | 45 | 67 | PE14 | I/O | FT | PE14 | FSMC_D11 | TIM1_CH4 |
| M8 | H7 | - | - | 46 | 68 | PE15 | I/O | FT | PE15 | FSMC_D12 | TIM1_BKIN |
| M9 | J7 | G3 | 29 | 47 | 69 | PB10 | I/O | FT | PB10 | I2C2_SCL/USART3_TX(9) | TIM2_CH3 |
| M10 | K7 | F3 | 30 | 48 | 70 | PB11 | I/O | FT | PB11 | I2C2_SDA/USART3_RX(9) | TIM2_CH4 |
| H7 | E7 | H2 | 31 | 49 | 71 | VSS_1 | S | - | VSS_1 | - | - |
| G7 | F7 | H1 | 32 | 50 | 72 | VDD_1 | S | - | VDD_1 | - | - |
| M11 | K8 | G2 | 33 | 51 | 73 | PB12 | I/O | FT | PB12 | SPI2_NSS/I2S2_WS/ I2C2_SMBA/ USART3_CK(9)/ TIM1_BKIN(9) | - |
| M12 | J8 | G1 | 34 | 52 | 74 | PB13 | I/O | FT | PB13 | SPI2_SCK/I2S2_CK USART3_CTS(9)/ TIM1_CH1N | - |

DS5792 Rev 13	33/143

##### Table 5. High-density STM32F103xC/D/E pin definitions (continued)

| Pins |  |  |  |  |  | Pin name | Type(1) | I / O Level(2) | Main function(3) (after reset) | Alternate functions(4) |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| LFBGA144 | LFBGA100 | WLCSP64 | LQFP64 | LQFP100 | LQFP144 |  |  |  |  | Default | Remap |
| L11 | H8 | F2 | 35 | 53 | 75 | PB14 | I/O | FT | PB14 | SPI2_MISO/TIM1_CH2N USART3_RTS(9)/ | - |
| L12 | G8 | F1 | 36 | 54 | 76 | PB15 | I/O | FT | PB15 | SPI2_MOSI/I2S2_SD TIM1_CH3N(9)/ | - |
| L9 | K9 | - | - | 55 | 77 | PD8 | I/O | FT | PD8 | FSMC_D13 | USART3_TX |
| K9 | J9 | - | - | 56 | 78 | PD9 | I/O | FT | PD9 | FSMC_D14 | USART3_RX |
| J9 | H9 | - | - | 57 | 79 | PD10 | I/O | FT | PD10 | FSMC_D15 | USART3_CK |
| H9 | G9 | - | - | 58 | 80 | PD11 | I/O | FT | PD11 | FSMC_A16 | USART3_CTS |
| L10 | K10 | - | - | 59 | 81 | PD12 | I/O | FT | PD12 | FSMC_A17 | TIM4_CH1 / USART3_RTS |
| K10 | J10 | - | - | 60 | 82 | PD13 | I/O | FT | PD13 | FSMC_A18 | TIM4_CH2 |
| G8 | - | - | - | - | 83 | VSS_8 | S | - | VSS_8 | - | - |
| F8 | - | - | - | - | 84 | VDD_8 | S | - | VDD_8 | - | - |
| K11 | H10 | - | - | 61 | 85 | PD14 | I/O | FT | PD14 | FSMC_D0 | TIM4_CH3 |
| K12 | G10 | - | - | 62 | 86 | PD15 | I/O | FT | PD15 | FSMC_D1 | TIM4_CH4 |
| J12 | - | - | - | - | 87 | PG2 | I/O | FT | PG2 | FSMC_A12 | - |
| J11 | - | - | - | - | 88 | PG3 | I/O | FT | PG3 | FSMC_A13 | - |
| J10 | - | - | - | - | 89 | PG4 | I/O | FT | PG4 | FSMC_A14 | - |
| H12 | - | - | - | - | 90 | PG5 | I/O | FT | PG5 | FSMC_A15 | - |
| H11 | - | - | - | - | 91 | PG6 | I/O | FT | PG6 | FSMC_INT2 | - |
| H10 | - | - | - | - | 92 | PG7 | I/O | FT | PG7 | FSMC_INT3 | - |
| G11 | - | - | - | - | 93 | PG8 | I/O | FT | PG8 | - | - |
| G10 | - | - | - | - | 94 | VSS_9 | S | - | VSS_9 | - | - |
| F10 | - | - | - | - | 95 | VDD_9 | S | - | VDD_9 | - | - |
| G12 | F10 | E1 | 37 | 63 | 96 | PC6 | I/O | FT | PC6 | I2S2_MCK/ TIM8_CH1/SDIO_D6 | TIM3_CH1 |
| F12 | E10 | E2 | 38 | 64 | 97 | PC7 | I/O | FT | PC7 | I2S3_MCK/ TIM8_CH2/SDIO_D7 | TIM3_CH2 |
| F11 | F9 | E3 | 39 | 65 | 98 | PC8 | I/O | FT | PC8 | TIM8_CH3/SDIO_D0 | TIM3_CH3 |
| E11 | E9 | D1 | 40 | 66 | 99 | PC9 | I/O | FT | PC9 | TIM8_CH4/SDIO_D1 | TIM3_CH4 |
| E12 | D9 | E4 | 41 | 67 | 100 | PA8 | I/O | FT | PA8 | USART1_CK/ TIM1_CH1(9)/MCO | - |

34/143	DS5792 Rev 13

##### Table 5. High-density STM32F103xC/D/E pin definitions (continued)

| Pins |  |  |  |  |  | Pin name | Type(1) | I / O Level(2) | Main function(3) (after reset) | Alternate functions(4) |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| LFBGA144 | LFBGA100 | WLCSP64 | LQFP64 | LQFP100 | LQFP144 |  |  |  |  | Default | Remap |
| D12 | C9 | D2 | 42 | 68 | 101 | PA9 | I/O | FT | PA9 | USART1_TX(9)/ TIM1_CH2(9) | - |
| D11 | D10 | D3 | 43 | 69 | 102 | PA10 | I/O | FT | PA10 | USART1_RX(9)/ TIM1_CH3(9) | - |
| C12 | C10 | C1 | 44 | 70 | 103 | PA11 | I/O | FT | PA11 | USART1_CTS/USBDM CAN_RX(9)/TIM1_CH4(9) | - |
| B12 | B10 | C2 | 45 | 71 | 104 | PA12 | I/O | FT | PA12 | USART1_RTS/USBDP/ CAN_TX(9)/TIM1_ETR(9) | - |
| A12 | A10 | D4 | 46 | 72 | 105 | PA13 | I/O | FT | JTMS-SWDIO | - | PA13 |
| C11 | F8 | - | - | 73 | 106 | Not connected |  |  |  |  | - |
| G9 | E6 | B1 | 47 | 74 | 107 | VSS_2 | S | - | VSS_2 | - | - |
| F9 | F6 | A1 | 48 | 75 | 108 | VDD_2 | S | - | VDD_2 | - | - |
| A11 | A9 | B2 | 49 | 76 | 109 | PA14 | I/O | FT | JTCK-SWCLK | - | PA14 |
| A10 | A8 | C3 | 50 | 77 | 110 | PA15 | I/O | FT | JTDI | SPI3_NSS/ I2S3_WS | TIM2_CH1_ET R PA15 / SPI1_NSS |
| B11 | B9 | A2 | 51 | 78 | 111 | PC10 | I/O | FT | PC10 | UART4_TX/SDIO_D2 | USART3_TX |
| B10 | B8 | B3 | 52 | 79 | 112 | PC11 | I/O | FT | PC11 | UART4_RX/SDIO_D3 | USART3_RX |
| C10 | C8 | C4 | 53 | 80 | 113 | PC12 | I/O | FT | PC12 | UART5_TX/SDIO_CK | USART3_CK |
| E10 | D8 | D8 | 5 | 81 | 114 | PD0 | I/O | FT | OSC_IN(10) | FSMC_D2(11) | CAN_RX |
| D10 | E8 | D7 | 6 | 82 | 115 | PD1 | I/O | FT | OSC_OUT(10) | FSMC_D3(11) | CAN_TX |
| E9 | B7 | A3 | 54 | 83 | 116 | PD2 | I/O | FT | PD2 | TIM3_ETR/UART5_RX SDIO_CMD | - |
| D9 | C7 | - | - | 84 | 117 | PD3 | I/O | FT | PD3 | FSMC_CLK | USART2_CTS |
| C9 | D7 | - | - | 85 | 118 | PD4 | I/O | FT | PD4 | FSMC_NOE | USART2_RTS |
| B9 | B6 | - | - | 86 | 119 | PD5 | I/O | FT | PD5 | FSMC_NWE | USART2_TX |
| E7 | - | - | - | - | 120 | VSS_10 | S | - | VSS_10 | - | - |
| F7 | - | - | - | - | 121 | VDD_10 | S | - | VDD_10 | - | - |
| A8 | C6 | - | - | 87 | 122 | PD6 | I/O | FT | PD6 | FSMC_NWAIT | USART2_RX |
| A9 | D6 | - | - | 88 | 123 | PD7 | I/O | FT | PD7 | FSMC_NE1/FSMC_NCE2 | USART2_CK |
| E8 | - | - | - | - | 124 | PG9 | I/O | FT | PG9 | FSMC_NE2/FSMC_NCE3 | - |
| D8 | - | - | - | - | 125 | PG10 | I/O | FT | PG10 | FSMC_NCE4_1/ FSMC_NE3 | - |

DS5792 Rev 13	35/143

##### Table 5. High-density STM32F103xC/D/E pin definitions (continued)

| Pins |  |  |  |  |  | Pin name | Type(1) | I / O Level(2) | Main function(3) (after reset) | Alternate functions(4) |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| LFBGA144 | LFBGA100 | WLCSP64 | LQFP64 | LQFP100 | LQFP144 |  |  |  |  | Default | Remap |
| C8 | - | - | - | - | 126 | PG11 | I/O | FT | PG11 | FSMC_NCE4_2 | - |
| B8 | - | - | - | - | 127 | PG12 | I/O | FT | PG12 | FSMC_NE4 | - |
| D7 | - | - | - | - | 128 | PG13 | I/O | FT | PG13 | FSMC_A24 | - |
| C7 | - | - | - | - | 129 | PG14 | I/O | FT | PG14 | FSMC_A25 | - |
| E6 | - | - | - | - | 130 | VSS_11 | S | - | VSS_11 | - | - |
| F6 | - | - | - | - | 131 | VDD_11 | S | - | VDD_11 | - | - |
| B7 | - | - | - | - | 132 | PG15 | I/O | FT | PG15 | - | - |
| A7 | A7 | A4 | 55 | 89 | 133 | PB3 | I/O | FT | JTDO | SPI3_SCK / I2S3_CK/ | PB3/TRACES WO TIM2_CH2 / SPI1_SCK |
| A6 | A6 | B4 | 56 | 90 | 134 | PB4 | I/O | FT | NJTRST | SPI3_MISO | PB4 / TIM3_CH1 SPI1_MISO |
| B6 | C5 | A5 | 57 | 91 | 135 | PB5 | I/O | - | PB5 | I2C1_SMBA/ SPI3_MOSI I2S3_SD | TIM3_CH2 / SPI1_MOSI |
| C6 | B5 | B5 | 58 | 92 | 136 | PB6 | I/O | FT | PB6 | I2C1_SCL(9)/ TIM4_CH1(9) | USART1_TX |
| D6 | A5 | C5 | 59 | 93 | 137 | PB7 | I/O | FT | PB7 | I2C1_SDA(9) / FSMC_NADV / TIM4_CH2(9) | USART1_RX |
| D5 | D5 | A6 | 60 | 94 | 138 | BOOT0 | I | - | BOOT0 | - | - |
| C5 | B4 | D5 | 61 | 95 | 139 | PB8 | I/O | FT | PB8 | TIM4_CH3(9)/SDIO_D4 | I2C1_SCL/ CAN_RX |
| B5 | A4 | B6 | 62 | 96 | 140 | PB9 | I/O | FT | PB9 | TIM4_CH4(9)/SDIO_D5 | I2C1_SDA / CAN_TX |
| A5 | D4 | - | - | 97 | 141 | PE0 | I/O | FT | PE0 | TIM4_ETR / FSMC_NBL0 | - |
| A4 | C4 | - | - | 98 | 142 | PE1 | I/O | FT | PE1 | FSMC_NBL1 | - |
| E5 | E5 | A7 | 63 | 99 | 143 | VSS_3 | S | - | VSS_3 | - | - |
| F5 | F5 | A8 | 64 | 10 0 | 144 | VDD_3 | S | - | VDD_3 | - | - |

- I = input, O = output, S = supply.

- FT = 5 V tolerant.

- Function availability depends on the chosen device.

- If several peripherals share the same I/O pin, to avoid conflict between these alternate functions only one peripheral should be enabled at a time through the peripheral clock enable bit (in the corresponding RCC peripheral clock enable register).

36/143	DS5792 Rev 13

- PC13, PC14 and PC15 are supplied through the power switch. Since the switch only sinks a limited amount of current (3 mA), the use of GPIOs PC13 to PC15 in output mode is limited: the speed should not exceed 2 MHz with a maximum load of 30 pF and these IOs must not be used as a current source (e.g. to drive an LED).

- Main function after the first backup domain power-up. Later on, it depends on the contents of the Backup registers even after reset (because these registers are not reset by the main reset). For details on how to manage these IOs, refer to the Battery backup domain and BKP register description sections in the STM32F10xxx reference manual, available from the STMicroelectronics website: www.st.com.

- In the WCLSP64 package, the PC3 I/O pin is not bonded and it must be configured by software to output mode (Push-pull) and writing 0 to the data register in order to avoid an extra consumption during low-power modes.

- Unlike in the LQFP64 package, there is no PC3 in the WLCSP package. The VREF+ functionality is provided instead.

- This alternate function can be remapped by software to some other port pins (if available on the used package). For more details, refer to the Alternate function I/O and debug configuration section in the STM32F10xxx reference manual, available from the STMicroelectronics website: www.st.com.

- For the WCLSP64/LQFP64 package, the pins number 5 and 6 are configured as OSC_IN/OSC_OUT after reset, however the functionality of PD0 and PD1 can be remapped by software on these pins. For the LQFP100/BGA100 and LQFP144/BGA144 packages, PD0 and PD1 are available by default, so there is no need for remapping. For more details, refer to Alternate function I/O and debug configuration section in the STM32F10xxx reference manual.

- For devices delivered in LQFP64 packages, the FSMC function is not available.

DS5792 Rev 13	37/143

##### Table 6. FSMC pin definition

| Pins | FSMC |  |  |  |  | LQFP100 BGA100(1) |
| --- | --- | --- | --- | --- | --- | --- |
|  | CF | CF/IDE | NOR/PSRAM/ SRAM | NOR/PSRAM Mux | NAND 16 bit |  |
| PE2 | - | - | A23 | A23 | - | Yes |
| PE3 | - | - | A19 | A19 | - | Yes |
| PE4 | - | - | A20 | A20 | - | Yes |
| PE5 | - | - | A21 | A21 | - | Yes |
| PE6 | - | - | A22 | A22 | - | Yes |
| PF0 | A0 | A0 | A0 | - | - | - |
| PF1 | A1 | A1 | A1 | - | - | - |
| PF2 | A2 | A2 | A2 | - | - | - |
| PF3 | A3 | - | A3 | - | - | - |
| PF4 | A4 | - | A4 | - | - | - |
| PF5 | A5 | - | A5 | - | - | - |
| PF6 | NIORD | NIORD | - | - | - | - |
| PF7 | NREG | NREG | - | - | - | - |
| PF8 | NIOWR | NIOWR | - | - | - | - |
| PF9 | CD | CD | - | - | - | - |
| PF10 | INTR | INTR | - | - | - | - |
| PF11 | NIOS16 | NIOS16 | - | - | - | - |
| PF12 | A6 | - | A6 | - | - | - |
| PF13 | A7 | - | A7 | - | - | - |
| PF14 | A8 | - | A8 | - | - | - |
| PF15 | A9 | - | A9 | - | - | - |
| PG0 | A10 | - | A10 | - | - | - |
| PG1 | - | - | A11 | - | - | - |
| PE7 | D4 | D4 | D4 | DA4 | D4 | Yes |
| PE8 | D5 | D5 | D5 | DA5 | D5 | Yes |
| PE9 | D6 | D6 | D6 | DA6 | D6 | Yes |
| PE10 | D7 | D7 | D7 | DA7 | D7 | Yes |
| PE11 | D8 | D8 | D8 | DA8 | D8 | Yes |
| PE12 | D9 | D9 | D9 | DA9 | D9 | Yes |
| PE13 | D10 | D10 | D10 | DA10 | D10 | Yes |
| PE14 | D11 | D11 | D11 | DA11 | D11 | Yes |
| PE15 | D12 | D12 | D12 | DA12 | D12 | Yes |
| PD8 | D13 | D13 | D13 | DA13 | D13 | Yes |

38/143	DS5792 Rev 13

##### Table 6. FSMC pin definition (continued)

| Pins | FSMC |  |  |  |  | LQFP100 BGA100(1) |
| --- | --- | --- | --- | --- | --- | --- |
|  | CF | CF/IDE | NOR/PSRAM/ SRAM | NOR/PSRAM Mux | NAND 16 bit |  |
| PD9 | D14 | D14 | D14 | DA14 | D14 | Yes |
| PD10 | D15 | D15 | D15 | DA15 | D15 | Yes |
| PD11 | - | - | A16 | A16 | CLE | Yes |
| PD12 | - | - | A17 | A17 | ALE | Yes |
| PD13 | - | - | A18 | A18 | - | Yes |
| PD14 | D0 | D0 | D0 | DA0 | D0 | Yes |
| PD15 | D1 | D1 | D1 | DA1 | D1 | Yes |
| PG2 | - | - | A12 | - | - | - |
| PG3 | - | - | A13 | - | - | - |
| PG4 | - | - | A14 | - | - | - |
| PG5 | - | - | A15 | - | - | - |
| PG6 | - | - | - | - | INT2 | - |
| PG7 | - | - | - | - | INT3 | - |
| PD0 | D2 | D2 | D2 | DA2 | D2 | Yes |
| PD1 | D3 | D3 | D3 | DA3 | D3 | Yes |
| PD3 | - | - | CLK | CLK | - | Yes |
| PD4 | NOE | NOE | NOE | NOE | NOE | Yes |
| PD5 | NWE | NWE | NWE | NWE | NWE | Yes |
| PD6 | NWAIT | NWAIT | NWAIT | NWAIT | NWAIT | Yes |
| PD7 | - | - | NE1 | NE1 | NCE2 | Yes |
| PG9 | - | - | NE2 | NE2 | NCE3 | - |
| PG10 | NCE4_1 | NCE4_1 | NE3 | NE3 | - | - |
| PG11 | NCE4_2 | NCE4_2 | - | - | - | - |
| PG12 | - | - | NE4 | NE4 | - | - |
| PG13 | - | - | A24 | A24 | - | - |
| PG14 | - | - | A25 | A25 | - | - |
| PB7 | - | - | NADV | NADV | - | Yes |
| PE0 | - | - | NBL0 | NBL0 | - | Yes |
| PE1 | - | - | NBL1 | NBL1 | - | Yes |

  - Ports F and G are not available in devices delivered in 100-pin packages.

DS5792 Rev 13	39/143

# Memory mapping

The memory map is shown in Figure 9.

0x3FFF FFFF

0x2001 0000

0x2000 FFFF

0x2000 0000

0x1FFF F800 - 0x1FFF F80F

0x1FFF F000- 0x1FFF F7FF

0x1FFF EFFF

0x0808 0000

0x0807 FFFF

0x0800 0000

0x07FF FFFF

0x0008 0000

0x0007 FFFF

0x0000 0000

SRAM (64 KB aliased by bit-banding)

Reserved

512-Mbyte block 0 Code

512-Mbyte block 1 SRAM

512-Mbyte block 2 Peripherals

512-Mbyte block 3 FSMC bank1 & bank2

512-Mbyte block 4

FSMC bank 3 & bank4

512-Mbyte block 5

FSMC register

512-Mbyte block 6 Not used

512-Mbyte block 7 Cortex-M3's internal peripherals

0xFFFF FFFF

0xE000 0000

0xDFFF FFFF

0xC000 0000

0xBFFF FFFF

0xA000 0000

0x9FFF FFFF

0x8000 0000

0x7FFF FFFF

0x6000 0000

0x5FFF FFFF

0x4000 0000

0x3FFF FFFF

0x2000 0000

0x1FFF FFFF

0xA000 1000 - 0xBFFF FFFF

0xA000 0000 - 0xA000 0FFF

0x9000 0000 - 0x9FFF FFFF

0x8000 0000 - 0x8FFF FFFF

0x7000 0000 - 0x7FFF FFFF

0x6C00 0000 - 0x6FFF FFFF

0x6800 0000 - 0x6BFF FFFF

0x6400 0000 - 0x67FF FFFF

0x6000 0000 - 0x63FF FFFF

0x4002 4400 - 0x5FFF FFFF

0x4002 3000 - 0x4002 33FF

0x4002 2400 - 0x4002 2FFF

0x4002 2000 - 0x4002 23FF

0x4002 1400 - 0x4002 1FFF

0x4002 1000 - 0x4002 13FF

0x4002 0400 - 0x4002 0FFF

0x4002 0400 - 0x4002 07FF

0x4002 0000 - 0x4002 03FF

0x4001 8400 - 0x4001 FFFF

0x4001 8000 - 0x4001 83FF

0x4001 400 - 0x4001 7FFF

0x4001 3C00 - 0x4001 3FFF

0x4001 3800 - 0x4001 3BFF

0x4001 3400 - 0x4001 37FF

0x4001 3000 - 0x4001 33FF

0x4001 2C00 - 0x4001 2FFF

0x4001 2800 - 0x4001 2BFF

0x4001 2400 - 0x4001 27FF

0x4001 2000 - 0x4001 23FF

0x4001 1C00 - 0x4001 1FFF

0x4001 1800 - 0x4001 1BFF

0x4001 1400 - 0x4001 17FF

0x4001 1000 - 0x4001 13FF

0x4001 0C00 - 0x4001 0FFF

0x4001 0800 - 0x4001 0BFF

0x4001 0400 - 0x4001 07FF

0x4001 0000 - 0x4001 03FF

0x4000 7800 - 0x4000 FFFF

0x4000 7400 - 0x4000 77FF

0x4000 7000 - 0x4000 73FF

0x4000 6C00 - 0x4000 6FFF

0x4000 6800 - 0x4000 6BFF

0x4000 6400 - 0x4000 67FF

0x4000 6000 - 0x4000 63FF

0x4000 5C00 - 0x4000 5FFF

0x4000 5800 - 0x4000 5BFF

0x4000 5400 - 0x4000 57FF

0x4000 5000 - 0x4000 53FF

0x4000 4C00 - 0x4000 4FFF

0x4000 4800 - 0x4000 4BFF

0x4000 4400 - 0x4000 47FF

0x4000 4000 - 0x4000 43FF

0x4000 3C00 - 0x4000 3FFF

0x4000 3800 - 0x4000 3BFF

0x4000 3400 - 0x4000 37FF

0x4000 3000 - 0x4000 33FF

0x4000 2C00 - 0x4000 2FFF

0x4000 2800 - 0x4000 2BFF

0x4000 1800 - 0x4000 27FF

0x4000 1400 - 0x4000 17FF

0x4000 1000 - 0x4000 13FF

0x4000 0C00 - 0x4000 0FFF

0x4000 0800 - 0x4000 0BFF

0x4000 0400 - 0x4000 07FF

0x4000 0000 - 0x4000 03FF

0x0000 0000

ai14753d

Figure 9. Memory map

Reserved

FSMC register FSMC bank4 PCCARD

FSMC bank3 NAND (NAND2)

FSMC bank2 NAND (NAND1)

FSMC bank1 NOR/PSRAM 4

FSMC bank1 NOR/PSRAM 3

FSMC bank1 NOR/PSRAM 2

FSMC bank1 NOR/PSRAM 1 Reserved

CRC

Reserved

Flash interface

Reserved

RCC

Reserved

DMA2

DMA1

Reserved

SDIO

Reserved

ADC3

USART1

TIM8

SPI1

TIM1

ADC2

ADC1

Port G

Port F

Port E

Port D

Port C

Port B

Port A

EXTI

AFIO

Reserved

DAC

PWR

BKP

Reserved

BxCAN

Shared USB/CAN SRAM 512 bytes

USB registers

I2C2

I2C1

UART5

UART4

USART3

USART2

Reserved

SPI3/I2S3

SPI2/I2S2

Reserved

IWDG

WWDG

RTC

Reserved

TIM7

TIM6

TIM5

TIM4

TIM3

TIM2

Option Bytes

System memory

Reserved

Flash

Reserved

Aliased to Flash or system memory depending on BOOT pins

40/143	DS5792 Rev 13

# Electrical characteristics

## Parameter conditions

Unless otherwise specified, all voltages are referenced to VSS.

### Minimum and maximum values

Unless otherwise specified the minimum and maximum values are guaranteed in the worst conditions of ambient temperature, supply voltage and frequencies by tests in production on 100% of the devices with an ambient temperature at TA = 25 °C and TA = TAmax (given by the selected temperature range).

Data based on characterization results, design simulation and/or technology characteristics are indicated in the table footnotes and are not tested in production. Based on characterization, the minimum and maximum values refer to sample tests and represent the mean value plus or minus three times the standard deviation (mean±3).

### Typical values

Unless otherwise specified, typical data are based on TA = 25 °C, VDD = 3.3 V (for the 2 V ≤ VDD ≤ 3.6 V voltage range). They are given only as design guidelines and are not tested.

Typical ADC accuracy values are determined by characterization of a batch of samples from a standard diffusion lot over the full temperature range, where 95% of the devices have an error less than or equal to the value indicated (mean±2).

### Typical curves

Unless otherwise specified, all typical curves are given only as design guidelines and are not tested.

### Loading capacitor

The loading conditions used for pin parameter measurement are shown in Figure 10.

### Pin input voltage

The input voltage measurement on a pin of the device is described in Figure 11.

-31901062

6).

-#5 PIN

Figure 11. Pin input voltage

-31901162

# = 50 P&

-#5 PIN

Figure 10. Pin loading conditions

DS5792 Rev 13	41/143

### Power supply scheme

s^^A

AŶĂůŽŐ͗ ZCƐ͕ W>>͕

͘͘͘

ADCͬ DAC

ZE&Ͳ

s

ϭϬ Ŷ& н ϭ ђ&

ϭϬ Ŷ& н ϭ ђ&

sZE&	sZE&н

s^^ϭͬϮͬͬ͘͘͘ϭϭ

sDDA

ϭϭ п ϭ ϬϬ Ŷ& н ϭ п ϰ͘ϳ ђ&

sDD

ZĞŐ ƵůĂƚŽƌ

sDD

sDDϭͬϮͬͬ͘͘͘ϭϭ

/E

/K

>ŽŐŝĐ

'W /ͬK Ɛ

Khd

WŽ ǁĞƌ Ɛǁŝ ƚĐŚ

ϭ͘ϴͲϯ͘ϲs

sBAd

<ĞƌŶĞů ůŽŐŝĐ

;CWh͕   DŝŐŝƚĂů   Θ DĞŵŽƌŝĞƐͿ

BĂĐŬƵƉ ĐŝƌĐƵŝƚƌǇ

;K^CϯϮ<͕ZdC͕ tĂŬĞͲƵƉ ůŽŐŝĐ

BĂĐŬƵƉ ƌĞŐŝƐƚĞƌƐͿ

ĂŝϭϱϰϬϭ

>ĞǀĞů ƐŚŝĨƚĞƌ

Figure 12. Power supply scheme

Caution:	In Figure 12, the 4.7 µF capacitor must be connected to VDD3.

### Current consumption measurement

9''$

9''

,''

,''B9%$7

9%$7

Figure 13. Current consumption measurement scheme

DL14126

42/143	DS5792 Rev 13

## Absolute maximum ratings

Stresses above the absolute maximum ratings listed in Table 7: Voltage characteristics, Table 8: Current characteristics, and Table 9: Thermal characteristics may cause permanent damage to the device. These are stress ratings only and functional operation of the device at these conditions is not implied. Exposure to maximum rating conditions for extended periods may affect device reliability.

##### Table 7. Voltage characteristics

| Symbol | Ratings | Min | Max | Unit |
| --- | --- | --- | --- | --- |
| VDD–VSS | External main supply voltage (including VDDA and VDD)(1) | –0.3 | 4.0 | V |
| V (2) IN | Input voltage on five volt tolerant pin | VSS 0.3 | VDD + 4.0 |  |
|  | Input voltage on any other pin | VSS 0.3 | 4.0 |  |
| ¦ΔVDDx¦ | Variations between different VDD power pins | - | 50 | mV |
| ¦VSSX VSS¦ | Variations between all the different ground pins(3) | - | 50 |  |
| VESD(HBM) | Electrostatic discharge voltage (human body model) | see Section 5.3.12: Absolute maximum ratings (electrical sensitivity) |  | - |

- All main power (VDD, VDDA) and ground (VSS, VSSA) pins must always be connected to the external power supply, in the permitted range.

- VIN maximum must always be respected. Refer to Table 8: Current characteristics for the maximum allowed injected current values.

- Include VREF- pin.

##### Table 8. Current characteristics

| Symbol | Ratings | Max. | Unit |
| --- | --- | --- | --- |
| IVDD | Total current into VDD/VDDA power lines (source)(1) | 150 | mA |
| IVSS | Total current out of VSS ground lines (sink)(1) | 150 |  |
| IIO | Output current sunk by any I/O and control pin | 25 |  |
|  | Output current source by any I/Os and control pin | 25 |  |
| IINJ(PI (2) N) | Injected current on five volt tolerant pins(3) | -5/+0 |  |
|  | Injected current on any other pin(4) | ± 5 |  |
| IINJ(PIN) | Total injected current (sum of all I/O and control pins)(5) | ± 25 |  |

- All main power (VDD, VDDA) and ground (VSS, VSSA) pins must always be connected to the external power supply, in the permitted range.

- Negative injection disturbs the analog performance of the device. See note 3 below Table 62 on page 108.

- Positive injection is not possible on these I/Os. A negative injection is induced by VIN<VSS. IINJ(PIN) must never be exceeded. Refer to Table 7: Voltage characteristics for the maximum allowed input voltage values.

- A positive injection is induced by VIN>VDD while a negative injection is induced by VIN<VSS. IINJ(PIN) must never be exceeded. Refer to Table 7: Voltage characteristics for the maximum allowed input voltage values.

- When several inputs are submitted to a current injection, the maximum IINJ(PIN) is the absolute sum of the positive and negative injected currents (instantaneous values).

DS5792 Rev 13	43/143

##### Table 9. Thermal characteristics

| Symbol | Ratings | Value | Unit |
| --- | --- | --- | --- |
| TSTG | Storage temperature range | –65 to +150 | °C |
| TJ | Maximum junction temperature | 150 | °C |

## Operating conditions

### General operating conditions

Table 10. General operating conditions

| Symbol | Parameter | Conditions | Min | Max | Unit |
| --- | --- | --- | --- | --- | --- |
| fHCLK | Internal AHB clock frequency | - | 0 | 72 | MHz |
| fPCLK1 | Internal APB1 clock frequency | - | 0 | 36 |  |
| fPCLK2 | Internal APB2 clock frequency | - | 0 | 72 |  |
| VDD | Standard operating voltage | - | 2 | 3.6 | V |
| VDDA(1) | Analog operating voltage (ADC not used) | Must be the same potential as VDD(2) | 2 | 3.6 | V |
|  | Analog operating voltage (ADC used) |  | 2.4 | 3.6 |  |
| VBAT | Backup operating voltage | - | 1.8 | 3.6 | V |
| PD | Power dissipation at TA = 85 °C for suffix 6 or TA = 105 °C for suffix 7(3) | LQFP144 | - | 666 | mW |
|  |  | LQFP100 | - | 434 |  |
|  |  | LQFP64 | - | 444 |  |
|  |  | LFBGA100 | - | 500 |  |
|  |  | LFBGA144 | - | 500 |  |
|  |  | WLCSP64 | - | 400 |  |
| TA | Ambient temperature for 6 suffix version | Maximum power dissipation | -40 | 85 | °C |
|  |  | Low-power dissipation(4) | -40 | 105 |  |
|  | Ambient temperature for 7 suffix version | Maximum power dissipation | -40 | 105 | °C |
|  |  | Low-power dissipation(4) | -40 | 125 |  |
| TJ | Junction temperature range | 6 suffix version | -40 | 105 | °C |
|  |  | 7 suffix version | -40 | 125 |  |

      - When the ADC is used, refer to Table 59: ADC characteristics.

      - It is recommended to power VDD and VDDA from the same source. A maximum difference of 300 mV between VDD and VDDA can be tolerated during power-up and operation.

      - If TA is lower, higher PD values are allowed as long as TJ does not exceed TJmax (see Table 6.7: Thermal characteristics on page 132).

      - In low-power dissipation state, TA can be extended to this range as long as TJ does not exceed TJmax (see

Table 6.7: Thermal characteristics on page 132).

44/143	DS5792 Rev 13

### Operating conditions at power-up / power-down

The parameters given in Table 11 are derived from tests performed under the ambient temperature condition summarized in Table 10.

##### Table 11. Operating conditions at power-up / power-down

| Symbol | Parameter | Conditions | Min | Max | Unit |
| --- | --- | --- | --- | --- | --- |
| tVDD | VDD rise time rate | - | 0 | ∞ | µs/V |
|  | VDD fall time rate |  | 20 | ∞ |  |

### Embedded reset and power control block characteristics

The parameters given in Table 12 are derived from tests performed under ambient temperature and VDD supply voltage conditions summarized in Table 10.

##### Table 12. Embedded reset and power control block characteristics

| Symbol | Parameter | Conditions | Min | Typ | Max | Unit |
| --- | --- | --- | --- | --- | --- | --- |
| VPVD | Programmable voltage detector level selection | PLS[2:0]=000 (rising edge) | 2.1 | 2.18 | 2.26 | V |
|  |  | PLS[2:0]=000 (falling edge) | 2 | 2.08 | 2.16 |  |
|  |  | PLS[2:0]=001 (rising edge) | 2.19 | 2.28 | 2.37 |  |
|  |  | PLS[2:0]=001 (falling edge) | 2.09 | 2.18 | 2.27 |  |
|  |  | PLS[2:0]=010 (rising edge) | 2.28 | 2.38 | 2.48 |  |
|  |  | PLS[2:0]=010 (falling edge) | 2.18 | 2.28 | 2.38 |  |
|  |  | PLS[2:0]=011 (rising edge) | 2.38 | 2.48 | 2.58 |  |
|  |  | PLS[2:0]=011 (falling edge) | 2.28 | 2.38 | 2.48 |  |
|  |  | PLS[2:0]=100 (rising edge) | 2.47 | 2.58 | 2.69 |  |
|  |  | PLS[2:0]=100 (falling edge) | 2.37 | 2.48 | 2.59 |  |
|  |  | PLS[2:0]=101 (rising edge) | 2.57 | 2.68 | 2.79 |  |
|  |  | PLS[2:0]=101 (falling edge) | 2.47 | 2.58 | 2.69 |  |
|  |  | PLS[2:0]=110 (rising edge) | 2.66 | 2.78 | 2.9 |  |
|  |  | PLS[2:0]=110 (falling edge) | 2.56 | 2.68 | 2.8 |  |
|  |  | PLS[2:0]=111 (rising edge) | 2.76 | 2.88 | 3 |  |
|  |  | PLS[2:0]=111 (falling edge) | 2.66 | 2.78 | 2.9 |  |
| (2) VPVDhyst | PVD hysteresis | - | - | 100 | - | mV |
| VPOR/PDR | Power on/power down reset threshold | Falling edge | 1.8(1) | 1.88 | 1.96 | V |
|  |  | Rising edge | 1.84 | 1.92 | 2.0 |  |
| VPDRhyst(2) | PDR hysteresis | - | - | 40 | - | mV |
| TRSTTEMPO(2) | Reset temporization | - | 1 | 2.5 | 4.5 | ms |

      - The product behavior is guaranteed by design down to the minimum VPOR/PDR value.

      - Guaranteed by design.

DS5792 Rev 13	45/143

### Embedded reference voltage

The parameters given in Table 13 are derived from tests performed under ambient temperature and VDD supply voltage conditions summarized in Table 10.

##### Table 13. Embedded internal reference voltage

| Symbol | Parameter | Conditions | Min | Typ | Max | Unit |
| --- | --- | --- | --- | --- | --- | --- |
| VREFINT | Internal reference voltage | –40 °C < TA < +105 °C | 1.16 | 1.20 | 1.26 | V |
|  |  | –40 °C < TA < +85 °C | 1.16 | 1.20 | 1.24 |  |
| TS_vrefint(1) | ADC sampling time when reading the internal reference voltage | - | - | 5.1 | 17.1(2) | µs |
| VRERINT(2) | Internal reference voltage spread over the temperature range | VDD = 3 V ±10 mV | - | - | 10 | mV |
| (2) TCoeff | Temperature coefficient | - | - | - | 100 | ppm/°C |

      - Shortest sampling time can be determined in the application by multiple iterations.

      - Guaranteed by design.

### Supply current characteristics

The current consumption is a function of several parameters and factors such as the operating voltage, ambient temperature, I/O pin loading, device software configuration, operating frequencies, I/O pin switching rate, program location in memory and executed binary code.

The current consumption is measured as described in Figure 13: Current consumption measurement scheme.

All Run-mode current consumption measurements given in this section are performed with a reduced code that gives a consumption equivalent to Dhrystone 2.1 code.

#### Maximum current consumption

The MCU is placed under the following conditions:

- All I/O pins are in input mode with a static value at VDD or VSS (no load)

- All peripherals are disabled except when explicitly mentioned

- The Flash memory access time is adjusted to the fHCLK frequency (0 wait state from 0 to 24 MHz, 1 wait state from 24 to 48 MHz and 2 wait states above)

- Prefetch in ON (reminder: this bit must be set before clock setting and bus prescaling)

- When the peripherals are enabled fPCLK1 = fHCLK/2, fPCLK2 = fHCLK

The parameters given in Table 14, Table 15 and Table 16 are derived from tests performed under ambient temperature and VDD supply voltage conditions summarized in Table 10.

46/143	DS5792 Rev 13

##### Table 14. Maximum current consumption in Run mode, code with data processing running from Flash

| Symbol | Parameter | Conditions | fHCLK | Max(1) |  | Unit |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | TA = 85 °C | TA = 105 °C |  |
| IDD | Supply current in Run mode | External clock(2), all peripherals enabled | 72 MHz | 69 | 70 | mA |
|  |  |  | 48 MHz | 50 | 50.5 |  |
|  |  |  | 36 MHz | 39 | 39.5 |  |
|  |  |  | 24 MHz | 27 | 28 |  |
|  |  |  | 16 MHz | 20 | 20.5 |  |
|  |  |  | 8 MHz | 11 | 11.5 |  |
|  |  | External clock(2), all peripherals disabled | 72 MHz | 37 | 37.5 |  |
|  |  |  | 48 MHz | 28 | 28.5 |  |
|  |  |  | 36 MHz | 22 | 22.5 |  |
|  |  |  | 24 MHz | 16.5 | 17 |  |
|  |  |  | 16 MHz | 12.5 | 13 |  |
|  |  |  | 8 MHz | 8 | 8 |  |

      - Guaranteed by characterization results.

      - External clock is 8 MHz and PLL is on when fHCLK > 8 MHz.

CIAO

Table 15. Maximum current consumption in Run mode, code with data processing running from RAM

| Symbol | Parameter | Conditions | fHCLK | Max(1) |  | Unit |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | TA = 85 °C | TA = 105 °C |  |
| IDD | Supply current in Run mode | External clock(2), all peripherals enabled | 72 MHz | 66 | 67 | mA |
|  |  |  | 48 MHz | 43.5 | 45.5 |  |
|  |  |  | 36 MHz | 33 | 35 |  |
|  |  |  | 24 MHz | 23 | 24.5 |  |
|  |  |  | 16 MHz | 16 | 18 |  |
|  |  |  | 8 MHz | 9 | 10.5 |  |
|  |  | External clock(2), all peripherals disabled | 72 MHz | 33 | 33.5 |  |
|  |  |  | 48 MHz | 23 | 23.5 |  |
|  |  |  | 36 MHz | 18 | 18.5 |  |
|  |  |  | 24 MHz | 13 | 13.5 |  |
|  |  |  | 16 MHz | 10 | 10.5 |  |
|  |  |  | 8 MHz | 6 | 6.5 |  |

- Guaranteed by characterization results at VDD max, fHCLK max.

- External clock is 8 MHz and PLL is on when fHCLK > 8 MHz.

DS5792 Rev 13	47/143

105

85

70

4EMPERATURE ( #)

25

-45

10

0

72-(Z

48-(Z

36-(Z

24-(Z

16-(Z

8-(Z

70

60

50

40

30

20

#ONSUMPTION (M!)

Figure 14. Typical current consumption in Run mode versus frequency (at 3.6 V) - code with data processing running from RAM, peripherals enabled

AI18569

AI18520

105

85

70

4EMPERATURE ( #)

25

-45

10

5

0

40

35

30

25

20

15

#ONSUMPTION (M!)

Figure 15. Typical current consumption in Run mode versus frequency (at 3.6 V)- code with data processing running from RAM, peripherals disabled

72-(Z

48-(Z

36-(Z

24-(Z

16-(Z

8-(Z

48/143	DS5792 Rev 13

##### Table 16. Maximum current consumption in Sleep mode, code running from Flash or RAM

| Symbol | Parameter | Conditions | fHCLK | Max(1) |  | Unit |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | TA = 85 °C | TA = 105 °C |  |
| IDD | Supply current in Sleep mode | External clock(2), all peripherals enabled | 72 MHz | 45 | 46 | mA |
|  |  |  | 48 MHz | 31 | 32 |  |
|  |  |  | 36 MHz | 24 | 25 |  |
|  |  |  | 24 MHz | 17 | 17.5 |  |
|  |  |  | 16 MHz | 12.5 | 13 |  |
|  |  |  | 8 MHz | 8 | 8 |  |
|  |  | External clock(2), all peripherals disabled | 72 MHz | 8.5 | 9 |  |
|  |  |  | 48 MHz | 7 | 7.5 |  |
|  |  |  | 36 MHz | 6 | 6.5 |  |
|  |  |  | 24 MHz | 5 | 5.5 |  |
|  |  |  | 16 MHz | 4.5 | 5 |  |
|  |  |  | 8 MHz | 4 | 4 |  |

- Guaranteed by characterization results at VDD max, fHCLK max with peripherals enabled.

- External clock is 8 MHz and PLL is on when fHCLK > 8 MHz.

DS5792 Rev 13	49/143

##### Table 17. Typical and maximum current consumptions in Stop and Standby modes

| Symbol | Parameter | Conditions | Typ(1) |  |  | Max |  | Unit |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | VDD/VBAT = 2.0 V | VDD/VBAT = 2.4 V | VDD/VBAT = 3.3 V | TA = 85 °C | TA = 105 °C |  |
| IDD | Supply current in Stop mode | Regulator in run mode, low- speed and high-speed internal RC oscillators and high-speed oscillator OFF (no independent watchdog) | - | 34.5 | 35 | 379 | 1130 | µA |
|  |  | Regulator in low-power mode, low-speed and high-speed internal RC oscillators and high- speed oscillator OFF (no independent watchdog) | - | 24.5 | 25 | 365 | 1110 |  |
|  | Supply current in Standby mode | Low-speed internal RC oscillator and independent watchdog ON | - | 3 | 3.8 | - | - |  |
|  |  | Low-speed internal RC oscillator ON, independent watchdog OFF | - | 2.8 | 3.6 | - | - |  |
|  |  | Low-speed internal RC oscillator and independent watchdog OFF, low-speed oscillator and RTC OFF | - | 1.9 | 2.1 | 5(2) | 6.5(2) |  |
| IDD_VBAT | Backup domain supply current | Low-speed oscillator and RTC ON | 1.05 | 1.1 | 1.4 | 2(2) | 2.3(2) |  |

- Typical values are measured at TA = 25 °C.

- Guaranteed by characterization results.

50/143	DS5792 Rev 13

DL17337

7HPSHUDWXUH (&)

105

85

25

±45

0

0.5

1.8 9 2 9

2.4 9

3.3 9

3.6 9

1.5

1

2

2.5

&RQVXPSWLRQ ($)

Figure 16. Typical current consumption on VBAT with RTC on vs. temperature at different VBAT values

600

500

400

!)

ȝ

N		 2.46

(

O   I

P 300	2.76

T

M

U		 3.06

S N

#O 		 3.36

200

3.66

100

0

-45#	25#	85#	105#

4EMPERATURE ( #)

AI18521

AI18521

4EMPERATURE ( #)

105#

85#

25#

-45#

0

100

2.46

2.76

3.06

3.36

3.66

300

200

400

500

600

#ONSUMPTION (ȝ!)

Figure 17. Typical current consumption in Stop mode with regulator in run mode versus temperature at different VDD values

DS5792 Rev 13	51/143

AI18522

4EMPERATURE ( #)

105#

85#

25#

-45#

0

100

300

200

400

500

600

#ONSUMPTION (ȝ!)

Figure 18. Typical current consumption in Stop mode with regulator in low-power mode versus temperature at different VDD values

2.46

2.76

3.06

3.36

3.66

AI18523

4EMPERATURE ( #)

85#

70#

25#

-45#

0.5

0

4.5

4

3.5

3

2.5

2

1.5

1

#ONSUMPTION (ȝ!)

Figure 19. Typical current consumption in Standby mode versus temperature at different VDD values

2.46

2.76

3.06

3.36

3.66

52/143	DS5792 Rev 13

#### Typical current consumption

The MCU is placed under the following conditions:

  - All I/O pins are in input mode with a static value at VDD or VSS (no load).

  - All peripherals are disabled except if it is explicitly mentioned.

  - The Flash access time is adjusted to fHCLK frequency (0 wait state from 0 to 24 MHz, 1 wait state from 24 to 48 MHZ and 2 wait states above).

  - Ambient temperature and VDD supply voltage conditions summarized in Table 10.

  - Prefetch is ON (Reminder: this bit must be set before clock setting and bus prescaling) When the peripherals are enabled fPCLK1 = fHCLK/4, fPCLK2 = fHCLK/2, fADCCLK = fPCLK2/4

##### Table 18. Typical current consumption in Run mode, code with data processing running from Flash

| Symbol | Parameter | Conditions | fHCLK | Typ(1) |  | Unit |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | All peripherals enabled(2) | All peripherals disabled |  |
| IDD | Supply current in Run mode | External clock(3) | 72 MHz | 51 | 30.5 | mA |
|  |  |  | 48 MHz | 34.6 | 20.7 |  |
|  |  |  | 36 MHz | 26.6 | 16.2 |  |
|  |  |  | 24 MHz | 18.5 | 11.4 |  |
|  |  |  | 16 MHz | 12.8 | 8.2 |  |
|  |  |  | 8 MHz | 7.2 | 5 |  |
|  |  |  | 4 MHz | 4.2 | 3.1 |  |
|  |  |  | 2 MHz | 2.7 | 2.1 |  |
|  |  |  | 1 MHz | 2 | 1.7 |  |
|  |  |  | 500 kHz | 1.6 | 1.4 |  |
|  |  |  | 125 kHz | 1.3 | 1.2 |  |
|  |  | Running on high speed internal RC (HSI), AHB prescaler used to reduce the frequency | 64 MHz | 45 | 27 | mA |
|  |  |  | 48 MHz | 34 | 20.1 |  |
|  |  |  | 36 MHz | 26 | 15.6 |  |
|  |  |  | 24 MHz | 17.9 | 10.8 |  |
|  |  |  | 16 MHz | 12.2 | 7.6 |  |
|  |  |  | 8 MHz | 6.6 | 4.4 |  |
|  |  |  | 4 MHz | 3.6 | 2.5 |  |
|  |  |  | 2 MHz | 2.1 | 1.5 |  |
|  |  |  | 1 MHz | 1.4 | 1.1 |  |
|  |  |  | 500 kHz | 1 | 0.8 |  |
|  |  |  | 125 kHz | 0.7 | 0.6 |  |

- Typical values are measures at TA = 25 °C, VDD = 3.3 V.

DS5792 Rev 13	53/143

- Add an additional power consumption of 0.8 mA per ADC for the analog part. In applications, this consumption occurs only while the ADC is on (ADON bit is set in the ADC_CR2 register).

- External clock is 8 MHz and PLL is on when fHCLK > 8 MHz.

##### Table 19. Typical current consumption in Sleep mode, code running from Flash or RAM

| Symbol | Parameter | Conditions | fHCLK | Typ(1) |  | Unit |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | All peripherals enabled(2) | All peripherals disabled |  |
| IDD | Supply current in Sleep mode | External clock(3) | 72 MHz | 29.5 | 6.4 | mA |
|  |  |  | 48 MHz | 20 | 4.6 |  |
|  |  |  | 36 MHz | 15.1 | 3.6 |  |
|  |  |  | 24 MHz | 10.4 | 2.6 |  |
|  |  |  | 16 MHz | 7.2 | 2 |  |
|  |  |  | 8 MHz | 3.9 | 1.3 |  |
|  |  |  | 4 MHz | 2.6 | 1.2 |  |
|  |  |  | 2 MHz | 1.85 | 1.15 |  |
|  |  |  | 1 MHz | 1.5 | 1.1 |  |
|  |  |  | 500 kHz | 1.3 | 1.05 |  |
|  |  |  | 125 kHz | 1.2 | 1.05 |  |
|  |  | Running on high speed internal RC (HSI), AHB prescaler used to reduce the frequency | 64 MHz | 25.6 | 5.1 |  |
|  |  |  | 48 MHz | 19.4 | 4 |  |
|  |  |  | 36 MHz | 14.5 | 3 |  |
|  |  |  | 24 MHz | 9.8 | 2 |  |
|  |  |  | 16 MHz | 6.6 | 1.4 |  |
|  |  |  | 8 MHz | 3.3 | 0.7 |  |
|  |  |  | 4 MHz | 2 | 0.6 |  |
|  |  |  | 2 MHz | 1.25 | 0.55 |  |
|  |  |  | 1 MHz | 0.9 | 0.5 |  |
|  |  |  | 500 kHz | 0.7 | 0.45 |  |
|  |  |  | 125 kHz | 0.6 | 0.45 |  |

- Typical values are measures at TA = 25 °C, VDD = 3.3 V.

- Add an additional power consumption of 0.8 mA per ADC for the analog part. In applications, this consumption occurs only while the ADC is on (ADON bit is set in the ADC_CR2 register).

- External clock is 8 MHz and PLL is on when fHCLK > 8 MHz.

54/143	DS5792 Rev 13

#### On-chip peripheral current consumption

The current consumption of the on-chip peripherals is given in Table 20. The MCU is placed under the following conditions:

  - all I/O pins are in input mode with a static value at VDD or VSS (no load)

  - all peripherals are disabled unless otherwise mentioned

  - the given value is calculated by measuring the current consumption

    - with all peripherals clocked off

    - with only one peripheral clocked on

  - ambient operating temperature and VDD supply voltage conditions summarized in

Table 7

##### Table 20. Peripheral current consumption

| Peripheral |  | Current consumption | Unit |
| --- | --- | --- | --- |
| AHB (up to 72 MHz) | DMA1 | 20,42 | μA/MHz |
|  | DMA2 | 19,03 |  |
|  | FSMC | 52,36 |  |
|  | CRC | 2,36 |  |
|  | SDIO | 33,33 |  |
|  | BusMatrix(1) | 9,72 |  |

DS5792 Rev 13	55/143

##### Table 20. Peripheral current consumption (continued)

| Peripheral |  | Current consumption | Unit |
| --- | --- | --- | --- |
| APB1 (up to 36 MHz) | APB1-Bridge | 7,78 | μA/MHz |
|  | TIM2 | 33,06 |  |
|  | TIM3 | 31,94 |  |
|  | TIM4 | 31,67 |  |
|  | TIM5 | 31,94 |  |
|  | TIM6 | 8,06 |  |
|  | TIM7 | 8,06 |  |
|  | SPI2/I2S2(2) | 8,33 |  |
|  | SPI3/I2S3(2) | 8,33 |  |
|  | USART2 | 12,22 |  |
|  | USART3 | 12,22 |  |
|  | UART4 | 12,22 |  |
|  | UART5 | 12,22 |  |
|  | I2C1 | 10,28 |  |
|  | I2C2 | 10,00 |  |
|  | USB | 18,06 |  |
|  | CAN1 | 18,33 |  |
|  | DAC(3) | 8,06 |  |
|  | WWDG | 3,89 |  |
|  | PWR | 1,11 |  |
|  | BKP | 1,11 |  |
|  | IWDG | 5,28 |  |

56/143	DS5792 Rev 13

##### Table 20. Peripheral current consumption (continued)

| Peripheral |  | Current consumption | Unit |
| --- | --- | --- | --- |
| APB2 (up to 72 MHz) | APB2-Bridge | 4,17 | μA/MHz |
|  | GPIOA | 8,47 |  |
|  | GPIOB | 8,47 |  |
|  | GPIOC | 6,53 |  |
|  | GPIOD | 8,47 |  |
|  | GPIOE | 6,53 |  |
|  | GPIOF | 6,53 |  |
|  | GPIOG | 6,11 |  |
|  | SPI1 | 4,72 |  |
|  | USART1 | 12,50 |  |
|  | TIM1 | 22,92 |  |
|  | TIM8 | 22,92 |  |
|  | ADC1(4) | 17,32 |  |
|  | ADC2(4) | 15,18 |  |
|  | ADC3(4) | 14,82 |  |

- The BusMatrix is automatically active when at least one master is ON. (CPU, DMA1 or DMA2).

- When the I2S is enabled, a current consumption equal to 0.02 mA must be added.

- When DAC_OU1 or DAC_OUT2 is enabled, a current consumption equal to 0.36 mA must be added.

- Specific conditions for measuring ADC current consumption: fHCLK = 56 MHz, fAPB1 = fHCLK/2, fAPB2 = fHCLK, fADCCLK = fAPB2/4. When ADON bit in the ADCx_CR2 register is set to 1, a current consumption of analog part equal to 0.54 mA must be added for each ADC.

DS5792 Rev 13	57/143

### External clock source characteristics

#### High-speed external user clock generated from an external source

The characteristics given in Table 21 result from tests performed using an high-speed external clock source, and under ambient temperature and supply voltage conditions summarized in Table 10.

##### Table 21. High-speed external user clock characteristics

| Symbol | Parameter | Conditions | Min | Typ | Max | Unit |
| --- | --- | --- | --- | --- | --- | --- |
| fHSE_ext | User external clock source frequency(1) | - | 1 | 8 | 25 | MHz |
| VHSEH | OSC_IN input pin high level voltage |  | 0.7VDD | - | VDD | V |
| VHSEL | OSC_IN input pin low level voltage |  | VSS | - | 0.3VDD |  |
| tw(HSE) tw(HSE) | OSC_IN high or low time(1) |  | 5 | - | - | ns |
| tr(HSE) tf(HSE) | OSC_IN rise or fall time(1) |  | - | - | 20 |  |
| Cin(HSE) | OSC_IN input capacitance(1) | - | - | 5 | - | pF |
| DuCy(HSE) | Duty cycle | - | 45 | - | 55 | % |
| IL | OSC_IN Input leakage current | VSS VIN VDD | - | - | ±1 | µA |

      - Guaranteed by design.

#### Low-speed external user clock generated from an external source

The characteristics given in Table 22 result from tests performed using an low-speed external clock source, and under ambient temperature and supply voltage conditions summarized in Table 10.

##### Table 22. Low-speed external user clock characteristics

| Symbol | Parameter | Conditions | Min | Typ | Max | Unit |
| --- | --- | --- | --- | --- | --- | --- |
| fLSE_ext | User External clock source frequency(1) | - | - | 32.768 | 1000 | kHz |
| VLSEH | OSC32_IN input pin high level voltage |  | 0.7VDD | - | VDD | V |
| VLSEL | OSC32_IN input pin low level voltage |  | VSS | - | 0.3VDD |  |
| tw(LSE) tw(LSE) | OSC32_IN high or low time(1) |  | 450 | - | - | ns |
| tr(LSE) tf(LSE) | OSC32_IN rise or fall time(1) |  | - | - | 50 |  |
| Cin(LSE) | OSC32_IN input capacitance(1) | - | - | 5 | - | pF |
| DuCy(LSE) | Duty cycle | - | 30 | - | 70 | % |
| IL | OSC32_IN Input leakage current | VSS VIN VDD | - | - | ±1 | µA |

1. Guaranteed by design.

58/143	DS5792 Rev 13

67032)

,/

26&32B,1

I/6(BH[W

([WHUQDO FORFN VRXUFH

4(3%

T7((3%)    T

T7((3%)

TF((3%)

TR((3%)

6(3%(

90 %

10 %

6(3%,

Figure 20. High-speed external clock source AC timing diagram

AI17528

34-32&

),

/3#?).

F(3%?EXT

%XTERNAL CLOCK SOURCE

7/6(

W

W:(/6()

W:(/6()

WI(/6()

WU(/6()

9/6(+

90%

10%

9/6(/

Figure 21. Low-speed external clock source AC timing diagram

DL17529

DS5792 Rev 13	59/143

#### High-speed external clock generated from a crystal/ceramic resonator

The high-speed external (HSE) clock can be supplied with a 4 to 16 MHz crystal/ceramic resonator oscillator. All the information given in this paragraph are based on characterization results obtained with typical external components specified in Table 23. In the application, the resonator and the load capacitors have to be placed as close as possible to the oscillator pins in order to minimize output distortion and startup stabilization time. Refer to the crystal resonator manufacturer for more details on the resonator characteristics (frequency, package, accuracy).

##### Table 23. HSE 4-16 MHz oscillator characteristics(1)(2)

| Symbol | Parameter | Conditions | Min | Typ | Max | Unit |
| --- | --- | --- | --- | --- | --- | --- |
| fOSC_IN | Oscillator frequency | - | 4 | 8 | 16 | MHz |
| RF | Feedback resistor | - | - | 200 | - | kΩ |
| C | Recommended load capacitance versus equivalent serial resistance of the crystal (RS)(3) | RS = 30 Ω | - | 30 | - | pF |
| i2 | HSE driving current | VDD= 3.3 V, VIN = VSS with 30 pF load | - | - | 1 | mA |
| gm | Oscillator transconductance | Startup | 25 | - | - | mA/V |
| (4) tSU(HSE) | Startup time | VDD is stabilized | - | 2 | - | ms |

- Resonator characteristics given by the crystal/ceramic resonator manufacturer.

- Guaranteed by characterization results.

- The relatively low value of the RF resistor offers a good protection against issues resulting from use in a humid environment, due to the induced leakage and the bias condition change. However, it is recommended to take this point into account if the MCU is used in tough humidity conditions.

- tSU(HSE) is the startup time measured from the moment it is enabled (by software) to a stabilized 8 MHz oscillation is reached. This value is measured for a standard crystal resonator and it can vary significantly with the crystal manufacturer

For CL1 and CL2, it is recommended to use high-quality external ceramic capacitors in the 5 pF to 25 pF range (typ.), designed for high-frequency applications, and selected to match the requirements of the crystal or resonator (see Figure 22). CL1 and CL2 are usually the same size. The crystal manufacturer typically specifies a load capacitance which is the series combination of CL1 and CL2. PCB and MCU pin capacitance must be included (10 pF can be used as a rough estimate of the combined pin and board capacitance) when sizing CL1 and CL2. Refer to the application note AN2867 “Oscillator design guide for ST microcontrollers” available from the ST website www.st.com.

67032)

26&B287

5(;7(1)

&/2

%LDV FRQWUROOHG JDLQ

)

5

8 0+] UHVRQDWRU

I+6(

26&B,1

5HVRQDWRU ZLWK LQWHJUDWHG    FDSDFLWRUV

&/1

Figure 22. Typical application with an 8 MHz crystal

DL17530

1. REXT value depends on the crystal characteristics.

60/143	DS5792 Rev 13

#### Low-speed external clock generated from a crystal/ceramic resonator

The low-speed external (LSE) clock can be supplied with a 32.768 kHz crystal/ceramic resonator oscillator. All the information given in this paragraph are based on characterization results obtained with typical external components specified in Table 24. In the application, the resonator and the load capacitors have to be placed as close as possible to the oscillator pins in order to minimize output distortion and startup stabilization time. Refer to the crystal resonator manufacturer for more details on the resonator characteristics (frequency, package, accuracy).

##### Table 24. LSE oscillator characteristics (fLSE = 32.768 kHz)(1)(2)

| Symbol | Parameter | Conditions |  | Min | Typ | Max | Unit |
| --- | --- | --- | --- | --- | --- | --- | --- |
| RF | Feedback resistor | - |  | - | 5 | - | MΩ |
| C(2) | Recommended load capacitance versus equivalent serial resistance of the crystal (RS) | RS = 30 kΩ |  | - | - | 15 | pF |
| I2 | LSE driving current | VDD = 3.3 V, VIN = VSS |  | - | - | 1.4 | µA |
| gm | Oscillator transconductance | - |  | 5 | - | - | µA/V |
| tSU(LSE)(3) | Startup time | VDD is stabilized | TA = 50 °C | - | 1.5 | - | s |
|  |  |  | TA = 25 °C | - | 2.5 | - |  |
|  |  |  | TA = 10 °C | - | 4 | - |  |
|  |  |  | TA = 0 °C | - | 6 | - |  |
|  |  |  | TA = -10 °C | - | 10 | - |  |
|  |  |  | TA = -20 °C | - | 17 | - |  |
|  |  |  | TA = -30 °C | - | 32 | - |  |
|  |  |  | TA = -40 °C | - | 60 | - |  |

- Guaranteed by characterization results.

- Refer to the note and caution paragraphs below the table, and to the application note AN2867 “Oscillator design guide for ST microcontrollers”.

- tSU(LSE) is the startup time measured from the moment it is enabled (by software) until a stabilized 32.768 kHz oscillation is reached. This value is measured for a standard crystal and it can vary significantly with the crystal manufacturer, PCB layout and humidity.

Note:	For CL1 and CL2, it is recommended to use high-quality ceramic capacitors in the 5 pF to 15 pF range selected to match the requirements of the crystal or resonator (see Figure 23). CL1 and CL2, are usually the same size. The crystal manufacturer typically specifies a load capacitance which is the series combination of CL1 and CL2.

Load capacitance CL has the following formula: CL = CL1 x CL2 / (CL1 + CL2) + Cstray where

Cstray is the pin capacitance and board or trace PCB-related capacitance. Typically, it is between 2 pF and 7 pF.

Caution:	To avoid exceeding the maximum value of CL1 and CL2 (15 pF) it is strongly recommended to use a resonator with a load capacitance CL  7 pF. Never use a resonator with a load capacitance of 12.5 pF.

Example: if you choose a resonator with a load capacitance of CL = 6 pF, and Cstray = 2 pF, then CL1 = CL2 = 8 pF.

DS5792 Rev 13	61/143

67032)

26&32B287

&/2

%LDV

5) FRQWUROOHG

JDLQ

32.768 N+] UHVRQDWRU

I/6(

26&32B,1

5HVRQDWRU ZLWK LQWHJUDWHG FDSDFLWRUV

&/1

Figure 23. Typical application with a 32.768 kHz crystal

DL17531

### Internal clock source characteristics

The parameters given in Table 25 are derived from tests performed under ambient temperature and VDD supply voltage conditions summarized in Table 10.

#### High-speed internal (HSI) RC oscillator

##### Table 25. HSI oscillator characteristics(1)

| Symbol | Parameter | Conditions |  | Min | Typ | Max | Unit |
| --- | --- | --- | --- | --- | --- | --- | --- |
| fHSI | Frequency | - |  | - | 8 | - | MHz |
| DuCy(HSI) | Duty cycle | - |  | 45 | - | 55 | % |
| ACCHSI | Accuracy of the HSI oscillator | User-trimmed with the RCC_CR register(2) |  | - | - | 1(3) | % |
|  |  | Factory- calibrated(4) | TA = –40 to 105 °C | –2 | - | 2.5 | % |
|  |  |  | TA = –10 to 85 °C | –1.5 | - | 2.2 | % |
|  |  |  | TA = 0 to 70 °C | –1.3 | - | 2 | % |
|  |  |  | TA = 25 °C | –1.1 | - | 1.8 | % |
| tsu(HSI)(4) | HSI oscillator startup time | - |  | 1 | - | 2 | µs |
| IDD(HSI)(4) | HSI oscillator power consumption | - |  | - | 80 | 100 | µA |

      - VDD = 3.3 V, TA = –40 to 105 °C unless otherwise specified.

      - Refer to application note AN2868 “STM32F10xxx internal RC oscillator (HSI) calibration” available from the ST website www.st.com.

      - Guaranteed by design.

      - Guaranteed by characterization results.

62/143	DS5792 Rev 13

#### Low-speed internal (LSI) RC oscillator

##### Table 26. LSI oscillator characteristics (1)

| Symbol | Parameter | Min | Typ | Max | Unit |
| --- | --- | --- | --- | --- | --- |
| fLSI(2) | Frequency | 30 | 40 | 60 | kHz |
| tsu(LSI)(3) | LSI oscillator startup time | - | - | 85 | µs |
| (3) IDD(LSI) | LSI oscillator power consumption | - | 0.65 | 1.2 | µA |

- VDD  = 3 V, TA  = –40 to 105 °C unless otherwise specified.

- Guaranteed by characterization results.

- Guaranteed by design.

#### Wakeup time from low-power mode

The wakeup times given in Table 27 is measured on a wakeup phase with a 8-MHz HSI RC oscillator. The clock source used to wake up the device depends from the current operating mode:

- Stop or Standby mode: the clock source is the RC oscillator

- Sleep mode: the clock source is the clock that was set before entering Sleep mode.

All timings are derived from tests performed under ambient temperature and VDD supply voltage conditions summarized in Table 10.

##### Table 27. Low-power mode wakeup timings

| Symbol | Parameter | Typ | Unit |
| --- | --- | --- | --- |
| tWUSLEEP(1) | Wakeup from Sleep mode | 1.8 | µs |
| tWUSTO (1) P | Wakeup from Stop mode (regulator in run mode) | 3.6 | µs |
|  | Wakeup from Stop mode (regulator in low-power mode) | 5.4 |  |
| (1) tWUSTDBY | Wakeup from Standby mode | 50 | µs |

1. The wakeup times are measured from the wakeup event to the point in which the user application code reads the first instruction.

DS5792 Rev 13	63/143

### PLL characteristics

The parameters given in Table 28 are derived from tests performed under ambient temperature and VDD supply voltage conditions summarized in Table 10.

##### Table 28. PLL characteristics

| Symbol | Parameter | Value |  |  | Unit |
| --- | --- | --- | --- | --- | --- |
|  |  | Min | Typ | Max(1) |  |
| fPLL_IN | PLL input clock(2) | 1 | 8.0 | 25 | MHz |
|  | PLL input clock duty cycle | 40 | - | 60 | % |
| fPLL_OUT | PLL multiplier output clock | 16 | - | 72 | MHz |
| tLOCK | PLL lock time | - | - | 200 | µs |
| Jitter | Cycle-to-cycle jitter | - | - | 300 | ps |

      - Guaranteed by characterization results.

      - Take care of using the appropriate multiplier factors so as to have PLL input clock values compatible with the range defined by fPLL_OUT.

### Memory characteristics

#### Flash memory

The characteristics are given at TA = –40 to 105 °C unless otherwise specified.

##### Table 29. Flash memory characteristics

| Symbol | Parameter | Conditions | Min | Typ | Max(1) | Unit |
| --- | --- | --- | --- | --- | --- | --- |
| tprog | 16-bit programming time | TA  –40 to +105 °C | 40 | 52.5 | 70 | µs |
| tERASE | Page (2 KB) erase time | TA  –40 to +105 °C | 20 | - | 40 | ms |
| tME | Mass erase time | TA  –40 to +105 °C | 20 | - | 40 | ms |
| IDD | Supply current | Read mode fHCLK = 72 MHz with 2 wait states, VDD = 3.3 V | - | - | 28 | mA |
|  |  | Write mode fHCLK = 72 MHz, VDD = 3.3 V | - | - | 7 | mA |
|  |  | Erase mode fHCLK = 72 MHz, VDD = 3.3 V | - | - | 5 | mA |
|  |  | Power-down mode / Halt, VDD = 3.0 to 3.6 V | - | - | 50 | µA |
| Vprog | Programming voltage | - | 2 | - | 3.6 | V |

      - Guaranteed by design.

64/143	DS5792 Rev 13

##### Table 30. Flash memory endurance and data retention

| Symbol | Parameter | Conditions | Value | Unit |
| --- | --- | --- | --- | --- |
|  |  |  | Min(1) |  |
| NEND | Endurance | TA = -40 to +85 °C (6 suffix versions) TA = -40 to +105 °C (7 suffix versions) | 10 | kcycles |
| tRET | Data retention | 1 kcycle(2) at TA = 85 °C | 30 | Years |
|  |  | 1 kcycle(2) at TA = 105 °C | 10 |  |
|  |  | 10 kcycles(2) at TA = 55 °C | 20 |  |

- Guaranteed by characterization results.

- Cycling performed over the whole temperature range.

DS5792 Rev 13	65/143

### FSMC characteristics

#### Asynchronous waveforms and timings

Figure 24 through Figure 27 represent asynchronous waveforms and Table 31 through Table 34 provide the corresponding timings. The results shown in these tables are obtained with the following FSMC configuration:

- AddressSetupTime = 0

- AddressHoldTime = 1

- DataSetupTime = 1

W  Y(1$'9B1()

WZ(1$'9)

W K('DWDB1()

WK('DWDB12()

WVX('DWDB12()

WVX('DWDB1()

'DWD

W K(%/B12()

WY(%/B1()

$GGUHVV

W K($B12()

WY($B1()

W K(1(B12()

W Z(12()

W Y(12(B1()

WZ(1()

Figure 24. Asynchronous non-multiplexed SRAM/PSRAM/NOR read waveforms

061858692

)60&B1$'9(1)

)60&B'>15:0@

)60&B1%/>1:0@

)60&B$>25:0@

)60&B1:(

)60&B12(

)60&B1(

      - Mode 2/B, C and D only. In Mode 1, FSMC_NADV is not used.

66/143	DS5792 Rev 13

##### Table 31. Asynchronous non-multiplexed SRAM/PSRAM/NOR read timings(1)

| Symbol | Parameter | Min | Max | Unit |
| --- | --- | --- | --- | --- |
| tw(NE) | FSMC_NE low time | 5tHCLK – 1.5 | 5tHCLK + 2 | ns |
| tv(NOE_NE) | FSMC_NEx low to FSMC_NOE low | 0.5 | 1.5 | ns |
| tw(NOE) | FSMC_NOE low time | 5tHCLK – 1.5 | 5tHCLK + 1.5 | ns |
| th(NE_NOE) | FSMC_NOE high to FSMC_NE high hold time | –1.5 | - | ns |
| tv(A_NE) | FSMC_NEx low to FSMC_A valid | - | 0 | ns |
| th(A_NOE) | Address hold time after FSMC_NOE high | 0.1 | - | ns |
| tv(BL_NE) | FSMC_NEx low to FSMC_BL valid | - | 0 | ns |
| th(BL_NOE) | FSMC_BL hold time after FSMC_NOE high | 0 | - | ns |
| tsu(Data_NE) | Data to FSMC_NEx high setup time | 2tHCLK + 25 | - | ns |
| tsu(Data_NOE) | Data to FSMC_NOEx high setup time | 2tHCLK + 25 | - | ns |
| th(Data_NOE) | Data hold time after FSMC_NOE high | 0 | - | ns |
| th(Data_NE) | Data hold time after FSMC_NEx high | 0 | - | ns |
| tv(NADV_NE) | FSMC_NEx low to FSMC_NADV low | - | 5 | ns |
| tw(NADV) | FSMC_NADV low time | - | tHCLK + 1.5 | ns |

1. CL = 15 pF.

WY(1$'9B1()

WZ(1$'9)

)60&B1$'9(1)

$GGUHVV

WK(%/B1:()

1%/ WK('DWDB1:()

'DWD

)60&B'>15:0@

WY('DWDB1()

)60&B1%/>1:0@

WY(%/B1()

)60&B$>25:0@

WK($B1:()

WY($B1()

)60&B1:(

WK(1(B1:()

WZ(1:()

WY(1:(B1()

)60&B12(

)60&B1([

WZ(1()

Figure 25. Asynchronous non-multiplexed SRAM/PSRAM/NOR write waveforms

DL14990

1. Mode 2/B, C and D only. In Mode 1, FSMC_NADV is not used.

DS5792 Rev 13	67/143

##### Table 32. Asynchronous non-multiplexed SRAM/PSRAM/NOR write timings(1)(2)

| Symbol | Parameter | Min | Max | Unit |
| --- | --- | --- | --- | --- |
| tw(NE) | FSMC_NE low time | 3tHCLK – 1 | 3tHCLK + 2 | ns |
| tv(NWE_NE) | FSMC_NEx low to FSMC_NWE low | tHCLK – 0.5 | tHCLK + 1.5 | ns |
| tw(NWE) | FSMC_NWE low time | tHCLK – 0.5 | tHCLK + 1.5 | ns |
| th(NE_NWE) | FSMC_NWE high to FSMC_NE high hold time | tHCLK | - | ns |
| tv(A_NE) | FSMC_NEx low to FSMC_A valid | - | 7.5 | ns |
| th(A_NWE) | Address hold time after FSMC_NWE high | tHCLK | - | ns |
| tv(BL_NE) | FSMC_NEx low to FSMC_BL valid | - | 0 | ns |
| th(BL_NWE) | FSMC_BL hold time after FSMC_NWE high | tHCLK – 0.5 | - | ns |
| tv(Data_NE) | FSMC_NEx low to Data valid | - | tHCLK + 7 | ns |
| th(Data_NWE) | Data hold time after FSMC_NWE high | tHCLK | - | ns |
| tv(NADV_NE) | FSMC_NEx low to FSMC_NADV low | - | 5.5 | ns |
| tw(NADV) | FSMC_NADV low time | - | tHCLK + 1.5 | ns |

- CL = 15 pF.

- Guaranteed by characterization results.

!DDRESS T V(.!$6?.%)

TW(.!$6)

TH($ATA?./%)

TSU($ATA?.%)

TSU($ATA?./%)

$ATA

TH(!$?.!$6)

T V(!?.%)

TH($ATA?.%)

.",

!DDRESS

TV(",?.%)	TH(",?./%)

TH(!?./%)

TV(!?.%)

T W(./%)

T H(.%?./%)

TV(./%?.%)

TW(.%)

Figure 26. Asynchronous multiplexed PSRAM/NOR read waveforms

AI14892B

&3-#?.!$6

&3-#?!$;15:0=

&3-#?.",;1:0=

&3-#?!;25:16=

&3-#?.7%

&3-#?./%

&3-#?.%

68/143	DS5792 Rev 13

##### Table 33. Asynchronous multiplexed PSRAM/NOR read timings(1)(2)

| Symbol | Parameter | Min | Max | Unit |
| --- | --- | --- | --- | --- |
| tw(NE) | FSMC_NE low time | 7tHCLK – 2 | 7tHCLK + 2 | ns |
| tv(NOE_NE) | FSMC_NEx low to FSMC_NOE low | 3tHCLK – 0.5 | 3tHCLK + 1.5 | ns |
| tw(NOE) | FSMC_NOE low time | 4tHCLK – 1 | 4tHCLK + 2 | ns |
| th(NE_NOE) | FSMC_NOE high to FSMC_NE high hold time | –1 | - | ns |
| tv(A_NE) | FSMC_NEx low to FSMC_A valid | - | 0 | ns |
| tv(NADV_NE) | FSMC_NEx low to FSMC_NADV low | 3 | 5 | ns |
| tw(NADV) | FSMC_NADV low time | tHCLK –1.5 | tHCLK + 1.5 | ns |
| th(AD_NADV) | FSMC_AD (address) valid hold time after FSMC_NADV high | tHCLK | - | ns |
| th(A_NOE) | Address hold time after FSMC_NOE high | tHCLK -2 | - | ns |
| th(BL_NOE) | FSMC_BL hold time after FSMC_NOE high | 0 | - | ns |
| tv(BL_NE) | FSMC_NEx low to FSMC_BL valid | - | 0 | ns |
| tsu(Data_NE) | Data to FSMC_NEx high setup time | 2tHCLK + 24 | - | ns |
| tsu(Data_NOE) | Data to FSMC_NOE high setup time | 2tHCLK + 25 | - | ns |
| th(Data_NE) | Data hold time after FSMC_NEx high | 0 | - | ns |
| th(Data_NOE) | Data hold time after FSMC_NOE high | 0 | - | ns |

- CL = 15 pF.

- Guaranteed by characterization results.

DS5792 Rev 13	69/143

)60&B$'>15:0@

WK('DWDB1:()

$GGUHVV

WK(%/B1:()

1%/ WY('DWDB1$'9)

'DWD

WK($'B1$'9)

W Y($B1()

$GGUHVV W Y(1$'9B1()

WZ(1$'9)

)60&B1%/>1:0@

WY(%/B1()

)60&B$>25:16@

WK($B1:()

WY($B1()

)60&B1:(

W K(1(B1:()

WZ(1:()

WY(1:(B1()

)60&B12(

)60&B1([

WZ(1()

Figure 27. Asynchronous multiplexed PSRAM/NOR write waveforms

DL14891%

)60&B1$'9

Table 34. Asynchronous multiplexed PSRAM/NOR write timings(1)(2)

| Symbol | Parameter | Min | Max | Unit |
| --- | --- | --- | --- | --- |
| tw(NE) | FSMC_NE low time | 5tHCLK – 1 | 5tHCLK + 2 | ns |
| tv(NWE_NE) | FSMC_NEx low to FSMC_NWE low | 2tHCLK | 2tHCLK + 1 | ns |
| tw(NWE) | FSMC_NWE low time | 2tHCLK – 1 | 2tHCLK + 2 | ns |
| th(NE_NWE) | FSMC_NWE high to FSMC_NE high hold time | tHCLK – 1 | - | ns |
| tv(A_NE) | FSMC_NEx low to FSMC_A valid | - | 7 | ns |
| tv(NADV_NE) | FSMC_NEx low to FSMC_NADV low | 3 | 5 | ns |
| tw(NADV) | FSMC_NADV low time | tHCLK – 1 | tHCLK + 1 | ns |
| th(AD_NADV) | FSMC_AD (address) valid hold time after FSMC_NADV high | tHCLK – 3 | - | ns |
| th(A_NWE) | Address hold time after FSMC_NWE high | 4tHCLK | - | ns |
| tv(BL_NE) | FSMC_NEx low to FSMC_BL valid | - | 1.6 | ns |
| th(BL_NWE) | FSMC_BL hold time after FSMC_NWE high | tHCLK – 1.5 | - | ns |
| tv(Data_NADV) | FSMC_NADV high to Data valid | - | tHCLK + 1.5 | ns |
| th(Data_NWE) | Data hold time after FSMC_NWE high | tHCLK – 5 | - | ns |

- CL = 15 pF.

- BGuaranteed by characterization results.

70/143	DS5792 Rev 13

#### Synchronous waveforms and timings

Figure 28 through Figure 31 represent synchronous waveforms and Table 36 through Table 38 provide the corresponding timings. The results shown in these tables are obtained with the following FSMC configuration:

- BurstAccessMode = FSMC_BurstAccessMode_Enable;

- MemoryType = FSMC_MemoryType_CRAM;

- WriteBurst = FSMC_WriteBurst_Enable;

- CLKDivision = 1; (0 is not supported, see the STM32F10xxx reference manual)

- DataLatency = 1 for NOR Flash; DataLatency = 0 for PSRAM

TH(#,+(-.7!)46)

TSU(.7!)46-#,+()

&3-#?.7!)4

(7!)4#&' = 0B, 7!)40/, + 0B)

TH(#,+(-.7!)46)

TSU(.7!)46-#,+()

&3-#?.7!)4

(7!)4#&' = 1B, 7!)40/, + 0B)

TH(#,+(-.7!)46)

TH(#,+(-!$6)

$2	$3

TH(#,+(-!$6)

TSU(!$6-#,+()

$1

TD(#,+,-!$)6)

TSU(!$6-#,+()

!$;15:0=

TSU(.7!)46-#,+()

TD(#,+,-!$6)

&3-#?!$;15:0=

&3-#?./%

TD(#,+,-./%()

TD(#,+(-./%,)

&3-#?!;25:16=

TD(#,+,-!)6)

TD(#,+,-!6)

TD(#,+,-.!$6()

&3-#?.%X

TD(#,+,-.!$6,) &3-#?.!$6

TD(#,+,-.%X()

$ATA LATENCY = 0 TD(#,+,-.%X,)

&3-#?#,+

"53452. = 0

TW(#,+)

TW(#,+)

Figure 28. Synchronous multiplexed NOR/PSRAM read timings

AI14893I

DS5792 Rev 13	71/143

##### Table 35. Synchronous multiplexed NOR/PSRAM read timings(1)(2)

| Symbol | Parameter | Min | Max | Unit |
| --- | --- | --- | --- | --- |
| tw(CLK) | FSMC_CLK period | 27.7 | - | ns |
| td(CLKL-NExL) | FSMC_CLK low to FSMC_NEx low (x = 0...2) | - | 1.5 | ns |
| td(CLKL-NExH) | FSMC_CLK low to FSMC_NEx high (x = 0...2) | 2 | - | ns |
| td(CLKL-NADVL) | FSMC_CLK low to FSMC_NADV low | - | 4 | ns |
| td(CLKL-NADVH) | FSMC_CLK low to FSMC_NADV high | 5 | - | ns |
| td(CLKL-AV) | FSMC_CLK low to FSMC_Ax valid (x = 16...25) | - | 0 | ns |
| td(CLKL-AIV) | FSMC_CLK low to FSMC_Ax invalid (x = 16...25) | 2 | - | ns |
| td(CLKL-NOEL) | FSMC_CLK low to FSMC_NOE low | - | 1 | ns |
| td(CLKL-NOEH) | FSMC_CLK low to FSMC_NOE high | 1.5 | - | ns |
| td(CLKL-ADV) | FSMC_CLK low to FSMC_AD[15:0] valid | - | 12 | ns |
| td(CLKL-ADIV) | FSMC_CLK low to FSMC_AD[15:0] invalid | 0 | - | ns |
| tsu(ADV-CLKH) | FSMC_A/D[15:0] valid data before FSMC_CLK high | 6 | - | ns |
| th(CLKH-ADV) | FSMC_A/D[15:0] valid data after FSMC_CLK high | 0 | - | ns |
| tsu(NWAITV-CLKH) | FSMC_NWAIT valid before FSMC_CLK high | 8 | - | ns |
| th(CLKH-NWAITV) | FSMC_NWAIT valid after FSMC_CLK high | 2 | - | ns |

- CL = 15 pF.

- Guaranteed by characterization results.

72/143	DS5792 Rev 13

TH(#,+(-.7!)46)

TD(#,+,-.",()

TSU(.7!)46-#,+()

&3-#?.7!)4

(7!)4#&' = 0B, 7!)40/, + 0B)

$2

TD(#,+,-!$6)

&3-#?!$;15:0=	$1

TD(#,+,-$ATA)

TD(#,+,-!$)6)

TD(#,+,-$ATA)

!$;15:0=

&3-#?.7%

TD(#,+,-.7%()

TD(#,+,-.7%,)

&3-#?!;25:16=

TD(#,+,-!)6)

TD(#,+,-!6)

TD(#,+,-.!$6()

&3-#?.%X

TD(#,+,-.!$6,) &3-#?.!$6

TD(#,+,-.%X()

$ATA LATENCY = 0 TD(#,+,-.%X,)

&3-#?#,+

"53452. = 0

TW(#,+)

TW(#,+)

Figure 29. Synchronous multiplexed PSRAM write timings

AI14992G

&3-#?.",

DS5792 Rev 13	73/143

##### Table 36. Synchronous multiplexed PSRAM write timings(1)(2)

| Symbol | Parameter | Min | Max | Unit |
| --- | --- | --- | --- | --- |
| tw(CLK) | FSMC_CLK period | 27.7 | - | ns |
| td(CLKL-NExL) | FSMC_CLK low to FSMC_Nex low (x = 0...2) | - | 2 | ns |
| td(CLKL-NExH) | FSMC_CLK low to FSMC_NEx high (x = 0...2) | 2 | - | ns |
| td(CLKL-NADVL) | FSMC_CLK low to FSMC_NADV low | - | 4 | ns |
| td(CLKL-NADVH) | FSMC_CLK low to FSMC_NADV high | 5 | - | ns |
| td(CLKL-AV) | FSMC_CLK low to FSMC_Ax valid (x = 16...25) | - | 0 | ns |
| td(CLKL-AIV) | FSMC_CLK low to FSMC_Ax invalid (x = 16...25) | 2 | - | ns |
| td(CLKL-NWEL) | FSMC_CLK low to FSMC_NWE low | - | 1 | ns |
| td(CLKL-NWEH) | FSMC_CLK low to FSMC_NWE high | 1 | - | ns |
| td(CLKL-ADV) | FSMC_CLK low to FSMC_AD[15:0] valid | - | 12 | ns |
| td(CLKL-ADIV) | FSMC_CLK low to FSMC_AD[15:0] invalid | 3 | - | ns |
| td(CLKL-Data) | FSMC_A/D[15:0] valid after FSMC_CLK low | - | 6 | ns |
| td(CLKL-NBLH) | FSMC_CLK low to FSMC_NBL high | 1 | - | ns |
| tsu(NWAITV-CLKH) | FSMC_NWAIT valid before FSMC_CLK high | 7 | - | ns |
| th(CLKH-NWAITV) | FSMC_NWAIT valid after FSMC_CLK high | 2 | - | ns |

- CL = 15 pF.

- Guaranteed by characterization results.

74/143	DS5792 Rev 13

TH(#,+(-.7!)46)

TSU(.7!)46-#,+()

&3-#?.7!)4

(7!)4#&' = 0B, 7!)40/, + 0B)

T H(#,+(-.7!)46)

TSU(.7!)46-#,+()

&3-#?.7!)4

(7!)4#&' = 1B, 7!)40/, + 0B)

TSU(.7!)46-#,+()	TH(#,+(-.7!)46)

TH(#,+(-$6)

$2	$3

$1

&3-#?$;15:0=

TH(#,+(-$6)

TSU($6-#,+()

TSU($6-#,+()

&3-#?./%

TD(#,+,-./%()

TD(#,+(-./%,)

&3-#?!;25:0=

TD(#,+,-!)6)

TD(#,+,-!6)

TD(#,+,-.!$6()

TD(#,+,-.!$6,)

&3-#?.!$6

TD(#,+,-.%X()

$ATA LATENCY = 0

&3-#?#,+

TD(#,+,-.%X,) &3-#?.%X

"53452. = 0

TW(#,+)

TW(#,+)

Figure 30. Synchronous non-multiplexed NOR/PSRAM read timings

AI14894H

Table 37. Synchronous non-multiplexed NOR/PSRAM read timings(1)(2)

| Symbol | Parameter | Min | Max | Unit |
| --- | --- | --- | --- | --- |
| tw(CLK) | FSMC_CLK period | 27.7 | - | ns |
| td(CLKL-NExL) | FSMC_CLK low to FSMC_NEx low (x = 0...2) | - | 1.5 | ns |
| td(CLKL-NExH) | FSMC_CLK low to FSMC_NEx high (x = 0...2) | 2 | - | ns |
| td(CLKL-NADVL) | FSMC_CLK low to FSMC_NADV low | - | 4 | ns |
| td(CLKL-NADVH) | FSMC_CLK low to FSMC_NADV high | 5 | - | ns |
| td(CLKL-AV) | FSMC_CLK low to FSMC_Ax valid (x = 0...25) | - | 0 | ns |
| td(CLKL-AIV) | FSMC_CLK low to FSMC_Ax invalid (x = 0...25) | 4 | - | ns |
| td(CLKL-NOEL) | FSMC_CLK low to FSMC_NOE low | - | 1.5 | ns |
| td(CLKL-NOEH) | FSMC_CLK low to FSMC_NOE high | 1.5 | - | ns |
| tsu(DV-CLKH) | FSMC_D[15:0] valid data before FSMC_CLK high | 6.5 | - | ns |
| th(CLKH-DV) | FSMC_D[15:0] valid data after FSMC_CLK high | 7 | - | ns |
| tsu(NWAITV-CLKH) | FSMC_NWAIT valid before FSMC_SMCLK high | 7 | - | ns |
| th(CLKH-NWAITV) | FSMC_NWAIT valid after FSMC_CLK high | 2 | - | ns |

- CL = 15 pF.

- Guaranteed by characterization results.

DS5792 Rev 13	75/143

WK(&/.+-1:$,79)

WVX(1:$,79-&/.+)

)60&B1:$,7

(:$,7&)*   0E, :$,732/ + 0E)

WK(&/.+-1:$,79)

WVX(1:$,79-&/.+)

WK(&/.+-1:$,79)

)60&B1:$,7

(:$,7&)*   1E, :$,732/ + 0E)

'2

WK(&/.+-$'9)

WK(&/.+-$'9)

WVX($'9-&/.+)

'1

WG(&/./-$',9)

WVX($'9-&/.+)

$'>15:0@

WVX(1:$,79-&/.+)

)60&B12(

WG(&/./-$'9)

)60&B$'>15:0@

WG(&/.+-12(/)	WG(&/./-12(+)

)60&B$>25:16@

WG(&/./-$,9)

WG(&/./-$9)

WG(&/./-1$'9+)

)60&B1([

WG(&/./-1$'9/)

)60&B1$'9

WG(&/./-1([+)

'DWD ODWHQF\ 0

WG(&/./-1([/)

)60&B&/.

%867851 0

WZ(&/.)

WZ(&/.)

Figure 31. Synchronous non-multiplexed PSRAM write timings

DL14893K

Table 38. Synchronous non-multiplexed PSRAM write timings(1)(2)

| Symbol | Parameter | Min | Max | Unit |
| --- | --- | --- | --- | --- |
| tw(CLK) | FSMC_CLK period | 27.7 | - | ns |
| td(CLKL-NExL) | FSMC_CLK low to FSMC_NEx low (x = 0...2) | - | 2 | ns |
| td(CLKL-NExH) | FSMC_CLK low to FSMC_NEx high (x = 0...2) | 2 | - | ns |
| td(CLKL-NADVL) | FSMC_CLK low to FSMC_NADV low | - | 4 | ns |
| td(CLKL-NADVH) | FSMC_CLK low to FSMC_NADV high | 5 | - | ns |
| td(CLKL-AV) | FSMC_CLK low to FSMC_Ax valid (x = 16...25) | - | 0 | ns |
| td(CLKL-AIV) | FSMC_CLK low to FSMC_Ax invalid (x = 16...25) | 2 | - | ns |
| td(CLKL-NWEL) | FSMC_CLK low to FSMC_NWE low | - | 1 | ns |
| td(CLKL-NWEH) | FSMC_CLK low to FSMC_NWE high | 1 | - | ns |
| td(CLKL-Data) | FSMC_D[15:0] valid data after FSMC_CLK low | - | 6 | ns |
| td(CLKL-NBLH) | FSMC_CLK low to FSMC_NBL high | 1 | - | ns |
| tsu(NWAITV-CLKH) | FSMC_NWAIT valid before FSMC_CLK high | 7 | - | ns |
| th(CLKH-NWAITV) | FSMC_NWAIT valid after FSMC_CLK high | 2 | - | ns |

- CL = 15 pF.

- Guaranteed by characterization results.

76/143	DS5792 Rev 13

#### PC Card/CompactFlash controller waveforms and timings

Figure 32 through Figure 37 represent synchronous waveforms and Table 39 provides the corresponding timings. The results shown in this table are obtained with the following FSMC configuration:

- COM.FSMC_SetupTime = 0x04;

- COM.FSMC_WaitSetupTime = 0x07;

- COM.FSMC_HoldSetupTime = 0x04;

- COM.FSMC_HiZSetupTime = 0x00;

- ATT.FSMC_SetupTime = 0x04;

- ATT.FSMC_WaitSetupTime = 0x07;

- ATT.FSMC_HoldSetupTime = 0x04;

- ATT.FSMC_HiZSetupTime = 0x00;

- IO.FSMC_SetupTime = 0x04;

- IO.FSMC_WaitSetupTime = 0x07;

- IO.FSMC_HoldSetupTime = 0x04;

- IO.FSMC_HiZSetupTime = 0x00;

- TCLRSetupTime = 0;

- TARSetupTime = 0;

WK(12(-')

WVX('-12()

WZ(12()

)60&B1,25'

)60&B1:(

WG(1&(4B1-12()

)60&B12(

)60&B15(*

)60&B1,2:5

WK(1&([-15(*)

WK(1&([-1,25')

WK(1&([-1,2:5)

WG(15(*-1&([)

WG(1,25'-1&([)

)60&B$>10:0@

WK(1&([-$,)

WY(1&([-$)

)60&B1&(4B2(1)

)60&B1&(4B1

Figure 32. PC Card/CompactFlash controller waveforms for common memory read access

DL14895E

)60&B'>15:0@

1. FSMC_NCE4_2 remains high (inactive during 8-bit access.

DS5792 Rev 13	77/143

)60&B'>15:0@

WK(1:(-')

WG('-1:()

WY(1:(-')

0(0[+,= 1

)60&B12(

WG(1:(-1&(4B1)

WZ(1:()

)60&B1,25'

WG(1&(4B1-1:()

)60&B1:(

)60&B15(*

)60&B1,2:5

WK(1&(4B1-1,2:5)

WK(1&(4B1-1,25')

WG(1,25'-1&(4B1)

WK(1&(4B1-15(*)

WG(15(*-1&(4B1)

)60&B$>10:0@

WK(1&(4B1-$,)

WY(1&(4B1-$)

)60&B1&(4B2 +LJK

)60&B1&(4B1

Figure 33. PC Card/CompactFlash controller waveforms for common memory write access

DL14896E

78/143	DS5792 Rev 13

)60&B'>15:0@(1)

WK(12(-')

WVX('-12()

WG(12(-1&(4B1)

WZ(12()

)60&B1:(

WG(1&(4B1-12()

)60&B12(

)60&B15(*

WK(1&(4B1-15(*)

WG(15(*-1&(4B1)

)60&B1,2:5

)60&B1,25'

)60&B$>10:0@

+LJK

)60&B1&(4B2

WK(1&(4B1-$,)

WY(1&(4B1-$)

)60&B1&(4B1

Figure 34. PC Card/CompactFlash controller waveforms for attribute memory read access

DL14897E

1. Only data bits 0...7 are read (bits 8...15 are disregarded).

DS5792 Rev 13	79/143

WY(1:(-')

WZ(1:()

WG(1:(-1&(4B1)

)60&B15(*

WG(1&(4B1-1:()

)60&B1:(

WK(1&(4B1-15(*)

WG(15(*-1&(4B1)

)60&B1,2:5

)60&B1,25'

)60&B$>10:0@

WK(1&(4B1-$,)

WY(1&(4B1-$)

)60&B1&(4B2 +LJK

)60&B1&(4B1

Figure 35. PC Card/CompactFlash controller waveforms for attribute memory write access

DL14898E

)60&B'>7:0@(1)

)60&B12(

1. Only data bits 0...7 are driven (bits 8...15 remains HiZ).

)60&B'>15:0@

WVX('-1,25')	WG(1,25'-')

WZ(1,25')

)60&B$>10:0@

)60&B15(*

)60&B1:(

)60&B12(

)60&B1,2:5

WG(1,25'-1&(4B1)

)60&B1,25'

WK(1&(4B1-$,)

WY(1&([-$)

)60&B1&(4B1

)60&B1&(4B2

Figure 36. PC Card/CompactFlash controller waveforms for I/O space read access

DL14899%

80/143	DS5792 Rev 13

DL14900F

)60&B'>15:0@

WK(1,2:5-')

WY(1,2:5-')

)60&B$>10:0@

)60&B15(*

)60&B1:(

)60&B12(

)60&B1,25'

WG(1&(4B1-1,2:5)	WZ(1,2:5)

)60&B1,2:5

$77[+,=   1

WK(1&(4B1-$,)

WY(1&([-$)

)60&B1&(4B1

)60&B1&(4B2

Figure 37. PC Card/CompactFlash controller waveforms for I/O space write access

Table 39. Switching characteristics for PC Card/CF read and write cycles(1)(2)

| Symbol | Parameter | Min | Max | Unit |
| --- | --- | --- | --- | --- |
| tv(NCEx-A) tv(NCE4_1-A) | FSMC_NCEx low (x = 4_1/4_2) to FSMC_Ay valid (y = 0...10) FSMC_NCE4_1 low (x = 4_1/4_2) to FSMC_Ay valid (y = 0...10) | - | 0 | ns |
| th(NCEx-AI) th(NCE4_1-AI) | FSMC_NCEx high (x = 4_1/4_2) to FSMC_Ax invalid (x = 0...10) FSMC_NCE4_1 high (x = 4_1/4_2) to FSMC_Ax invalid (x = 0...10) | 2.5 | - | ns |
| td(NREG-NCEx) td(NREG-NCE4_1) | FSMC_NCEx low to FSMC_NREG valid FSMC_NCE4_1 low to FSMC_NREG valid | - | 5 | ns |
| th(NCEx-NREG) th(NCE4_1-NREG) | FSMC_NCEx high to FSMC_NREG invalid FSMC_NCE4_1 high to FSMC_NREG invalid | tHCLK + 3 | - | ns |
| td(NCE4_1-NOE) | FSMC_NCE4_1 low to FSMC_NOE low | - | 5tHCLK + 2 | ns |
| tw(NOE) | FSMC_NOE low width | 8tHCLK –1.5 | 8tHCLK + 1 | ns |
| td(NOE-NCE4_1 | FSMC_NOE high to FSMC_NCE4_1 high | 5tHCLK + 2 | - | ns |
| tsu(D-NOE) | FSMC_D[15:0] valid data before FSMC_NOE high | 25 | - | ns |
| th(NOE-D) | FSMC_D[15:0] valid data after FSMC_NOE high | 15 | - | ns |
| tw(NWE) | FSMC_NWE low width | 8tHCLK – 1 | 8tHCLK + 2 | ns |
| td(NWE-NCE4_1) | FSMC_NWE high to FSMC_NCE4_1 high | 5tHCLK + 2 | - | ns |
| td(NCE4_1-NWE) | FSMC_NCE4_1 low to FSMC_NWE low | - | 5tHCLK + 1.5 | ns |
| tv(NWE-D) | FSMC_NWE low to FSMC_D[15:0] valid | - | 0 | ns |
| th(NWE-D) | FSMC_NWE high to FSMC_D[15:0] invalid | 11tHCLK | - | ns |
| td(D-NWE) | FSMC_D[15:0] valid before FSMC_NWE high | 13tHCLK | - | ns |

DS5792 Rev 13	81/143

##### Table 39. Switching characteristics for PC Card/CF read and write cycles(1)(2) (continued)

| Symbol | Parameter | Min | Max | Unit |
| --- | --- | --- | --- | --- |
| tw(NIOWR) | FSMC_NIOWR low width | 8tHCLK + 3 | - | ns |
| tv(NIOWR-D) | FSMC_NIOWR low to FSMC_D[15:0] valid | - | 5tHCLK +1 | ns |
| th(NIOWR-D) | FSMC_NIOWR high to FSMC_D[15:0] invalid | 11tHCLK | - | ns |
| td(NCE4_1-NIOWR) | FSMC_NCE4_1 low to FSMC_NIOWR valid | - | 5tHCLK+3ns | ns |
| th(NCEx-NIOWR) th(NCE4_1-NIOWR) | FSMC_NCEx high to FSMC_NIOWR invalid FSMC_NCE4_1 high to FSMC_NIOWR invalid | 5tHCLK – 5 | - | ns |
| td(NIORD-NCEx) td(NIORD-NCE4_1) | FSMC_NCEx low to FSMC_NIORD valid FSMC_NCE4_1 low to FSMC_NIORD valid | - | 5tHCLK + 2.5 | ns |
| th(NCEx-NIORD) th(NCE4_1-NIORD) | FSMC_NCEx high to FSMC_NIORD invalid FSMC_NCE4_1 high to FSMC_NIORD invalid | 5tHCLK – 5 | - | ns |
| tsu(D-NIORD) | FSMC_D[15:0] valid before FSMC_NIORD high | 4.5 | - | ns |
| td(NIORD-D) | FSMC_D[15:0] valid after FSMC_NIORD high | 9 | - | ns |
| tw(NIORD) | FSMC_NIORD low width | 8tHCLK + 2 | - | ns |

- CL = 15 pF.

- Guaranteed by characterization results.

82/143	DS5792 Rev 13

#### NAND controller waveforms and timings

Figure 38 through Figure 41 represent synchronous waveforms and Table 39 provides the corresponding timings. The results shown in this table are obtained with the following FSMC configuration:

  - COM.FSMC_SetupTime = 0x01;

  - COM.FSMC_WaitSetupTime = 0x03;

  - COM.FSMC_HoldSetupTime = 0x02;

  - COM.FSMC_HiZSetupTime = 0x01;

  - ATT.FSMC_SetupTime = 0x01;

  - ATT.FSMC_WaitSetupTime = 0x03;

  - ATT.FSMC_HoldSetupTime = 0x02;

  - ATT.FSMC_HiZSetupTime = 0x01;

  - Bank = FSMC_Bank_NAND;

  - MemoryDataWidth = FSMC_MemoryDataWidth_16b;

  - ECC = FSMC_ECC_Enable;

  - ECCPageSize = FSMC_ECCPageSize_512Bytes;

  - TCLRSetupTime = 0;

  - TARSetupTime = 0;

)60&B1&([  /RZ

$/( ()60&B$17)

&/( ()60&B$16)

)60&B1:(

WG($/(-12()	WK(12(-$/()

)60&B12( (15()

WVX('-12()	WK(12(-')

)60&B'>15:0@

DL14901E

Figure 38. NAND controller waveforms for read access

DS5792 Rev 13	83/143

WK(1:(-')

WY(1:(-')

)60&B12( (15()

)60&B1:(

WK(1:(-$/()

WG($/(-1:()

)60&B1&([

$/( ()60&B$17)

&/( ()60&B$16)

Figure 39. NAND controller waveforms for write access

AI14902C

)60&B'>15:0@

)60&B'>15:0@

WK(12(-')

WVX('-12()

)60&B1&([  /RZ

$/( ()60&B$17)

&/( ()60&B$16)

WG($/(-12()	WK(12(-$/()

)60&B1:(

WZ(12()

)60&B12(

Figure 40. NAND controller waveforms for common memory read access

DL14912E

84/143	DS5792 Rev 13

)60&B'>15:0@

WK(1:(-')

WG('-1:()

WY(1:(-')

)60&B12(

WK(1:(-$/()

WZ(1:()

$/( ()60&B$17)

&/( ()60&B$16)

WG($/(-1:()

)60&B1:(

Figure 41. NAND controller waveforms for common memory write access

DL14913E

)60&B1&([  /RZ

Table 40. Switching characteristics for NAND Flash read and write cycles(1)

| Symbol | Parameter | Min | Max | Unit |
| --- | --- | --- | --- | --- |
| td(D-NWE)(2) | FSMC_D[15:0] valid before FSMC_NWE high | 5tHCLK + 12 | - | ns |
| tw(NOE)(2) | FSMC_NWE low width | 4tHCLK-1.5 | 4tHCLK+1.5 | ns |
| (2) tsu(D-NOE) | FSMC_D[15:0] valid data before FSMC_NOE high | 25 | - | ns |
| th(NOE-D)(2) | FSMC_D[15:0] valid data after FSMC_NOE high | 7 | - | - |
| tw(NWE)(2) | FSMC_NWE low width | 4tHCLK-1 | 4tHCLK+1 | ns |
| tv(NWE-D)(2) | FSMC_NWE low to FSMC_D[15:0] valid | - | 0 | ns |
| th(NWE-D)(2) | FSMC_NWE high to FSMC_D[15:0] invalid | 2tHCLK + 4 | - | ns |
| td(ALE-NWE)(3) | FSMC_ALE valid before FSMC_NWE low | - | 3tHCLK + 1.5 | ns |
| th(NWE-ALE)(3) | FSMC_NWE high to FSMC_ALE invalid | 3tHCLK + 4.5 | - | ns |
| td(ALE-NOE)(3) | FSMC_ALE valid before FSMC_NOE low | - | 3tHCLK+ 2 | ns |
| th(NOE-ALE)(3) | FSMC_NWE high to FSMC_ALE invalid | 3tHCLK+ 4.5 | - | ns |

- CL = 15 pF.

- Guaranteed by characterization results.

- Guaranteed by design.

DS5792 Rev 13	85/143

### EMC characteristics

Susceptibility tests are performed on a sample basis during device characterization.

#### Functional EMS (electromagnetic susceptibility)

While a simple application is executed on the device (toggling 2 LEDs through I/O ports). the device is stressed by two electromagnetic events until a failure occurs. The failure is indicated by the LEDs:

- Electrostatic discharge (ESD) (positive and negative) is applied to all device pins until a functional disturbance occurs. This test is compliant with the IEC 61000-4-2 standard.

- FTB: A Burst of Fast Transient voltage (positive and negative) is applied to VDD and VSS through a 100 pF capacitor, until a functional disturbance occurs. This test is compliant with the IEC 61000-4-4 standard.

A device reset allows normal operations to be resumed.

The test results are given in Table 41. They are based on the EMS levels and classes defined in application note AN1709.

##### Table 41. EMS characteristics

| Symbol | Parameter | Conditions | Level/ Class |
| --- | --- | --- | --- |
| VFESD | Voltage limits to be applied on any I/O pin to induce a functional disturbance | VDD  3.3 V, LQFP144, TA  +25 °C, fHCLK  72 MHz conforms to IEC 61000-4-2 | 2B |
| VEFTB | Fast transient voltage burst limits to be applied through 100 pF on VDD and VSS pins to induce a functional disturbance | VDD  3.3 V, LQFP144, TA  +25 °C, fHCLK  72 MHz conforms to IEC 61000-4-4 | 4A |

Designing hardened software to avoid noise problems

EMC characterization and optimization are performed at component level with a typical application environment and simplified MCU software. It should be noted that good EMC performance is highly dependent on the user application and the software in particular.

Therefore it is recommended that the user applies EMC software optimization and prequalification tests in relation with the EMC level requested for his application.

Software recommendations

The software flowchart must include the management of runaway conditions such as:

- Corrupted program counter

- Unexpected reset

- Critical Data corruption (control registers...) Prequalification trials

Most of the common failures (unexpected reset and program counter corruption) can be reproduced by manually forcing a low state on the NRST pin or the Oscillator pins for 1 second.

86/143	DS5792 Rev 13

To complete these trials, ESD stress can be applied directly on the device, over the range of specification values. When unexpected behavior is detected, the software can be hardened to prevent unrecoverable errors occurring (see application note AN1015).

#### Electromagnetic Interference (EMI)

The electromagnetic field emitted by the device are monitored while a simple application is executed (toggling 2 LEDs through the I/O ports). This emission test is compliant with

IEC 61967-2 standard which specifies the test board and the pin loading.

##### Table 42. EMI characteristics

| Symbol | Parameter | Conditions | Monitored frequency band | Max vs. [fHSE/fHCLK] |  | Unit |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | 8/48 MHz | 8/72 MHz |  |
| SEMI | Peak level | VDD  3.3 V, TA  25 °C, LQFP144 package compliant with IEC 61967-2 | 0.1 to 30 MHz | 8 | 12 | dBµV |
|  |  |  | 30 to 130 MHz | 31 | 21 |  |
|  |  |  | 130 MHz to 1GHz | 28 | 33 |  |
|  |  |  | SAE EMI Level | 4 | 4 | - |

### Absolute maximum ratings (electrical sensitivity)

Based on three different tests (ESD, LU) using specific measurement methods, the device is stressed in order to determine its performance in terms of electrical sensitivity.

#### Electrostatic discharge (ESD)

Electrostatic discharges (a positive then a negative pulse separated by 1 second) are applied to the pins of each sample according to each pin combination. The sample size depends on the number of supply pins in the device (3 parts × (n+1) supply pins). This test conforms to the JESD22-A114/C101 standard.

##### Table 43. ESD absolute maximum ratings

| Symbol | Ratings | Conditions | Class | Maximum value(1) | Unit |
| --- | --- | --- | --- | --- | --- |
| VESD(HBM) | Electrostatic discharge voltage (human body model) | TA  +25 °C, conforming to JESD22-A114 | 2 | 2000 | V |
| VESD(CDM) | Electrostatic discharge voltage (charge device model) | TA  +25 °C, conforming to JESD22-C101 | III | 500 |  |

- Guaranteed by characterization results.

#### Static latch-up

Two complementary static tests are required on six parts to assess the latch-up performance:

  - A supply overvoltage is applied to each power supply pin

  - A current injection is applied to each input, output and configurable I/O pin These tests are compliant with EIA/JESD 78A IC latch-up standard.

DS5792 Rev 13	87/143

##### Table 44. Electrical sensitivities

| Symbol | Parameter | Conditions | Class |
| --- | --- | --- | --- |
| LU | Static latch-up class | TA  +105 °C conforming to JESD78A | II level A |

### I/O current injection characteristics

As a general rule, current injection to the I/O pins, due to external voltage below VSS or above VDD (for standard, 3 V-capable I/O pins) should be avoided during normal product operation. However, in order to give an indication of the robustness of the microcontroller in cases when abnormal injection accidentally happens, susceptibility tests are performed on a sample basis during device characterization.

#### Functional susceptibilty to I/O current injection

While a simple application is executed on the device, the device is stressed by injecting current into the I/O pins programmed in floating input mode. While current is injected into the I/O pin, one at a time, the device is checked for functional failures.

The failure is indicated by an out of range parameter: ADC error above a certain limit (>5 LSB TUE), out of spec current injection on adjacent pins or other functional failure (for example reset, oscillator frequency deviation).

The test results are given in Table 45

##### Table 45. I/O current injection susceptibility

| Symbol | Description | Functional susceptibility |  | Unit |
| --- | --- | --- | --- | --- |
|  |  | Negative injection | Positive injection |  |
| IINJ | Injected current on OSC_IN32, OSC_OUT32, PA4, PA5, PC13 | -0 | +0 | mA |
|  | Injected current on all FT pins | -5 | +0 |  |
|  | Injected current on any other pin | -5 | +5 |  |

88/143	DS5792 Rev 13

### I/O port characteristics

#### General input/output characteristics

Unless otherwise specified, the parameters given in Table 46 are derived from tests performed under the conditions summarized in Table 10. All I/Os are CMOS and TTL compliant.

##### Table 46. I/O static characteristics

| Symbol | Parameter | Conditions | Min | Typ | Max | Unit |
| --- | --- | --- | --- | --- | --- | --- |
| VIL | Standard IO input low level voltage | - | –0.3 | - | 0.28*(VDD-2 V)+0.8 V | V |
|  | IO FT(1) input low level voltage |  | –0.3 | - | 0.32*(VDD-2 V)+0.75 V | V |
| VIH | Standard IO input high level voltage | - | 0.41*(VDD-2 V)+1.3 V | - | VDD+0.3 | V |
|  | IO FT(1) input high level voltage | VDD > 2 V | 0.42*(VDD-2 V)+1 V | - | 5.5 | V |
|  |  | VDD 2 V |  |  | 5.2 |  |
| Vhys | Standard IO Schmitt trigger voltage hysteresis(2) | - | 200 | - | - | mV |
|  | IO FT Schmitt trigger voltage hysteresis(2) |  | 5% VDD(3) | - | - | mV |
| Ilkg | Input leakage current (4) | VSS VIN VDD Standard I/Os | - | - | 1 | µA |
|  |  | VIN= 5 V, I/O FT | - | - | 3 |  |
| RPU | Weak pull-up equivalent resistor(5) | VIN  VSS | 30 | 40 | 50 | kΩ |
| RPD | Weak pull-down equivalent resistor(5) | VIN  VDD | 30 | 40 | 50 | kΩ |
| CIO | I/O pin capacitance | - | - | 5 | - | pF |

- FT = Five-volt tolerant. In order to sustain a voltage higher than VDD+0.3 the internal pull-up/pull-down resistors must be disabled.

- Hysteresis voltage between Schmitt trigger switching levels. Guaranteed by characterization results.

- With a minimum of 100 mV.

- Leakage could be higher than max. if negative current is injected on adjacent pins.

- Pull-up and pull-down resistors are designed with a true resistance in series with a switchable PMOS/NMOS. This MOS/NMOS contribution to the series resistance is minimum (~10% order).

All I/Os are CMOS and TTL compliant (no software configuration required). Their characteristics cover more than the strict CMOS-technology or TTL parameters. The coverage of these requirements is shown in Figure 42 and Figure 43 for standard I/Os, and in Figure 44 and Figure 45 for 5 V tolerant I/Os.

DS5792 Rev 13	89/143

6$$ (6)

AI17277B

3.6

3.3

3

2.7

2

)NPUT RANGE NOT GUARANTEED

1.25

1.08

1

1

1.08

11.5.59 9

7)(MIN  1.3

7),MAX  0.8

0.7

1.71

1.71

1.96

6)(/6), (6)

Figure 42. Standard I/O input characteristics - CMOS port

6$$ (6)

AI17278

3.6

2	2.16

44, REQUIREMENTS 6),=0.86

1.25

1.3

)NPUT RANGE NOT GUARANTEED

1.96

)(MIN 2.0

44, REQUIREMENTS 6)(=26

7

7),MAX 0.8

6)(/6), (6)

Figure 43. Standard I/O input characteristics - TTL port

3.6

3.3

3

2.7

2

6$$ (6)

0.75

0.7

)NPUT RANGE NOT GUARANTEED

1

1.42

1.07

1.295

0.975

1

1.55

1.16

1.3

1.67

AI17279B

6$$

6)(/6), (6)

Figure 44. 5 V tolerant I/O input characteristics - CMOS port

90/143	DS5792 Rev 13

6$$ (6)

AI17280

3.6

2	2.16

44, REQUIREMENTS 6),=0.86

)(MIN

7),MAX    0.8

0.75

)NPUT RANGE NOT GUARANTEED

7

1.67

1

2.0

44, REQUIREMENT 6)(=26

6)(/6), (6)

Figure 45. 5 V tolerant I/O input characteristics - TTL port

Output driving current

The GPIOs (general purpose input/outputs) can sink or source up to ±8 mA, and sink or source up to ± 20 mA (with a relaxed VOL/VOH) except PC13, PC14 and PC15 which can sink or source up to ±3 mA. When using the GPIOs PC13 to PC15 in output mode, the speed should not exceed 2 MHz with a maximum load of 30 pF.

In the user application, the number of I/O pins which can drive current must be limited to respect the absolute maximum rating specified in Section 5.2:

  - The sum of the currents sourced by all the I/Os on VDD, plus the maximum Run consumption of the MCU sourced on VDD, cannot exceed the absolute maximum rating IVDD (see Table 8).

  - The sum of the currents sunk by all the I/Os on VSS plus the maximum Run consumption of the MCU sunk on VSS cannot exceed the absolute maximum rating IVSS (see Table 8).

#### Output voltage levels

Unless otherwise specified, the parameters given in Table 47 are derived from tests performed under ambient temperature and VDD supply voltage conditions summarized in Table 10. All I/Os are CMOS and TTL compliant.

##### Table 47. Output voltage characteristics

| Symbol | Parameter | Conditions | Min | Max | Unit |
| --- | --- | --- | --- | --- | --- |
| V (1) OL | Output low level voltage for an I/O pin when 8 pins are sunk at same time | TTL port(3) IIO = +8 mA 2.7 V < VDD < 3.6 V | - | 0.4 | V |
| V (2) OH | Output high level voltage for an I/O pin when 8 pins are sourced at same time |  | VDD–0.4 | - |  |
| VOL (1) | Output low level voltage for an I/O pin when 8 pins are sunk at same time | CMOS port(3) IIO =+ 8mA 2.7 V < VDD < 3.6 V | - | 0.4 | V |
| VOH (2) | Output high level voltage for an I/O pin when 8 pins are sourced at same time |  | 2.4 | - |  |

DS5792 Rev 13	91/143

##### Table 47. Output voltage characteristics (continued)

| Symbol | Parameter | Conditions | Min | Max | Unit |
| --- | --- | --- | --- | --- | --- |
| V (1)(4) OL | Output low level voltage for an I/O pin when 8 pins are sunk at same time | IIO = +20 mA 2.7 V < VDD < 3.6 V | - | 1.3 | V |
| V (2)(4) OH | Output high level voltage for an I/O pin when 8 pins are sourced at same time |  | VDD–1.3 | - |  |
| V (1)(4) OL | Output low level voltage for an I/O pin when 8 pins are sunk at same time | IIO = +6 mA 2 V < VDD < 2.7 V | - | 0.4 | V |
| V (2)(4) OH | Output high level voltage for an I/O pin when 8 pins are sourced at same time |  | VDD–0.4 | - |  |

- The IIO current sunk by the device must always respect the absolute maximum rating specified in Table 8

and the sum of IIO (I/O ports and control pins) must not exceed IVSS.

- The IIO current sourced by the device must always respect the absolute maximum rating specified in

Table 8 and the sum of IIO (I/O ports and control pins) must not exceed IVDD.

- TTL and CMOS outputs are compatible with JEDEC standards JESD36 and JESD52.

- Guaranteed by characterization results.

92/143	DS5792 Rev 13

#### Input/output AC characteristics

The definition and values of input/output AC characteristics are given in Figure 46 and

Table 48, respectively.

Unless otherwise specified, the parameters given in Table 48 are derived from tests performed under ambient temperature and VDD supply voltage conditions summarized in Table 10.

##### Table 48. I/O AC characteristics(1)

| MODEx[1:0] bit value(1) | Symbol | Parameter | Conditions | Min | Max | Unit |
| --- | --- | --- | --- | --- | --- | --- |
| 10 | fmax(IO)out | Maximum frequency(2) | CL = 50 pF, VDD = 2 V to 3.6 V | - | 2 | MHz |
|  | tf(IO)out | Output high to low level fall time | CL = 50 pF, VDD = 2 V to 3.6 V | - | 125(3) | ns |
|  | tr(IO)out | Output low to high level rise time |  | - | 125(3) |  |
| 01 | fmax(IO)out | Maximum frequency(2) | CL = 50 pF, VDD = 2 V to 3.6 V | - | 10 | MHz |
|  | tf(IO)out | Output high to low level fall time | CL = 50 pF, VDD = 2 V to 3.6 V | - | 25(3) | ns |
|  | tr(IO)out | Output low to high level rise time |  | - | 25(3) |  |
| 11 | Fmax(IO)out | Maximum frequency(2) | CL = 30 pF, VDD = 2.7 V to 3.6 V | - | 50 | MHz |
|  |  |  | CL = 50 pF, VDD = 2.7 V to 3.6 V | - | 30 | MHz |
|  |  |  | CL = 50 pF, VDD = 2 V to 2.7 V | - | 20 | MHz |
|  | tf(IO)out | Output high to low level fall time | CL = 30 pF, VDD = 2.7 V to 3.6 V | - | 5(3) | ns |
|  |  |  | CL = 50 pF, VDD = 2.7 V to 3.6 V | - | 8(3) |  |
|  |  |  | CL = 50 pF, VDD = 2 V to 2.7 V | - | 12(3) |  |
|  | tr(IO)out | Output low to high level rise time | CL = 30 pF, VDD = 2.7 V to 3.6 V | - | 5(3) |  |
|  |  |  | CL = 50 pF, VDD = 2.7 V to 3.6 V | - | 8(3) |  |
|  |  |  | CL = 50 pF, VDD = 2 V to 2.7 V | - | 12(3) |  |
| - | tEXTIpw | Pulse width of external signals detected by the EXTI controller | - | 10 | - | ns |

- The I/O speed is configured using the MODEx[1:0] bits. Refer to the STM32F10xxx reference manual for a description of GPIO Port configuration register.

- The maximum frequency is defined in Figure 46.

- Guaranteed by design.

DS5792 Rev 13	93/143

7

WI(,2)RXW

WU(,2)RXW

90%

10%

50%	50%

10%

90%

Figure 46. I/O AC characteristics definition

DL14131G

ZKHQ ORDGHG E\ &/ VSHFLILHG LQ WKH WDEOH ³ ,/2 $& FKDUDFWHULVWLFV´.

0D[LPXP IUHTXHQF\ LV DFKLHYHG LI (WU + WI)  (2/3)7 DQG LI WKH GXW\ F\FOH LV (45-55%)

(;7(51$/ 287387

21 &/

### NRST pin characteristics

The NRST pin input driver uses CMOS technology. It is connected to a permanent pull-up resistor, RPU (see Table 46).

Unless otherwise specified, the parameters given in Table 49 are derived from tests performed under ambient temperature and VDD supply voltage conditions summarized in Table 10.

##### Table 49. NRST pin characteristics

| Symbol | Parameter | Conditions | Min | Typ | Max | Unit |
| --- | --- | --- | --- | --- | --- | --- |
| VIL(NRST)(1) | NRST Input low level voltage | - | –0.5 | - | 0.8 | V |
| VIH(NRST (1) ) | NRST Input high level voltage | - | 2 | - | VDD+0.5 |  |
| Vhys(NRST) | NRST Schmitt trigger voltage hysteresis | - | - | 200 | - | mV |
| RPU | Weak pull-up equivalent resistor(2) | VIN  VSS | 30 | 40 | 50 | kΩ |
| VF(NRST)(1) | NRST Input filtered pulse | - | - | - | 100 | ns |
| (1) VNF(NRST) | NRST Input not filtered pulse | - | 300 | - | - | ns |

      - Guaranteed by design.

      - The pull-up is designed with a true resistance in series with a switchable PMOS. This PMOS contribution

to the series resistance must be minimum (~10% order).

94/143	DS5792 Rev 13

67032)

0.1 )

)LOWHU

,QWHUQDO 5HVHW

538

1567(2)

UHVHW FLUFXLW (1)

9''

([WHUQDO

Figure 47. Recommended NRST pin protection

DL14132F

- The reset network protects the device against parasitic resets.

- The user must ensure that the level on the NRST pin can go below the VIL(NRST) max level specified in

Table 49. Otherwise the reset will not be taken into account by the device.

### TIM timer characteristics

The parameters given in Table 50 are guaranteed by design.

Refer to Section 5.3.14: I/O port characteristics for details on the input/output alternate function characteristics (output compare, input capture, external clock, PWM output).

##### Table 50. TIMx(1) characteristics

| Symbol | Parameter | Conditions | Min | Max | Unit |
| --- | --- | --- | --- | --- | --- |
| tres(TIM) | Timer resolution time | - | 1 | - | tTIMxCLK |
|  |  | fTIMxCLK = 72 MHz | 13.9 | - | ns |
| fEXT | Timer external clock frequency on CH1 to CH4 | - | 0 | fTIMxCLK/2 | MHz |
|  |  | fTIMxCLK = 72 MHz | 0 | 36 | MHz |
| ResTIM | Timer resolution | - | - | 16 | bit |
| tCOUNTER | 16-bit counter clock period when internal clock is selected | - | 1 | 65536 | tTIMxCLK |
|  |  | fTIMxCLK = 72 MHz | 0.0139 | 910 | µs |
| tMAX_COUNT | Maximum possible count | - | - | 65536 × 65536 | tTIMxCLK |
|  |  | fTIMxCLK = 72 MHz | - | 59.6 | s |

      - TIMx is used as a general term to refer to the TIM1, TIM2, TIM3 and TIM4 timers.

DS5792 Rev 13	95/143

### Communications interfaces

#### I2C interface characteristics

The STM32F103xC, STM32F103xD and STM32F103xESTM32F103xF and STM32F103xG

performance line I2C interface meets the requirements of the standard I2C communication protocol with the following restrictions: the I/O pins SDA and SCL are mapped to are not “true” open-drain. When configured as open-drain, the PMOS connected between the I/O pin and VDD is disabled, but is still present.

The I2C characteristics are described in Table 51. Refer also to Section 5.3.14: I/O port characteristics for more details on the input/output alternate function characteristics (SDA and SCL).

##### Table 51. I2C characteristics

| Symbol | Parameter | Standard mode I2C(1)(2) |  | Fast mode I2C(1)(2) |  | Unit |
| --- | --- | --- | --- | --- | --- | --- |
|  |  | Min | Max | Min | Max |  |
| tw(SCLL) | SCL clock low time | 4.7 | - | 1.3 | - | µs |
| tw(SCLH) | SCL clock high time | 4.0 | - | 0.6 | - |  |
| tsu(SDA) | SDA setup time | 250 | - | 100 | - | ns |
| th(SDA) | SDA data hold time | - | 3450(3) | - | 900(3) |  |
| tr(SDA) tr(SCL) | SDA and SCL rise time | - | 1000 | - | 300 |  |
| tf(SDA) tf(SCL) | SDA and SCL fall time | - | 300 | - | 300 |  |
| th(STA) | Start condition hold time | 4.0 | - | 0.6 | - | µs |
| tsu(STA) | Repeated Start condition setup time | 4.7 | - | 0.6 | - |  |
| tsu(STO) | Stop condition setup time | 4.0 | - | 0.6 | - | s |
| tw(STO:STA) | Stop to Start condition time (bus free) | 4.7 | - | 1.3 | - | s |
| Cb | Capacitive load for each bus line | - | 400 | - | 400 | pF |
| tSP | Pulse width of the spikes that are suppressed by the analog filter for standard and fast mode | 0 | 50(4) | 0 | 50(4) | s |

      - Guaranteed by design.

      - fPCLK1 must be at least 2 MHz to achieve standard mode I2C frequencies. It must be at least 4 MHz to achieve the fast mode I2C frequencies and it must be a multiple of 10 MHz in order to reach the I2C fast mode maximum clock speed of 400 kHz.

      - The device must internally provide a hold time of at least 300ns for the SDA signal in order to bridge the undefined region on the falling edge of SCL.

      - The minimum width of the spikes filtered by the analog filter is above tSP(max).

96/143	DS5792 Rev 13

6'$

6&/

56

6

67032

5

53

53

WVX(672)

WI(6&/)

WU(6&/)

6&/

WZ(6&/+)

Z(672:67$)

6 723

WK(6'$)

WK(67$)	WZ(6&//)

W

WVX(6'$)

WU(6'$)

6'$ WI(6'$)

67$57

WVX(67$)

67$57

Figure 48. I2C bus AC waveforms and measurement circuit

DL14979G

67$57 5(3($7('

,ð& EXV

9''B,2&    9''B,2&

- Measurement points are done at CMOS levels: 0.3VDD and 0.7VDD.

- Rs: Series protection resistors.

- Rp: Pull-up resistors.

- VDD_I2C : I2C bus supply

Table 52. SCL frequency (fPCLK1= 36 MHz.,VDD_I2C = 3.3 V)(1)(2)

| fSCL (kHz) | I2C_CCR value |
| --- | --- |
|  | RP = 4.7 kΩ |
| 400 | 0x801E |
| 300 | 0x8028 |
| 200 | 0x803C |
| 100 | 0x00B4 |
| 50 | 0x0168 |
| 20 | 0x0384 |

- RP = External pull-up resistance, fSCL = I2C speed.

- For speeds around 200 kHz, the tolerance on the achieved speed is of 5%. For other speed ranges, the tolerance on the achieved speed 2%. These variations depend on the accuracy of the external components used to design the application.

DS5792 Rev 13	97/143

#### I2S - SPI characteristics

Unless otherwise specified, the parameters given in Table 53 for SPI or in Table 54 for I2S are derived from tests performed under ambient temperature, fPCLKx frequency and VDD supply voltage conditions summarized in Table 10.

Refer to Section 5.3.14: I/O port characteristics for more details on the input/output alternate function characteristics (NSS, SCK, MOSI, MISO for SPI and WS, CK, SD for I2S).

##### Table 53. SPI characteristics

| Symbol | Parameter | Conditions | Min | Max | Unit |
| --- | --- | --- | --- | --- | --- |
| fSCK 1/tc(SCK) | SPI clock frequency | Master mode | - | 18 | MHz |
|  |  | Slave mode | - | 18 |  |
| tr(SCK) tf(SCK) | SPI clock rise and fall time | Capacitive load: C = 30 pF | - | 8 | ns |
| DuCy(SCK) | SPI slave input clock duty cycle | Slave mode | 30 | 70 | % |
| tsu(NSS)(1) | NSS setup time | Slave mode | 4tPCLK | - | ns |
| th(NSS)(1) | NSS hold time | Slave mode | 2tPCLK | - |  |
| tw(SC (1) KH) tw(SC (1) KL) | SCK high and low time | Master mode, fPCLK = 36 MHz, presc = 4 | 50 | 60 |  |
| tsu(MI) (1) t (1) su(SI) | Data input setup time | Master mode | 5 | - |  |
|  |  | Slave mode | 5 | - |  |
| th(MI) (1) | Data input hold time | Master mode | 5 | - |  |
| t (1) h(SI) |  | Slave mode | 4 | - |  |
| (1)(2) ta(SO) | Data output access time | Slave mode, fPCLK = 20 MHz | 0 | 3tPCLK |  |
| tdis(SO)(1)(3) | Data output disable time | Slave mode | 2 | 10 |  |
| tv(SO) (1) | Data output valid time | Slave mode (after enable edge) | - | 25 |  |
| tv(MO)(1) | Data output valid time | Master mode (after enable edge) | - | 5 |  |
| t (1) h(SO) | Data output hold time | Slave mode (after enable edge) | 15 | - |  |
| th(MO (1) ) |  | Master mode (after enable edge) | 2 | - |  |

- Guaranteed by characterization results.

- Min time is for the minimum time to drive the output and the max time is for the maximum time to validate the data.

- Min time is for the minimum time to invalidate the output and the max time is for the maximum time to put the data in Hi-Z

98/143	DS5792 Rev 13

##### Figure 49. SPI timing diagram - slave mode and CPHA = 0

Figure 50. SPI timing diagram - slave mode and CPHA = 1(1)

/6% ,1

%,7 1 ,1

06% ,1

WK(6,)

/6% 287

%,76 287

06% 287

WVX(6,)

WGLV(62)

WU(6&.)

WI(6&.)

WK(62)

WY(62)

D(62)

W

WZ(6&.+)

WZ(6&./)

WK(166)

WF(6&.)

W68(166)

DL14135E

026,

,1387

0,62

287387

&3+$ 1 &32/ 0

&3+$ 1 &32/ 1

166 LQSXW

6&. LQSXW

1. Measurement points are done at CMOS levels: 0.3VDD and 0.7VDD.

DS5792 Rev 13	99/143

/6% 287

% , 71 287

WK(02)

WU(6&.)

WI(6&.)

/6% ,1

%,76 ,1

WZ(6&.+)

WZ(6&./)

06% ,1

WK(0,)

06% 287

WY(02)

&3+$ 1 &32/ 0

&3+$ 1 &32/ 1

WVX(0,)

0,62

,13 87

&3+$ 0 &32/ 0

&3+$ 0 &32/ 1

WF(6&.)

DL14136F

026,

287387

166 LQSXW

+LJK

6&. 2XWSXW

6&. 2XWSXW

Figure 51. SPI timing diagram - master mode(1)

1. Measurement points are done at CMOS levels: 0.3VDD and 0.7VDD.

100/143	DS5792 Rev 13

##### Table 54. I2S characteristics

| Symbol | Parameter | Conditions |  | Min | Max | Unit |
| --- | --- | --- | --- | --- | --- | --- |
| DuCy(SCK) | I2S slave input clock duty cycle | Slave mode |  | 30 | 70 | % |
| fCK 1/tc(CK) | I2S clock frequency | Master mode (data: 16 bits, Audio frequency = 48 kHz) |  | 1.522 | 1.525 | MHz |
|  |  | Slave mode |  | 0 | 6.5 |  |
| tr(CK) tf(CK) | I2S clock rise and fall time | Capacitive load CL = 50 pF |  | - | 8 | ns |
| tv(WS) (1) | WS valid time | Master mode |  | 3 | - |  |
| th(WS) (1) | WS hold time | Master mode | I2S2 | 2 | - |  |
|  |  |  | I2S3 | 0 | - |  |
| tsu(WS) (1) | WS setup time | Slave mode |  | 4 | - |  |
| th(WS) (1) | WS hold time | Slave mode |  | 0 | - |  |
| tw(CKH) (1) | CK high and low time | Master fPCLK= 16 MHz, audio frequency = 48 kHz |  | 312.5 | - |  |
| tw(CKL) (1) |  |  |  | 345 | - |  |
| tsu(SD_MR) (1) | Data input setup time | Master receiver | I2S2 | 2 | - |  |
|  |  |  | I2S3 | 6.5 | - |  |
| tsu(SD_SR) (1) | Data input setup time | Slave receiver |  | 1.5 | - |  |
| th(SD_M (1)(2) R) | Data input hold time | Master receiver |  | 0 | - |  |
| th(SD_SR) (1)(2) |  | Slave receiver |  | 0.5 | - |  |
| tv(SD_ST) (1)(2) | Data output valid time | Slave transmitter (after enable edge) |  | - | 18 |  |
| th(SD_ST) (1) | Data output hold time | Slave transmitter (after enable edge) |  | 11 | - |  |
| tv(SD_MT) (1)(2) | Data output valid time | Master transmitter (after enable edge) |  | - | 3 |  |
| th(SD_MT) (1) | Data output hold time | Master transmitter (after enable edge) |  | 0 | - |  |

- Guaranteed by design and/or characterization results.

- Depends on fPCLK. For example, if fPCLK=8 MHz, then TPCLK = 1/fPLCLK =125 ns.

DS5792 Rev 13	101/143

##### Figure 52. I2S slave timing diagram (Philips protocol)(1)

  - Measurement points are done at CMOS levels: 0.3 × VDD and 0.7 × VDD.

  - LSB transmit/receive of the previously transmitted byte. No LSB transmit/receive is sent before the first byte.

##### Figure 53. I2S master timing diagram (Philips protocol)(1)

- Guaranteed by characterization results.

- LSB transmit/receive of the previously transmitted byte. No LSB transmit/receive is sent before the first byte.

102/143	DS5792 Rev 13

#### SD/SDIO MMC card host interface (SDIO) characteristics

Unless otherwise specified, the parameters given in Table 55 are derived from tests performed under ambient temperature, fPCLKx frequency and VDD supply voltage conditions summarized in Table 10.

Refer to Section 5.3.14: I/O port characteristics for more details on the input/output alternate function characteristics (D[7:0], CMD, CK).

##### Figure 54. SDIO high-speed mode

T/($

T/6$

Figure 55. SD default mode

AI14888

$, #-$ (OUTPUT)

#+

Table 55. SD / MMC characteristics

| Symbol | Parameter | Conditions | Min | Max | Unit |
| --- | --- | --- | --- | --- | --- |
| fPP | Clock frequency in data transfer mode | CL  30 pF | 0 | 48 | MHz |
| tW(CKL) | Clock low time, fPP = 16 MHz | CL  30 pF | 32 | - | ns |
| tW(CKH) | Clock high time, fPP = 16 MHz | CL  30 pF | 30 | - |  |
| tr | Clock rise time | CL  30 pF | - | 4 |  |
| tf | Clock fall time | CL  30 pF | - | 5 |  |

DS5792 Rev 13	103/143

##### Table 55. SD / MMC characteristics

| Symbol | Parameter | Conditions | Min | Max | Unit |
| --- | --- | --- | --- | --- | --- |
| CMD, D inputs (referenced to CK) |  |  |  |  |  |
| tISU | Input setup time | CL  30 pF | 2 | - | ns |
| tIH | Input hold time | CL  30 pF | 0 | - |  |
| CMD, D outputs (referenced to CK) in MMC and SD HS mode |  |  |  |  |  |
| tOV | Output valid time | CL  30 pF | - | 6 | ns |
| tOH | Output hold time | CL  30 pF | 0 | - |  |
| CMD, D outputs (referenced to CK) in SD default mode(1) |  |  |  |  |  |
| tOVD | Output valid default time | CL  30 pF | - | 7 | ns |
| tOHD | Output hold default time | CL  30 pF | 0.5 | - |  |

1. Refer to SDIO_CLKCR, the SDI clock control register to control the CK output.

#### USB characteristics

The USB interface is USB-IF certified (Full Speed).

##### Table 56. USB startup time

| Symbol | Parameter | Max | Unit |
| --- | --- | --- | --- |
| (1) tSTARTUP | USB transceiver startup time | 1 | µs |

1. Guaranteed by design.

104/143	DS5792 Rev 13

##### Table 57. USB DC electrical characteristics

| Symbol | Parameter | Conditions | Min.(1) | Max.(1) | Unit |
| --- | --- | --- | --- | --- | --- |
| Input levels |  |  |  |  |  |
| VDD | USB operating voltage(2) | - | 3.0(3) | 3.6 | V |
| VDI(4) | Differential input sensitivity | I(USB_DP, USB_DM) | 0.2 | - | V |
| (4) VCM | Differential common mode range | Includes VDI range | 0.8 | 2.5 |  |
| (4) VSE | Single ended receiver threshold | - | 1.3 | 2.0 |  |
| Output levels |  |  |  |  |  |
| VOL | Static output level low | RL of 1.5 kΩ to 3.6 V(5) | - | 0.3 | V |
| VOH | Static output level high | RL of 15 kΩ to V (5) SS | 2.8 | 3.6 |  |

- All the voltages are measured from the local ground potential.

- To be compliant with the USB 2.0 full-speed electrical specification, the USB_DP (D+) pin should be pulled up with a 1.5 kΩ resistor to a 3.0-to-3.6 V voltage range.

- The STM32F103xC/D/E USB functionality is ensured down to 2.7 V but not the full USB electrical characteristics which are degraded in the 2.7-to-3.0 V VDD voltage range.

- Guaranteed by characterization results.

- RL is the load connected on the USB drivers

##### Figure 56. USB timings: definition of data signal rise and fall time

DL14137E

966

&URVV RYHU SRLQWV

'LIIHUHQWLDO GDWD OLQHV

9&56

WU

WI

Table 58. USB: full-speed electrical characteristics

| Driver characteristics(1) |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Symbol | Parameter | Conditions | Min | Max | Unit |
| tr | Rise time(2) | CL = 50 pF | 4 | 20 | ns |
| tf | Fall Time(2) | CL = 50 pF | 4 | 20 | ns |
| trfm | Rise/ fall time matching | tr/tf | 90 | 110 | % |
| VCRS | Output signal crossover voltage | - | 1.3 | 2.0 | V |

- Guaranteed by design.

- Measured from 10% to 90% of the data signal. For more detailed informations, please refer to USB Specification - Chapter 7 (version 2.0).

DS5792 Rev 13	105/143

### CAN (controller area network) interface

Refer to Section 5.3.14: I/O port characteristics for more details on the input/output alternate function characteristics (CAN_TX and CAN_RX).

### 12-bit ADC characteristics

Unless otherwise specified, the parameters given in Table 59 are preliminary values derived from tests performed under ambient temperature, fPCLK2 frequency and VDDA supply voltage conditions summarized in Table 10.

Note:	It is recommended to perform a calibration after each power-up.

##### Table 59. ADC characteristics

| Symbol | Parameter | Conditions | Min | Typ | Max | Unit |
| --- | --- | --- | --- | --- | --- | --- |
| VDDA | Power supply | - | 2.4 | - | 3.6 | V |
| VREF+ | Positive reference voltage | - | 2.4 | - | VDDA | V |
| VREF- | Negative reference voltage | - | 0 |  |  | V |
| IVREF | Current on the VREF input pin | - | - | 160(1) | 220 | µA |
| fADC | ADC clock frequency | - | 0.6 | - | 14 | MHz |
| (2) fS | Sampling rate | - | 0.05 | - | 1 | MHz |
| fTR (2) IG | External trigger frequency | fADC = 14 MHz | - | - | 823 | kHz |
|  |  | - | - | - | 17 | 1/fADC |
| VAIN | Conversion voltage range(3) | - | 0 (VSSA or VREF- tied to ground) | - | VREF+ | V |
| RAIN(2) | External input impedance | See Equation 1 and Table 60 for details | - | - | 50 | Ω |
| (2) RADC | Sampling switch resistance | - | - | - | 1 | Ω |
| CAD (2) C | Internal sample and hold capacitor | - | - | - | 8 | pF |
| tCAL(2) | Calibration time | fADC = 14 MHz | 5.9 |  |  | µs |
|  |  | - | 83 |  |  | 1/fADC |
| tlat(2) | Injection trigger conversion latency | fADC = 14 MHz | - | - | 0.214 | µs |
|  |  | - | - | - | 3(4) | 1/fADC |
| tla (2) tr | Regular trigger conversion latency | fADC = 14 MHz | - | - | 0.143 | µs |
|  |  | - | - | - | 2(4) | 1/fADC |
| t (2) S | Sampling time | fADC = 14 MHz | 0.107 | - | 17.1 | µs |
|  |  | - | 1.5 | - | 239.5 | 1/fADC |
| (2) tSTAB | Power-up time | - | 0 | 0 | 1 | µs |
| tCONV(2) | Total conversion time (including sampling time) | fADC = 14 MHz | 1 | - | 18 | µs |
|  |  | - | 14 to 252 (tS for sampling +12.5 for successive approximation) |  |  | 1/fADC |

106/143	DS5792 Rev 13

- Guaranteed by characterization results.

- Guaranteed by design.

- VREF+ can be internally connected to VDDA and VREF- can be internally connected to VSSA, depending on the package. Refer to Section 3: Pinouts and pin descriptions for further details.

- For external triggers, a delay of 1/fPCLK2 must be added to the latency specified in Table 59.

##### Equation 1: RAIN max formula

RAIN 

TS

----------------------------------------------------N----+----2--- –

RADC

fADC   CADC  ln2	

The formula above (Equation 1) is used to determine the maximum external impedance allowed for an error below 1/4 of LSB. Here N = 12 (from 12-bit resolution).

Table 60. RAIN max for fADC = 14 MHz(1)

| Ts (cycles) | tS (µs) | RAIN max (kΩ) |
| --- | --- | --- |
| 1.5 | 0.11 | 0.4 |
| 7.5 | 0.54 | 5.9 |
| 13.5 | 0.96 | 11.4 |
| 28.5 | 2.04 | 25.2 |
| 41.5 | 2.96 | 37.2 |
| 55.5 | 3.96 | 50 |
| 71.5 | 5.11 | NA |
| 239.5 | 17.1 | NA |

  - Guaranteed by design.

##### Table 61. ADC accuracy - limited test conditions(1)(2)

| Symbol | Parameter | Test conditions | Typ | Max(3) | Unit |
| --- | --- | --- | --- | --- | --- |
| ET | Total unadjusted error | fPCLK2 = 56 MHz, fADC = 14 MHz, RAIN < 10 kΩ, VDDA = 3 V to 3.6 V TA = 25 °C Measurements made after ADC calibration VREF+ = VDDA | ±1.3 | ±2 |  |
| EO | Offset error |  | ±1 | ±1.5 |  |
| EG | Gain error |  | ±0.5 | ±1.5 | LSB |
| ED | Differential linearity error |  | ±0.7 | ±1 |  |
| EL | Integral linearity error |  | ±0.8 | ±1.5 |  |

- ADC DC accuracy values are measured after internal calibration.

- ADC Accuracy vs. Negative Injection Current: Injecting negative current on any analog input pins should be avoided as this significantly reduces the accuracy of the conversion being performed on another analog input. It is recommended to add a Schottky diode (pin to ground) to analog pins which may potentially inject negative current.

Any positive injection current within the limits specified for IINJ(PIN) and IINJ(PIN) in Section 5.3.14 does not affect the ADC accuracy.

- Guaranteed by characterization results.

DS5792 Rev 13	107/143

Table 62. ADC accuracy(1) (2)(3)

| Symbol | Parameter | Test conditions | Typ | Max(4) | Unit |
| --- | --- | --- | --- | --- | --- |
| ET | Total unadjusted error | fPCLK2 = 56 MHz, fADC = 14 MHz, RAIN < 10 kΩ, VDDA = 2.4 V to 3.6 V Measurements made after ADC calibration | ±2 | ±5 |  |
| EO | Offset error |  | ±1.5 | ±2.5 |  |
| EG | Gain error |  | ±1.5 | ±3 | LSB |
| ED | Differential linearity error |  | ±1 | ±2 |  |
| EL | Integral linearity error |  | ±1.5 | ±3 |  |

- ADC DC accuracy values are measured after internal calibration.

- Better performance could be achieved in restricted VDD, frequency, VREF and temperature ranges.

- ADC Accuracy vs. Negative Injection Current: Injecting negative current on any analog input pins should be avoided as this significantly reduces the accuracy of the conversion being performed on another analog input. It is recommended to add a Schottky diode (pin to ground) to analog pins which may potentially inject negative current.

Any positive injection current within the limits specified for IINJ(PIN) and IINJ(PIN) in Section 5.3.14 does not affect the ADC accuracy.

- Guaranteed by characterization results.

AI14395C

6 $$!

62%&+

4093   4094   4095 4096

6$$!

7

3	456

1	2

0

633!

1,  3")$%!,

%$

%,

%/

5

4

3

2

1

(1)

(3)

7

6

(2)

%4

4095

4094

4093

%'

;1,3" )$%!,= 4096 (OR 4096 DEPENDING ON PACKAGE)=

Figure 57. ADC accuracy characteristics

- Example of an actual transfer curve.

- Ideal transfer curve.

- End point correlation line.

- ET = Total Unadjusted Error: maximum deviation between the actual and the ideal transfer curves. EO = Offset Error: deviation between the first actual transition and the first ideal one.

EG = Gain Error: deviation between the last ideal transition and the last actual one.

ED = Differential Linearity Error: maximum deviation between actual steps and the ideal one.

EL = Integral Linearity Error: maximum deviation between any actual transition and the end point correlation line.

108/143	DS5792 Rev 13

12-ELW FRQYHUWHU

&$'&(1)

,/1 $

97

0.6 9

&SDUDVLWLF

9$,1

5$'&(1)

$,1[

0.6 9

6DPSOH DQG KROG $'& FRQYHUWHU

5$,1(1)

97

67032)

9''

Figure 58. Typical connection diagram using the ADC

DL17534

- Refer to Table 59 for the values of RAIN, RADC and CADC.

- Cparasitic represents the capacitance of the PCB (dependent on soldering and PCB layout quality) plus the pad capacitance (roughly 7 pF). A high Cparasitic value will downgrade conversion accuracy. To remedy this, fADC should be reduced.

General PCB design guidelines

Power supply decoupling should be performed as shown in Figure 59 or Figure 60, depending on whether VREF+ is connected to VDDA or not. The 10 nF capacitors should be ceramic (good quality). They should be placed them as close as possible to the chip.

966$ /95()±

(VHH QRWH 1)

1 ) // 10 Q)

9''$

1 ) // 10 Q)

95()+

(VHH QRWH 1)

67032)103[[

Figure 59. Power supply and reference decoupling (VREF+ not connected to VDDA)

DL14388E

1. VREF+ and VREF– inputs are available only on 100-pin packages.

DS5792 Rev 13	109/143

Figure 60. Power supply and reference decoupling (VREF+ connected to VDDA)

67032)103[[

95()+/9''$ (6HH QRWH 1)

1 ) // 10 Q)

95()±/966$  (6HH QRWH 1)

DL14389

1. VREF+ and VREF– inputs are available only on 100-pin packages.

110/143	DS5792 Rev 13

### DAC electrical specifications

##### Table 63. DAC characteristics

| Symbol | Parameter | Min | Typ | Max | Unit | Comments |
| --- | --- | --- | --- | --- | --- | --- |
| VDDA | Analog supply voltage | 2.4 | - | 3.6 | V | - |
| VREF+ | Reference supply voltage | 2.4 | - | 3.6 | V | VREF+ must always be below VDDA |
| VSSA | Ground | 0 | - | 0 | V | - |
| RLOAD(1) | Resistive load with buffer ON | 5 | - | - | kΩ | - |
| RO(2) | Impedance output with buffer OFF | - | - | 15 | kΩ | When the buffer is OFF, the Minimum resistive load between DAC_OUT and VSS to have a 1% accuracy is 1.5 MΩ |
| CLOAD(1) | Capacitive load | - | - | 50 | pF | Maximum capacitive load at DAC_OUT pin (when the buffer is ON). |
| DAC_OUT min(1) | Lower DAC_OUT voltage with buffer ON | 0.2 | - | - | V | It gives the maximum output excursion of the DAC. It corresponds to 12-bit input code (0x0E0) to (0xF1C) at VREF+ = 3.6 V and (0x155) and (0xEAB) at VREF+ = 2.4 V |
| DAC_OUT max(1) | Higher DAC_OUT voltage with buffer ON | - | - | VDDA – 0.2 | V |  |
| DAC_OUT min(1) | Lower DAC_OUT voltage with buffer OFF | - | 0.5 | - | mV | It gives the maximum output excursion of the DAC. |
| DAC_OUT max(1) | Higher DAC_OUT voltage with buffer OFF | - | - | VREF+ – 1LSB | V |  |
| IDDVREF+ | DAC DC current consumption in quiescent mode (Standby mode) | - | - | 220 | µA | With no load, worst code (0xF1C) at VREF+ = 3.6 V in terms of DC consumption on the inputs |
| IDDA | DAC DC current consumption in quiescent mode(3) | - | - | 380 | µA | With no load, middle code (0x800) on the inputs |
|  |  | - | - | 480 | µA | With no load, worst code (0xF1C) at VREF+ = 3.6 V in terms of DC consumption on the inputs |
| DNL(4) | Differential non linearity Difference between two consecutive code-1LSB) | - | - | ±0.5 | LSB | Given for the DAC in 10-bit configuration |
|  |  | - | - | ±2 | LSB | Given for the DAC in 12-bit configuration |
| INL(3) | Integral non linearity (difference between measured value at Code i and the value at Code i on a line drawn between Code 0 and last Code 1023) | - | - | ±1 | LSB | Given for the DAC in 10-bit configuration |
|  |  | - | - | ±4 | LSB | Given for the DAC in 12-bit configuration |

DS5792 Rev 13	111/143

##### Table 63. DAC characteristics (continued)

| Symbol | Parameter | Min | Typ | Max | Unit | Comments |
| --- | --- | --- | --- | --- | --- | --- |
| Offset(3) | Offset error (difference between measured value at Code (0x800) and the ideal value = VREF+/2) | - | - | ±10 | mV | - |
|  |  | - | - | ±3 | LSB | Given for the DAC in 10-bit at VREF+ = 3.6 V |
|  |  | - | - | ±12 | LSB | Given for the DAC in 12-bit at VREF+ = 3.6 V |
| Gain error(3) | Gain error | - | - | ±0.5 | % | Given for the DAC in 12bit configuration |
| tSETTL (3) ING | Settling time (full scale: for a 10-bit input code transition between the lowest and the highest input codes when DAC_OUT reaches final value ±1LSB | - | 3 | 4 | µs | CLOAD  50 pF, RLOAD  5 kΩ |
| Update rate(3) | Max frequency for a correct DAC_OUT change when small variation in the input code (from code i to i+1LSB) | - | - | 1 | MS/ s | CLOAD  50 pF, RLOAD  5 kΩ |
| tWAKEU (3) P | Wakeup time from off state (Setting the ENx bit in the DAC Control register) | - | 6.5 | 10 | µs | CLOAD  50 pF, RLOAD  5 kΩ input code between lowest and highest possible ones. |
| PSRR+ (1) | Power supply rejection ratio (to VDDA) (static DC measurement | - | –67 | –40 | dB | No RLOAD, CLOAD = 50 pF |

- Guaranteed by design.

- Guaranteed by characterization.

- The quiescent mode corresponds to a state where the DAC maintains a stable output level to ensure that no dynamic consumption occurs.

- Guaranteed by characterization results.

& /

'$&B287[

12-ELW GLJLWDO WR DQDORJ FRQYHUWHU

5 /

%XIIHU(1)

Figure 61. 12-bit buffered /non-buffered DAC

AI1715763

%XIIHUHG/1RQ-EXIIHUHG '$&

  - The DAC integrates an output buffer that can be used to reduce the output impedance and to drive external loads directly without the use of an external operational amplifier. The buffer can be bypassed by configuring the BOFFx bit in the DAC_CR register.

112/143	DS5792 Rev 13

### Temperature sensor characteristics

##### Table 64. TS characteristics

| Symbol | Parameter | Min | Typ | Max | Unit |
| --- | --- | --- | --- | --- | --- |
| TL | VSENSE linearity with temperature | - | 1 | 2 | °C |
| Avg_Slope | Average slope | 4.0 | 4.3 | 4.6 | mV/°C |
| V25 | Voltage at 25 °C | 1.34 | 1.43 | 1.52 | V |
| tSTART(1) | Startup time | 4 | - | 10 | µs |
| TS_temp(2)(1) | ADC sampling time when reading the temperature | - | - | 17.1 | µs |

      - Guaranteed by design.

      - Shortest sampling time can be determined in the application by multiple iterations.

DS5792 Rev 13	113/143

# Package information

In order to meet environmental requirements, ST offers these devices in different grades of ECOPACK® packages, depending on their level of environmental compliance. ECOPACK® specifications, grade definitions and product status are available at: www.st.com.

ECOPACK® is an ST trademark.

## LFBGA144 package information

##### Figure 62. LFBGA144 – 144-ball low profile fine pitch ball grid array, 10 x 10 mm,

& 6HDWLQJ SODQH

$4

$3 $2

$1 $

'1

H

)

$1 %$//

3

$' &251(5

$1 %$// 3$' &251(5

%

'

$

)

(1

(

H

$

0

12

%27720 9,(:

1

E (144 EDOOV)

 HHH 0 & $ %

 III 0 &

723 9,(:

/)%*$144B;3B0(B92

0.8 mm pitch, package outline

|  | GGG | & |
| --- | --- | --- |

    - Drawing is not to scale.

##### Table 65. LFBGA144 – 144-ball low profile fine pitch ball grid array, 10 x 10 mm,

0.8 mm pitch, package mechanical data

| Symbol | millimeters |  |  | inches(1) |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Min | Typ | Max | Typ | Min | Max |
| A(2) | - | - | 1.700 | - | - | 0.0669 |
| A1 | 0.250 | 0.300 | 0.350 | 0.098 | 0.0118 | 0.0138 |
| A2 | 0.810 | 0.910 | 1.010 | 0.0319 | 0.0358 | 0.0398 |
| A3 | 0.225 | 0.26 | 0.295 | 0.0089 | 0.0102 | 0.0116 |
| A4 | 0.585 | 0.650 | 0.715 | 0.0230 | 0.0256 | 0.0281 |

114/143	DS5792 Rev 13

##### Table 65. LFBGA144 – 144-ball low profile fine pitch ball grid array, 10 x 10 mm,

0.8 mm pitch, package mechanical data (continued)

| Symbol | millimeters |  |  | inches(1) |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Min | Typ | Max | Typ | Min | Max |
| b | 0.350 | 0.400 | 0.450 | 0.0138 | 0.0157 | 0.0177 |
| D | 9.900 | 10.000 | 10.100 | 0.3898 | 0.3937 | 0.3976 |
| D1 | - | 8.800 | - | - | 0.3465 | - |
| E | 9.900 | 10.000 | 10.100 | 0.3898 | 0.3937 | 0.3976 |
| E1 | - | 8.800 | - | - | 0.3465 | - |
| e | - | 0.800 | - | - | 0.0315 | - |
| F | - | 0.600 | - | - | 0.0236 | - |
| ddd | - | - | 0.100 | - | - | 0.0039 |
| eee | - | - | 0.150 | - | - | 0.0059 |
| fff | - | - | 0.080 | - | - | 0.0031 |

- Values in inches are converted from mm and rounded to 4 decimal digits.

- STATSChipPAC package dimensions.

##### Figure 63. LFBGA144 – 144-ball low profile fine pitch ball grid array, 10 x 10 mm,

'SDG

'VP

/)%*$144B;3B)3B91

0.8 mm pitch, package recommended footprint

Table 66. LFBGA144 recommended PCB design rules (0.8 mm pitch BGA)

| Dimension | Recommended values |
| --- | --- |
| Pitch | 0.8 mm |
| Dpad | 0.400 mm |

DS5792 Rev 13	115/143

##### Table 66. LFBGA144 recommended PCB design rules (0.8 mm pitch BGA) (continued)

| Dimension | Recommended values |
| --- | --- |
| UBM | 0.350 mm |
| Dsm | 0.470 mm typ. (depends on the solder mask registration tolerance) |
| Stencil opening | 0.400 mm |
| Stencil thickness | Between 0.100 mm to 0.125 mm |
| Pad trace width | 0.120 mm |
| Ball Diameter | 0.400 mm |

Device marking for LFBGA144 package

The following figure gives an example of topside marking orientation versus ball A1 identifier location.

< ::

3URGXFW LGHQWLILFDWLRQ(1)

3

=&+6

67032)103

Figure 64. LFBGA144 marking example (package top view)

06Y3940591

%DOO

$1LGHQWLILHU

'DWH FRGH

1. Parts marked as ES or E or accompanied by an Engineering Sample notification letter are not yet qualified and therefore not approved for use in production. ST is not responsible for any consequences resulting from such use. In no event will ST be liable for the customer using any of these engineering samples in production. ST’s Quality department must be contacted prior to any decision to use these engineering samples to run a qualification activity

116/143	DS5792 Rev 13

## LFBGA100 package information

= 6HDWLQJ SODQH

$4 $2

$1 $

(1

$1 EDOO

$1 EDOO

;

H

)

LGHQWLILHU LQGH[ DUHD

(

$

)

'1

'

H

.

<

10

1

%27720 9,(:

E (100 EDOOV)

723 9,(:

+0B0(B92

Figure 65. LFBGA100 - 10 x 10 mm low profile fine pitch ball grid array package outline

|  | GGG | = |
| --- | --- | --- |

|  |  |  HHH 0 | = | < | ; |
| --- | --- | --- | --- | --- | --- |
|  |  |  III 0 | = |  |  |

    - Drawing is not to scale.

##### Table 67. LFBGA100 - 10 x 10 mm low profile fine pitch ball grid array package mechanical data

| Symbol | millimeters |  |  | inches(1) |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Min | Typ | Max | Min | Typ | Max |
| A | - | - | 1.700 | - | - | 0.0669 |
| A1 | 0.270 | - | - | 0.0106 | - | - |
| A2 | - | 0.300 | - | - | 0.0118 | - |
| A4 | - | - | 0.800 | - | - | 0.0315 |
| b | 0.450 | 0.500 | 0.550 | 0.0177 | 0.0197 | 0.0217 |
| D | 9.850 | 10.000 | 10.150 | 0.3878 | 0.3937 | 0.3996 |
| D1 | - | 7.200 | - | - | 0.2835 | - |
| E | 9.850 | 10.000 | 10.150 | 0.3878 | 0.3937 | 0.3996 |
| E1 | - | 7.200 | - | - | 0.2835 | - |
| e | - | 0.800 | - | - | 0.0315 | - |
| F | - | 1.400 | - | - | 0.0551 | - |
| ddd | - | - | 0.120 | - | - | 0.0047 |

DS5792 Rev 13	117/143

##### Table 67. LFBGA100 - 10 x 10 mm low profile fine pitch ball grid array package mechanical data

| Symbol | millimeters |  |  | inches(1) |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Min | Typ | Max | Min | Typ | Max |
| eee | - | - | 0.150 | - | - | 0.0059 |
| fff | - | - | 0.080 | - | - | 0.0031 |

1. Values in inches are converted from mm and rounded to 4 decimal digits.

##### Figure 66. LFBGA100 – 100-ball low profile fine pitch ball grid array, 10 x 10 mm,

'SDG

'VP

+0B)3B91

0.8 mm pitch, package recommended footprintoutline

Table 68. LFBGA100 recommended PCB design rules (0.8 mm pitch BGA)

| Dimension | Recommended values |
| --- | --- |
| Pitch | 0.8 |
| Dpad | 0.500 mm |
| Dsm | 0.570 mm typ. (depends on the soldermask registration tolerance) |
| Stencil opening | 0.500 mm |
| Stencil thickness | Between 0.100 mm and 0.125 mm |
| Pad trace width | 0.120 mm |

118/143	DS5792 Rev 13

Device marking for LFBGA100 package

The following figure gives an example of topside marking orientation versus ball A1 identifier location.

'DWH FRGH

< ::

3URGXFW LGHQWLILFDWLRQ(1)

;

98+6

67032)103

Figure 67. LFBGA100 marking example (package top view)

06Y3940891

%DOO $ LQGHQWLILHU

$GGLWLRQDO LQIRUPDWLRQ

1. Parts marked as ES or E or accompanied by an Engineering Sample notification letter are not yet qualified and therefore not approved for use in production. ST is not responsible for any consequences resulting from such use. In no event will ST be liable for the customer using any of these engineering samples in production. ST’s Quality department must be contacted prior to any decision to use these engineering samples to run a qualification activity

DS5792 Rev 13	119/143

## WLCSP64 package information

'HWDLO $

(URWDWHG 90)

DDD

(4[)

=

6HDWLQJ SODQH

E

FFF	;  < =

GGG	=

$1 2ULHQWDWLRQ UHIHUHQFH

(

HHH =

$1

%XPS

<

;

8 H	1 )

%XPS VLGH

'

*

+

H

H2

$

*

)

H1

$

$2

6LGH YLHZ

'HWDLO $

EEE =

Figure 68. WLCSP, 64-ball 4.466 × 4.395 mm, 0.500 mm pitch, wafer-level chip-scale package outline

&5B0(B92

:DIHU EDFN VLGH

    - Drawing is not to scale.

    - Primary datum Z and seating plane are defined by the spherical crowns of the ball.

##### Table 69. WLCSP, 64-ball 4.466 × 4.395 mm, 0.500 mm pitch, wafer-level chip-scale package mechanical data

| Symbol | millimeters |  |  | inches(1) |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Min | Typ | Max | Min | Typ | Max |
| A | 0.535 | 0.585 | 0.635 | 0.0211 | 0.0230 | 0.0250 |
| A1 | 0.205 | 0.230 | 0.255 | 0.0081 | 0.0091 | 0.0100 |
| A2 | 0.330 | 0.355 | 0.380 | 0.0130 | 0.0140 | 0.0150 |

120/143	DS5792 Rev 13

##### Table 69. WLCSP, 64-ball 4.466 × 4.395 mm, 0.500 mm pitch, wafer-level chip-scale package mechanical data

| Symbol | millimeters |  |  | inches(1) |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Min | Typ | Max | Min | Typ | Max |
| b(2) | 0.290 | 0.320 | 0.350 | 0.0114 | 0.0126 | 0.0138 |
| e | - | 0.500 | - | - | 0.0197 | - |
| e1 | - | 3.500 | - | - | 0.1378 | - |
| F | - | 0.447 | - | - | 0.0176 | - |
| G | - | 0.483 | - | - | 0.0190 | - |
| D | 4.446 | 4.466 | 4.486 | 0.1750 | 0.1758 | 0.1766 |
| E | 4.375 | 4.395 | 4.415 | 0.1722 | 0.1730 | 0.1738 |
| H | - | 0.250 | - | - | 0.0098 | - |
| L | - | 0.200 | - | - | 0.0079 | - |
| eee | - | 0.05 | - | - | 0.0020 | - |
| aaa | - | 0.10 | - | - | 0.0039 | - |
| Number of balls | 64 |  |  |  |  |  |

- Values in inches are converted from mm and rounded to 4 decimal digits.

- Dimension is measured at the maximum ball diameter parallel to primary datum Z.

##### Figure 69. WLCSP64 - 64-ball, 4.4757 x 4.4049 mm, 0.5 mm pitch wafer level chip scale package recommended footprint

'SDG

'VP

:/&6364B&5B)3B91

Table 70. WLCSP64 recommended PCB design rules (0.5 mm pitch)

| Dimension | Recommended values |
| --- | --- |
| Pitch | 0.5 |
| Dpad | 250 µm |
| Dsm | 300 µm |
| Stencil Opening | 320 µm |

DS5792 Rev 13	121/143

##### Table 70. WLCSP64 recommended PCB design rules (0.5 mm pitch) (continued)

| Dimension | Recommended values |
| --- | --- |
| Stencil Thickness | Between 100 µm to 125 µm |
| Pad trace width | 100 µm |
| Ball Diameter | 320 µm |

## LQFP144 package information

1$B0(B94

H

36

1

3,1 1

,'(17,),&$7,21

37

144

72

109

73

108

/

/1

'

'1

'3

0.25 PP

*$8*( 3/$1(

FFF &

6($7,1*

3/$1(

&

$

$2

$1

$1

Figure 70. LQFP144 - 144-pin, 20 x 20 mm low-profile quad flat package outline

E

(3

(1

(

Drawing is not to scale.

122/143	DS5792 Rev 13

##### Table 71. LQFP144 - 144-pin, 20 x 20 mm low-profile quad flat package mechanical data

| Symbol | millimeters |  |  | inches(1) |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Min | Typ | Max | Min | Typ | Max |
| A | - | - | 1.600 | - | - | 0.0630 |
| A1 | 0.050 | - | 0.150 | 0.0020 | - | 0.0059 |
| A2 | 1.350 | 1.400 | 1.450 | 0.0531 | 0.0551 | 0.0571 |
| b | 0.170 | 0.220 | 0.270 | 0.0067 | 0.0087 | 0.0106 |
| c | 0.090 | - | 0.200 | 0.0035 | - | 0.0079 |
| D | 21.800 | 22.000 | 22.200 | 0.8583 | 0.8661 | 0.8740 |
| D1 | 19.800 | 20.000 | 20.200 | 0.7795 | 0.7874 | 0.7953 |
| D3 | - | 17.500 | - | - | 0.6890 | - |
| E | 21.800 | 22.000 | 22.200 | 0.8583 | 0.8661 | 0.8740 |
| E1 | 19.800 | 20.000 | 20.200 | 0.7795 | 0.7874 | 0.7953 |
| E3 | - | 17.500 | - | - | 0.6890 | - |
| e | - | 0.500 | - | - | 0.0197 | - |
| L | 0.450 | 0.600 | 0.750 | 0.0177 | 0.0236 | 0.0295 |
| L1 | - | 1.000 | - | - | 0.0394 | - |
| k | 0° | 3.5° | 7° | 0° | 3.5° | 7° |
| ccc | - | - | 0.080 | - | - | 0.0031 |

1. Values in inches are converted from mm and rounded to 4 decimal digits.

DS5792 Rev 13	123/143

19.9

22.6

36

1

37

144

17.85

22.6

19.9

0.5

72

0.35

109

73 1.35

108

Figure 71. LQFP144 - 144-pin,20 x 20 mm low-profile quad flat package recommended footprint

DL14905H

1. Dimensions are expressed in millimeters.

124/143	DS5792 Rev 13

Device marking for LQFP144 package

The following figure gives an example of topside marking orientation versus pin 1 identifier location.

2SWLRQDO JDWH PDUN

5HYLVLRQ FRGH

3URGXFW LGHQWLILFDWLRQ(1)

'DWH FRGH

< ::

3LQ 1 LGHQWLILHU

06Y3656391

67032)103=(76

;

Figure 72. LQFP144 marking example (package top view)

1. Parts marked as ES or E or accompanied by an Engineering Sample notification letter are not yet qualified and therefore not approved for use in production. ST is not responsible for any consequences resulting from such use. In no event will ST be liable for the customer using any of these engineering samples in production. ST’s Quality department must be contacted prior to any decision to use these engineering samples to run a qualification activity

DS5792 Rev 13	125/143

## LQFP100 package information

3%!4).' 0,!.%

#

CCC #

$

$1

$3

,

,1

75	51

76

50

100

26

0). 1

)$%.4)&)#!4)/.

1

25

E

1,?-%?65

!

!2

!1

!1

Figure 73. LQFP100 – 14 x 14 mm 100 pin low-profile quad flat package outline

| 0.25 MM |  |
| --- | --- |
| '!5'% 0,!.% |  |

B

%3

%1

%

Drawing is not to scale.

##### Table 72. LQPF100 – 14 x 14 mm 100-pin low-profile quad flat package mechanical data

| Symbol | millimeters |  |  | inches(1) |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Min | Typ | Max | Min | Typ | Max |
| A | - | - | 1.600 | - | - | 0.0630 |
| A1 | 0.050 | - | 0.150 | 0.0020 | - | 0.0059 |
| A2 | 1.350 | 1.400 | 1.450 | 0.0531 | 0.0551 | 0.0571 |
| b | 0.170 | 0.220 | 0.270 | 0.0067 | 0.0087 | 0.0106 |
| c | 0.090 | - | 0.200 | 0.0035 | - | 0.0079 |
| D | 15.800 | 16.000 | 16.200 | 0.6220 | 0.6299 | 0.6378 |
| D1 | 13.800 | 14.000 | 14.200 | 0.5433 | 0.5512 | 0.5591 |
| D3 | - | 12.000 | - | - | 0.4724 | - |
| E | 15.800 | 16.000 | 16.200 | 0.6220 | 0.6299 | 0.6378 |
| E1 | 13.800 | 14.000 | 14.200 | 0.5433 | 0.5512 | 0.5591 |

126/143	DS5792 Rev 13

##### Table 72. LQPF100 – 14 x 14 mm 100-pin low-profile quad flat package mechanical data (continued)

| Symbol | millimeters |  |  | inches(1) |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Min | Typ | Max | Min | Typ | Max |
| E3 | - | 12.000 | - | - | 0.4724 | - |
| e | - | 0.500 | - | - | 0.0197 | - |
| L | 0.450 | 0.600 | 0.750 | 0.0177 | 0.0236 | 0.0295 |
| L1 | - | 1.000 | - | - | 0.0394 | - |
| k | 0° | 3.5° | 7° | 0° | 3.5° | 7° |
| ccc | - | - | 0.08 | - | - | 0.0031 |

- Values in inches are converted from mm and rounded to 4 decimal digits.

12.3

16.7

25

1

1.2

26

100

16.7 14.3

0.3

50

0.5

76

51

75

Figure 74. LQFP100 recommended footprint

AI14906C

  - Dimensions are in millimeters.

DS5792 Rev 13	127/143

Device marking for LQFP100 package

The following figure gives an example of topside marking orientation versus pin 1 identifier location.

3LQ 1 LGHQWLILHU

z tt

y

^dDϯϮ&ϭϬϯ

sϴdϲ

3URGXFW LGHQWLILFDWLRQ(1)

Figure 75. LQFP100 marking example (package top view)

06Y3656291

'DWH FRGH

5HYLVLRQ FRGH

2SWLRQDO JDWH PDUN

1. Parts marked as ES or E or accompanied by an Engineering Sample notification letter are not yet qualified and therefore not approved for use in production. ST is not responsible for any consequences resulting from such use. In no event will ST be liable for the customer using any of these engineering samples in production. ST’s Quality department must be contacted prior to any decision to use these engineering samples to run a qualification activity

128/143	DS5792 Rev 13

## LQFP64 package information

16

H

1

3,1 1

,'(17,),&$7,21

17

64

E

32

49

48	33

/

/1

FFF &

'

'1

'3

0.25 PP

*$8*( 3/$1(

6($7,1* 3/$1(

&

5:B0(B93

$

$2

$1

F

$1

Figure 76. LQFP64 – 10 x 10 mm 64 pin low-profile quad flat package outline

(3

(1

(

Drawing is not in scale.

##### Table 73. LQFP64 – 10 x 10 mm 64 pin low-profile quad flat package mechanical data

| Symbol | millimeters |  |  | inches(1) |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Min | Typ | Max | Min | Typ | Max |
| A | - | - | 1.600 | - | - | 0.0630 |
| A1 | 0.050 | - | 0.150 | 0.0020 | - | 0.0059 |
| A2 | 1.350 | 1.400 | 1.450 | 0.0531 | 0.0551 | 0.0571 |
| b | 0.170 | 0.220 | 0.270 | 0.0067 | 0.0087 | 0.0106 |
| c | 0.090 | - | 0.200 | 0.0035 | - | 0.0079 |
| D | - | 12.000 | - | - | 0.4724 | - |
| D1 | - | 10.000 | - | - | 0.3937 | - |
| D3 | - | 7.500 | - | - | 0.2953 | - |
| E | - | 12.000 | - | - | 0.4724 | - |
| E1 | - | 10.000 | - | - | 0.3937 | - |

DS5792 Rev 13	129/143

##### Table 73. LQFP64 – 10 x 10 mm 64 pin low-profile quad flat package mechanical data (continued)

| Symbol | millimeters |  |  | inches(1) |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Min | Typ | Max | Min | Typ | Max |
| E3 | - | 7.500 | - | - | 0.2953 | - |
| e | - | 0.500 | - | - | 0.0197 | - |
|  | 0° | 3.5° | 7° | 0° | 3.5° | 7° |
| L | 0.450 | 0.600 | 0.750 | 0.0177 | 0.0236 | 0.0295 |
| L1 | - | 1.000 | - | - | 0.0394 | - |
| ccc | - | - | 0.080 | - | - | 0.0031 |

- Values in inches are converted from mm and rounded to 4 decimal digits.

7.8

12.7

16

1

1.2

17

64

10.3

10.3

12.7

32

0.5

49

0.3

33

48

Figure 77. LQFP64 - 64-pin, 10 x 10 mm low-profile quad flat recommended footprint

AI14909C

  - Dimensions are in millimeters.

130/143	DS5792 Rev 13

Device marking for LQFP64 package

The following figure gives an example of topside marking orientation versus pin 1 identifier location.

z tt

3LQ 1 LGHQWLILHU

A

3URGXFW LGHQWLILFDWLRQ(1)

ZEdϲ

^dDϯϮ&ϭϬϯ

Figure 78. LQFP64 marking example (package top view)

06Y3656491

'DWH FRGH

5HYLVLRQ FRGH

1. Parts marked as ES or E or accompanied by an Engineering Sample notification letter are not yet qualified and therefore not approved for use in production. ST is not responsible for any consequences resulting from such use. In no event will ST be liable for the customer using any of these engineering samples in production. ST’s Quality department must be contacted prior to any decision to use these engineering samples to run a qualification activity

DS5792 Rev 13	131/143

## Thermal characteristics

The maximum chip junction temperature (TJmax) must never exceed the values given in

Table 10: General operating conditions on page 44.

The maximum chip-junction temperature, TJ max, in degrees Celsius, may be calculated using the following equation:

TJ max = TA max + (PD max x JA) Where:

- TA max is the maximum ambient temperature in °C,

- JA is the package junction-to-ambient thermal resistance, in °C/W,

- PD max is the sum of PINT max and PI/O max (PD max = PINT max + PI/Omax),

- PINT max is the product of IDD and VDD, expressed in Watts. This is the maximum chip internal power.

PI/O max represents the maximum power dissipation on output pins where: PI/O max =  (VOL × IOL) + ((VDD – VOH) × IOH),

taking into account the actual VOL / IOL and VOH / IOH of the I/Os at low and high level in the application.

##### Table 74. Package thermal characteristics

| Symbol | Parameter | Value | Unit |
| --- | --- | --- | --- |
| JA | Thermal resistance junction-ambient LFBGA144 - 10 × 10 mm / 0.8 mm pitch | 40 | °C/W |
|  | Thermal resistance junction-ambient LQFP144 - 20 × 20 mm / 0.5 mm pitch | 30 |  |
|  | Thermal resistance junction-ambient LFBGA100 - 10 × 10 mm / 0.8 mm pitch | 40 |  |
|  | Thermal resistance junction-ambient LQFP100 - 14 × 14 mm / 0.5 mm pitch | 46 |  |
|  | Thermal resistance junction-ambient LQFP64 - 10 × 10 mm / 0.5 mm pitch | 45 |  |
|  | Thermal resistance junction-ambient WLCSP64 | 50 |  |

### Reference document

JESD51-2 Integrated Circuits Thermal Test Method Environment Conditions - Natural Convection (Still Air). Available from www.jedec.org

132/143	DS5792 Rev 13

### Selecting the product temperature range

When ordering the microcontroller, the temperature range is specified in the ordering information scheme shown in Table 75: Ordering information scheme.

Each temperature range suffix corresponds to a specific guaranteed ambient temperature at maximum dissipation and, to a specific maximum junction temperature.

As applications do not commonly use the STM32F103xC, STM32F103xD and STM32F103xE at maximum dissipation, it is useful to calculate the exact power consumption and junction temperature to determine which temperature range will be best suited to the application.

The following examples show how to calculate the temperature range needed for a given application.

#### Example 1: High-performance application

Assuming the following application conditions:

Maximum ambient temperature TAmax = 82 °C (measured according to JESD51-2), IDDmax = 50 mA, VDD = 3.5 V, maximum 20 I/Os used at the same time in output at low level with IOL = 8 mA, VOL= 0.4 V and maximum 8 I/Os used at the same time in output at low level with IOL = 20 mA, VOL= 1.3 V

PINTmax = 50 mA × 3.5 V= 175 mW

PIOmax = 20 × 8 mA × 0.4 V + 8 × 20 mA × 1.3 V = 272 mW

This gives: PINTmax = 175 mW and PIOmax = 272 mW:

PDmax = 175 + 272 = 447 mW

Thus: PDmax = 447 mW

Using the values obtained in Table 74 TJmax is calculated as follows:

–	For LQFP100, 46 °C/W

TJmax = 82 °C + (46 °C/W × 447 mW) = 82 °C + 20.6 °C = 102.6 °C

This is within the range of the suffix 6 version parts (–40 < TJ < 105 °C).

In this case, parts must be ordered at least with the temperature range suffix 6 (see

Table 75: Ordering information scheme).

#### Example 2: High-temperature application

Using the same rules, it is possible to address applications that run at high ambient temperatures with a low dissipation, as long as junction temperature TJ remains within the specified range.

Assuming the following application conditions:

Maximum ambient temperature TAmax = 115 °C (measured according to JESD51-2), IDDmax = 20 mA, VDD = 3.5 V, maximum 20 I/Os used at the same time in output at low level with IOL = 8 mA, VOL= 0.4 V

PINTmax = 20 mA × 3.5 V= 70 mW

PIOmax = 20 × 8 mA × 0.4 V = 64 mW

This gives: PINTmax = 70 mW and PIOmax = 64 mW: PDmax = 70 + 64 = 134 mW

Thus: PDmax = 134 mW

DS5792 Rev 13	133/143

Using the values obtained in Table 74 TJmax is calculated as follows:

–	For LQFP100, 46 °C/W

TJmax = 115 °C + (46 °C/W × 134 mW) = 115 °C + 6.2 °C = 121.2 °C

This is within the range of the suffix 7 version parts (–40 < TJ < 125 °C).

In this case, parts must be ordered at least with the temperature range suffix 7 (see

Table 75: Ordering information scheme).

700

600

500

400

300

200

100

0

65	75	85	95	105	115	125	135

TA (°C)

Suffix 6

Suffix 7

PD (mW)

Figure 79. LQFP100 PD max vs. TA

134/143	DS5792 Rev 13

# Ordering information

##### Table 75. Ordering information scheme

6 = Industrial temperature range, –40 to 85 °C.

7 = Industrial temperature range, –40 to 105 °C.

Options

Device family

STM32 = Arm-based 32-bit microcontroller

Product type

F = general-purpose

Device subfamily

103 = performance line

Pin count

R = 64 pins V = 100 pins Z = 144 pins

Flash memory size

C = 256 Kbytes of Flash memory D = 384 Kbytes of Flash memory E = 512 Kbytes of Flash memory

Package

H = BGA T = LQFP

Y = WLCSP64

Temperature range

Example:	STM32	F  103 R  C	T	6   xxx

xxx = programmed parts TR = tape and real

For a list of available options (speed, package, etc.) or for further information on any aspect of this device, please contact your nearest ST sales office.

DS5792 Rev 13	135/143

# Revision history

##### Table 76.Document revision history

| Date | Revision | Changes |
| --- | --- | --- |
| 07-Apr-2008 | 1 | Initial release. |
| 22-May-2008 | 2 | Document status promoted from Target Specification to Preliminary Data. Section 1: Introduction and Section 2.2: Full compatibility throughout the family modified. Small text changes. Note 2 added in Table 2: STM32F103xC, STM32F103xD and STM32F103xE features and peripheral counts on page 11. LQPF100/BGA100 column added to Table 6: FSMC pin definition on page 38. Values and Figures added to Maximum current consumption on page 62 (see Table 18, Table 19, Table 20 and Table 21 and see Figure 14, Figure 15, Figure 17, Figure 18 and Figure 19). Values added to Typical current consumption on page 73 (see Table 22, Table 23 and Table 24). Table 19: Typical current consumption in Standby mode removed. Note 4 and Note 1 added to Table 65: USB DC electrical characteristics and Table 66: USB: full-speed electrical characteristics on page 129, respectively. VUSB added to Table 65: USB DC electrical characteristics on page 129. Figure 68: Recommended footprint(1) on page 143 corrected. Equation 1 corrected. Figure 73: LQFP100 PD max vs. TA on page 149 modified. Tolerance values corrected in Table 74: LFBGA144 – 144-ball low profile fine pitch ball grid array, 10 x 10 mm, 0.8 mm pitch, package data on page 139. |

136/143	DS5792 Rev 13

##### Table 76.Document revision history

| Date | Revision | Changes |
| --- | --- | --- |
| 21-Jul-2008 | 3 | Document status promoted from Preliminary Data to full datasheet. FSMC (flexible static memory controller) on page 22 modified. Number of complementary channels corrected in Figure 1: STM32F103xF, STM32F103xD and STM32F103xGSTM32F103xF and STM32F103xG performance line block diagram. Power supply supervisor on page 23 modified and VDDA added to Table 14: General operating conditions on page 59. Table notes revised in Section 5: Electrical characteristics. Capacitance modified in Figure 12: Power supply scheme on page 57. Table 60: SCL frequency (fPCLK1= 36 MHz.,VDD = 3.3 V) updated. Table 61: SPI characteristics modified, th(NSS) modified in Figure 49: SPI timing diagram - slave mode and CPHA = 0 on page 123. Minimum SDA and SCL fall time value for Fast mode removed from Table 59: I2C characteristics on page 120, note 1 modified. IDD_VBAT values and some IDD values with regulator in run mode added to Table 21: Typical and maximum current consumptions in Stop and Standby modes on page 68. Table 34: Flash memory endurance and data retention on page 87 updated. tsu(NSS) modified in Table 61: SPI characteristics on page 122. EO corrected in Table 70: ADC accuracy on page 132. Figure 58: Typical connection diagram using the ADC on page 133 and note below corrected. Typical TS_temp value removed from Table 72: TS characteristics on page 137. Section 6.1: Package mechanical data on page 138 updated. Small text changes. |

DS5792 Rev 13	137/143

##### Table 76.Document revision history

| Date | Revision | Changes |
| --- | --- | --- |
| 12-Dec-2008 | 4 | Timers specified on page 1 (motor control capability mentioned). Section 2.2: Full compatibility throughout the family updated. Table 6: High-density timer feature comparison added. General-purpose timers (TIMx) and Advanced-control timers (TIM1 and TIM8) on page 27 updated. Figure 1: STM32F103xF, STM32F103xD and STM32F103xGSTM32F103xF and STM32F103xG performance line block diagram modified. Note 10 added, main function after reset and Note 5 on page 44 updated in Table 8: High-density STM32F103xx pin definitions. Note 2 modified below Table 11: Voltage characteristics on page 58, ¦DVDDx¦ min and ¦DVDDx¦ min removed. Note 2 and PD values for LQFP144 and LFBGA144 packages added to Table 14: General operating conditions on page 59. Measurement conditions specified in Section 5.3.5: Supply current characteristics on page 62. Max values at TA = 85 °C and TA = 105 °C updated in Table 21: Typical and maximum current consumptions in Stop and Standby modes on page 68. Section 5.3.10: FSMC characteristics on page 87 updated. Data added to Table 50: EMI characteristics on page 111. IVREF added to Table 67: ADC characteristics on page 130. Table 81: Package thermal characteristics on page 146 updated. Small text changes. |

138/143	DS5792 Rev 13

##### Table 76.Document revision history

| Date | Revision | Changes |
| --- | --- | --- |
| 30-Mar-2009 | 5 | I/O information clarified on page 1. Figure 4: STM32F103xC and STM32F103xE performance line BGA100 ballout corrected. I/O information clarified on page 1. In Table 5: High-density STM32F103xx pin definitions: - I/O level of pins PF11, PF12, PF13, PF14, PF15, G0, G1 and G15 updated - PB4, PB13, PB14, PB15, PB3/TRACESWO moved from Default column to Remap column PG14 pin description modified in Table 6: FSMC pin definition. Figure 9: Memory map on page 54 modified. Note modified in Table 18: Maximum current consumption in Run mode, code with data processing running from Flash and Table 20: Maximum current consumption in Sleep mode, code running from Flash or RAM. Figure 17, Figure 18 and Figure 19 show typical curves (titles changed). Table 25: High-speed external user clock characteristics and Table 26: Low-speed external user clock characteristics modified. ACCHSI max values modified in Table 29: HSI oscillator characteristics. FSMC configuration modified for Asynchronous waveforms and timings. Notes modified below Figure 24: Asynchronous non- multiplexed SRAM/PSRAM/NOR read waveforms and Figure 25: Asynchronous non-multiplexed SRAM/PSRAM/NOR write waveforms. tw(NADV) values modified in Table 35: Asynchronous non-multiplexed SRAM/PSRAM/NOR read timings and Table 39: Asynchronous multiplexed PSRAM/NOR write timings. th(Data_NWE) modified in Table 36: Asynchronous non-multiplexed SRAM/PSRAM/NOR write timings In Table 41: Synchronous multiplexed PSRAM write timings and Table 43: Synchronous non-multiplexed PSRAM write timings: - tv(Data-CLK) renamed as td(CLKL-Data) - td(CLKL-Data) min value removed and max value added - th(CLKL-DV) / th(CLKL-ADV) removed Figure 28: Synchronous multiplexed NOR/PSRAM read timings, Figure 29: Synchronous multiplexed PSRAM write timings and Figure 31: Synchronous non-multiplexed PSRAM write timings modified. Figure 52: I2S slave timing diagram (Philips protocol)(1) and Figure 53: I2S master timing diagram (Philips protocol)(1) modified. WLCSP64 package added (see Figure 8: STM32F103xC and STM32F103xE performance line WLCSP64 ballout, ball side, Table 8: High-density STM32F103xx pin definitions, Figure 65: WLCSP, 64-ball 4.466 × 4.395 mm, 0.500 mm pitch, wafer-level chip-scale package outline and Table 76: WLCSP, 64-ball 4.466 × 4.395 mm, 0.500 mm pitch, wafer-level chip-scale package mechanical data). Small text changes. |

DS5792 Rev 13	139/143

##### Table 76.Document revision history

| Date | Revision | Changes |
| --- | --- | --- |
| 21-Jul-2009 | 6 | Figure 1: STM32F103xC, STM32F103xD and STM32F103xE performance line block diagram updated. Note 5 updated and Note 4 added in Table 5: High-density STM32F103xC/D/E pin definitions. VRERINT and TCoeff added to Table 13: Embedded internal reference voltage. Table 16: Maximum current consumption in Sleep mode, code running from Flash or RAM modified. fHSE_ext min modified in Table 21: High-speed external user clock characteristics. CL1 and CL2 replaced by C in Table 23: HSE 4-16 MHz oscillator characteristics and Table 24: LSE oscillator characteristics (fLSE = 32.768 kHz), notes modified and moved below the tables. Note 1 modified below Figure 29: Synchronous multiplexed PSRAM write timings. Table 25: HSI oscillator characteristics modified. Conditions removed from Table 27: Low-power mode wakeup timings. Jitter added to Table 28: PLL characteristics. Figure 47: Recommended NRST pin protection modified. In Table 31: Asynchronous non-multiplexed SRAM/PSRAM/NOR read timings: th(BL_NOE) and th(A_NOE) modified. In Table 32: Asynchronous non-multiplexed SRAM/PSRAM/NOR write timings: th(A_NWE) and th(Data_NWE) modified. In Table 33: Asynchronous multiplexed PSRAM/NOR read timings: th(AD_NADV) and th(A_NOE) modified. In Table 34: Asynchronous multiplexed PSRAM/NOR write timings: th(A_NWE) modified. In Table 35: Synchronous multiplexed NOR/PSRAM read timings: th(CLKH-NWAITV) modified. In Table 40: Switching characteristics for NAND Flash read and write cycles: th(NOE-D) modified. Table 53: SPI characteristics modified. Values added to Table 54: I2S characteristics and Table 55: SD / MMC characteristics. CADC and RAIN parameters modified in Table 59: ADC characteristics. RAIN max values modified in Table 60: RAIN max for fADC = 14 MHz. Table 71: DAC characteristics modified. Figure 61: 12-bit buffered /non- buffered DAC added. Figure 63: LFBGA100 - 10 x 10 mm low profile fine pitch ball grid array package outline and Table 75: LFBGA100 - 10 x 10 mm low profile fine pitch ball grid array package mechanical data updated. |
| 24-Sep-2009 | 7 | Number of DACs corrected in Table 3: STM32F103xx family. IDD_VBAT updated in Table 17: Typical and maximum current consumptions in Stop and Standby modes. Figure 16: Typical current consumption on VBAT with RTC on vs. temperature at different VBAT values added. IEC 1000 standard updated to IEC 61000 and SAE J1752/3 updated to IEC 61967-2 in Section 5.3.11: EMC characteristics on page 86. Table 63: DAC characteristics modified. Small text changes. |

140/143	DS5792 Rev 13

##### Table 76.Document revision history

| Date | Revision | Changes |
| --- | --- | --- |
| 19-Apr-2011 | 8 | Updated package choice for 103Rx in Table 2 Updated footnotes below Table 7: Voltage characteristics on page 43 and Table 8: Current characteristics on page 43 Updated tw min in Table 21: High-speed external user clock characteristics on page 58 Updated startup time in Table 24: LSE oscillator characteristics (fLSE = 32.768 kHz) on page 61 Updated note 2 in Table 51: I2C characteristics on page 96 Updated Figure 48: I2C bus AC waveforms and measurement circuit Updated Figure 47: Recommended NRST pin protection Updated Section 5.3.14: I/O port characteristics Updated Table 35: Synchronous multiplexed NOR/PSRAM read timings on page 72 Updated FSMC Figure 26 thru Figure 31 Updated Figure 41.: NAND controller waveforms for common memory write access and Figure 48.: I2C bus AC waveforms and measurement circuit Added Section 5.3.13: I/O current injection characteristics Updated Figure 67 and added Table 69: WLCSP, 64-ball 4.466 × 4.395 mm, 0.500 mm pitch, wafer-level chip-scale package mechanical data on page 120 LQFP64 package mechanical data updated: see Figure 73.: LQFP64 – 10 x 10 mm 64 pin low-profile quad flat package outline and Table 73: LQFP64 – 10 x 10 mm 64 pin low-profile quad flat package mechanical data on page 129. |
| 30-Sept-2014 | 9 | Added Note 7 in Table 5: High-density STM32F103xC/D/E pin definitions on page 31. Updated Note 10 in Table 5: High-density STM32F103xC/D/E pin definitions on page 31. Modified Note 2 in Table 62: ADC accuracy on page 108 Modified Note 3 in Table 62: ADC accuracy on page 108 Modified notes in Table 51: I2C characteristics on page 96 Updated Figure 51: SPI timing diagram - master mode(1) on page 100 |
| 23-Feb-2015 | 10 | Updated Figure 66.: BGA pad footprint, Figure 70: LQFP144 - 144-pin, 20 x 20 mm low-profile quad flat package outline, Figure 73.: LQFP100 – 14 x 14 mm 100 pin low-profile quad flat package outline, Figure 74.: LQFP100 recommended footprint, Figure 76.: LQFP64 – 10 x 10 mm 64 pin low-profile quad flat package outline, Figure 77.: LQFP64 - 64- pin, 10 x 10 mm low-profile quad flat recommended footprint Added Figure 72.: LQFP144 marking example (package top view), Figure 75.: LQFP100 marking example (package top view), Figure 78.: LQFP64 marking example (package top view) Updated Table 72: LQPF100 – 14 x 14 mm 100-pin low-profile quad flat package mechanical data, Table 73: LQFP64 – 10 x 10 mm 64 pin low- profile quad flat package mechanical data |

DS5792 Rev 13	141/143

##### Table 76.Document revision history

| Date | Revision | Changes |
| --- | --- | --- |
| 31-08-2015 | 11 | Replaced USBDP and USBDM by USB_DP and USB_DM in the whole document. Updated: - Introduction - Reference standard in Table 43: ESD absolute maximum ratings. - Updated IDDA description in Table 63: DAC characteristics. - Section : I2C interface characteristics - Figure 62: LFBGA144 – 144-ball low profile fine pitch ball grid array, 10 x 10 mm, 0.8 mm pitch, package outline - Updated sentence before Figure 78: LQFP64 marking example (package top view). - Figure 65: LFBGA100 - 10 x 10 mm low profile fine pitch ball grid array package outline and sentence before Figure 75: LQFP100 marking example (package top view) – Figure 68: WLCSP, 64-ball 4.466 × 4.395 mm, 0.500 mm pitch, wafer-level chip-scale package outline - Figure 48: I2C bus AC waveforms and measurement circuit on page 97 - Section 6.1: LFBGA144 package information and Section 6.2: LFBGA100 package information. - Table 20: Peripheral current consumption Added: - Figure 63: LFBGA144 – 144-ball low profile fine pitch ball grid array, 10 x 10 mm, 0.8 mm pitch, package recommended footprint - Figure 64: LFBGA144 marking example (package top view) - Figure 66: LFBGA100 – 100-ball low profile fine pitch ball grid array, 10 x 10 mm, 0.8 mm pitch, package recommended footprintoutline – Figure 69: WLCSP64 - 64-ball, 4.4757 x 4.4049 mm, 0.5 mm pitch wafer level chip scale package recommended footprint - Table 66: LFBGA144 recommended PCB design rules (0.8 mm pitch BGA) - Table 68: LFBGA100 recommended PCB design rules (0.8 mm pitch BGA) - Table 70: WLCSP64 recommended PCB design rules (0.5 mm pitch). |
| 26-Nov-2015 | 12 | Updated: - Table 59: ADC characteristics - Table 65: LFBGA144 – 144-ball low profile fine pitch ball grid array, 10 x 10 mm, 0.8 mm pitch, package mechanical data - Table 66: LFBGA144 recommended PCB design rules (0.8 mm pitch BGA) Added: - Note 3 on Table 7: Voltage characteristics |
| 10-Jul-2018 | 13 | Updated: - Table 2: STM32F103xC, STM32F103xD and STM32F103xE features and peripheral counts - Section 7: Ordering information |

142/143	DS5792 Rev 13

IMPORTANT NOTICE – PLEASE READ CAREFULLY

STMicroelectronics NV and its subsidiaries (“ST”) reserve the right to make changes, corrections, enhancements, modifications, and improvements to ST products and/or to this document at any time without notice. Purchasers should obtain the latest relevant information on ST products before placing orders. ST products are sold pursuant to ST’s terms and conditions of sale in place at the time of order acknowledgement.

Purchasers are solely responsible for the choice, selection, and use of ST products and ST assumes no liability for application assistance or the design of Purchasers’ products.

No license, express or implied, to any intellectual property right is granted by ST herein.

Resale of ST products with provisions different from the information set forth herein shall void any warranty granted by ST for such product.

ST and the ST logo are trademarks of ST. All other product or service names are the property of their respective owners.

Information in this document supersedes and replaces information previously supplied in any prior versions of this document.

© 2018 STMicroelectronics – All rights reserved

DS5792 Rev 13	143/143